Continuous Time Regulator for Linear Systems with Constrained Control

##### N.H. Mejhed +, A. Hmamed * and A. Benzaouia**

+ Dépt GII, ENSA , BP 33/S, Agadir, Morocco

Abstract: - In this work, A time varying control law is proposed for linear continuous-time systems with non Symmetrical constrained control. Necessary and sufficient conditions allowing us to obtain the largest non-symmetrical positively invariant polyhedral set with respect to (w.r.t) the system in the closed loop are given. The asymptotic stability of the origin is also guaranteed. The case of symmetrical constrained control is obtained as a particular case. The performances of our regulator with respect to the results of [3] are shown with the help of an example.

Keywords:- Constrained control,Positively invariant sets, Time varying regulator, Extended eigenvalues.

NOTATIONS: If x is a vector of ℜn then:

xi+=sup(xi,0)  and  xi−=sup(−xi,0), i=1,…,n

We will further note the following: for two vectors x, y of ℜn:

x≤y (Respectively, x<y) if xi≤yi (respectively, xi<yi) i=1,…,n.

In is the identity matrix of ℜnxn; σ(A) denotes the spectrum of matrix A; Re(λ) the real part of the eigenvalue λ and λi(A) the ith eigenvalue of A. μ(A) the measure of A , Int(ℜ+m) is the interior of ℜ+m, whereas ∂D denotes the boundary of D. KerF is the null space of matrix F.

1. INTRODUCTION

This paper is devoted to the study of linear continuous-time systems described by (1):

x(t)=Ax(t)+Bu(t),      x∈ℜn                             (1)

x is the state vector and u is the constrained control, that is:

u∈Ω⊂ℜm,m≤n                                                (2)

Matrices A and B are constant and satisfy assumption (3):

(A,B) Controllable                                                 (3)

Ωis the set of admissible controls defined by (4):

Ω=u∈ℜm−q2≤u≤q1;q1,q2∈intℜ+m           (4)

This is a non-symmetrical polyhedral set, as is generally the case in practical situation.

Practical control systems are often subject to technological and safety constraints, which are translated as bounds on the constraint and state variables. The respect of this constraint can be accomplished by designing suitable feedback law.

In many cases, this can be done by constructing positively invariant domains inside the set of the constraints. The purpose of a regulation law is to stabilise the system while maintaining its state vector in a positively invariant set [1-2]. Many approaches have been derived from this concept.

Particularly, one which consists on both, using large initialisation domain and respecting the constrained control, [3-6]. Recently, a piecewise linear control law has been derived for linear continuous time systems, leading to the use of non-symmetrical Lyapunov functions [3]. These functions were introduced in [2], and are also used in [1]. Otherwise, the proposed technique seems to be very long and the problem appears between the size of the initialisation do main and the dynamic of the closed loop system. This justifies the development of this technique by using a time varying regulator. The choose of such regulator has been the subject of many works from which we cite, [14-16] in the decentralized control case. Inspired by the work in [3], our contribution in the present paper is intended to improve the speed of regulation by setting the modified control law as follows:

u(t)=φ(t)F0x(t)=F(t)x(t), F0∈ℜmxn                     (5)

rang(F0)=mwith φ(t)>0, ∀t≥0.

Taking into account (5), system (1) becomes a non-stationary system in the following form:

x(t)=(A+φ(t)BF0)x(t), ∀t≥0                             (6)

Generally φ(t) and matrix F0 must be found that makes the system (6) asymptotically stable and inside the constraints. It is well known that a linear time invariant system is stable if and only if ail eigenvalues of the system matrix have negative real parts [9]. However, this is no longer true for linear time-varying systems. Under the assumption of the non-stationary systems, the eigenvalues method for proving the asymptotic stability is not adequate. An alternative method is the use of matrix measure that means:

μ(A+φ(t)BF0)≤−ξ, ∀t≥0, ξ≤0                         (7)

We will show latter in this work, how to choose the function φ(t).

Remark: Note that rang(F(t))=m, because rang(F0)=m and φ(t)≠0, ∀t≥0.

In the constrained case, we follow the approach proposed in [10] and further developed in [1]-[2] and [11] and references therein. This approach consists of giving conditions on the choice of the stabilizing regulator (5) such that model (6) remains valid. This is only possible if the state is constrained to evolve in a specified region defined by:

D(F(t),q1,q2)=x∈ℜn−q2≤F(t)x(t)≤q1; q1,q2∈intℜ+m(8)

Note that this domain is unbounded where m<n. In this case, if x(t)∈D(F(t),q1,q2) we may get x(t+λ)∈D(F(t),q1,q2), ∀λ≥0. Note that the main property of this set in the stationary case is not valid in our case that is the set x(t)∈D(F(t),q1,q2).

In particular, domain D(Im,q1,q2)can be described with function

v(z)=maximaxzi+q1i,zi−q2i                                    (9)

i.e., D(Im,q1,q2)=z∈ℜmv(z)≤1.

It follows from above that the main result of this note is to give the necessary and sufficient conditions under which the nonsymmetrical polyhedral domain Ω is positively invariant w.r.t. motions of system 6.

2. PRELIMINARIES

In this section, we present some definitions and useful results for the sequel. Consider a continuous-time non-linear system

z(t)=f(z(t)),   z∈ℜm,f(0)=0                              (10)

Definitions 2.1:  Consider a functionv:ℜm→ℜ+ with v(0)=0and assume that v is directionally differentiable at each direction and define v(z) by:

v(z(t))=d+dtv(z(t))=∂vz(t);f(z(t))

=limε→0+v(z+εf(z))−v(z)ε                            (11)

v(z) is the directional derivative of function v at z in the direction f(z) [9], with f(0)=0 and z(t)=f(z(t)).

Lemma2.2[12]: Let A, B∈Cnxn, we have:

a) Re(λi(A))≤μ(A),∀i=1,...,n.

b) μ(cA)=cμ(A),∀c≥0

c) μ(A+cI)=μ(A)+c,  ∀c∈ℜ

d) μ(A+B)≤μ(A)+μ(B)

e) μ:Cnxn→ℜ is convex on Cnxn

μ(λA+(1−λ)B)≤λμ(A)+(1−λ)μ(B), ∀λ∈[0,1]

Definition 2.3 [4]: A differentiable non-zeros vector e(t) is said to be the extended-eigenvector (x-eigenpair) of the nxn matrix G(t), associated with the extended-eigenvalues λ(t) (a scalar time function) if it satisfies,

G(t)e(t)=λ(t)e(t)+e(t)

Consider the following continuous non-stationary system,

z(t)=H(t)z(t), z∈ℑ⊆ℜm and 0∈Intℑ            (12)

The necessary and sufficient condition of function v defined by (9) to be a Lyapunov function for system (12) is given by the following result.

Theorem 2.4 : Function v(z)=maximaxzi+q1i,zi−q2i with q1>0,  q2>0, which is continuous positive definite, is a Lyapunov function of system (12) on the set ℑ and domain:

D(Im,q1,q2)=z∈ℜm−q2≤z≤q1⊆ℑ

is a stability domain of the system if and only if :

H(t)q≤0, ∀t≥0                                               (13)

H(t)=H1(t)H2(t)H2(t)H1(t),q=q1q2, ∀t≥0

H1(t)=hii(t)     i=jhij+(t)     i≠j, H2(t)=0           i=jhij−(t)     i≠j

Proof: (If) The same as [1], with:

v(z)≤maximaxH1(t)q1+H2(t)q2iq1iv(z);H2(t)q1+H1(t)q2iq2iv(z)∀t≥0                                                        (14)

From condition (13), we have:

v(z)≤0, ∀z∈ℑ⊆ℜm

Consequently, from [9], we conclude that domain D(Im,q1,q2) is a stability domain of the system.

(Only if): Assume that function v(z) is a Lyapunov function of system (6) and condition (13) does not hold, i.e., there exist only i∈[1,m] such that,

hii(t)q1i+j=1,j≠imhij+(t)q1j+hij−(t)q2j>0

At this step, we follow the proof given in [1].

# Remarks

1) When φ(t)=1, we obtain the result given in [1].

2) It is well known that a stability domain for system (12) is also a positively set for the system

3) The relation (13) is equivalent to the following matrix measure:

μ(H(t))≤0, ∀t≥0                                             (15)

Induced by the vector norm:

z=maximaxzi+q1i,zi−q2i                                      (16)

For more detail, see Appendix 1.

4) If there exist ξ>0 such that μ(H(t))≤−ξ, we have:

v(z)≤−ξv(z)                                                      (17)

and then from [9], system (12) is asymptotically stable.

The symmetrical case is obtained directly by :

Corollary 2.5 : Function v(z)=maxiziqi is a Lyapunov function of system (12) on the set ℑ and domain D(Im,q,q)=z∈ℜm−q≤z≤q⊆ℑ is a stability domain of the system if and only if:

H(t)q≤0 with H=hii(t)           i=jhij(t)          i≠j, ∀t≥0

Proof: Follows readily from Theorem 2.4.

3. MAIN RESULTS

In this section, we apply the results of Theorem 2.4 to the problem of the constrained regulator described in Section I.

Consider system (1) with the feedback law given by (5). The system in the closed loop is then given by (6). Let us make the change of variables,

z(t)=φ(t)F0x(t), F0∈ℜmxn                                (18)

=F(t)x(t)

with matrix F0 given by (5) and (7). It follows that:

z(t)=φ(t)F0+φ(t)F0(A+φ(t)BF0)x(t)

φ(t)F0φ(t)φ(t)In+A+φ(t)BF0x(t)

If there exists a matrix H(t)∈ℜmxm such that:

F(t)A+BF(t)=H(t)−φ(t)φ(t)InF(t)                    (19)

Then, the change of variables (18) allows us to transform dynamical system (6) to dynamical non-stationary system (12). The study of the stability of system (6) with x∈D(F(t),q1,q2) defined by (8), becomes possible by the use of system (12) and Theorem 2.4, with z∈ℑ=D(Im,q1,q2)=Ω.

Before giving the main result, we present all the necessary Lemmas. The first concerns (19), which is to be for every t.

For this, let us define the set ℵ(F) of the matrix F(t) as follows :

ℵ(F)=x(t)∈ℜn  F(t)x(t)=0,  ∀t≥0,  F(t)∈ℜmxn

In the stationary case, ℵ(F)=Ker(F)

We note ℏ(t)=H(t)−φ(t)φ(t)In and A0(t)=A+BF(t).

Lemma 3.1 : If a matrix H(t)∈ℜmxm satisfying (19) exists, then n-m stables extended eigenvectors common to matrices A and A0(t) belong to ℵ(F).

Proof: Let a matrix ℏ(t)satisfying equation (19) exists. Consider an extended eigenvector e(t) of matrix A0(t) corresponding to an extended eigenvalue λ(t), [13], i.e:

A0(t)e(t)=λ(t)e(t)+e(t)                                 (20)

Equation (19) allows us to write

F(t)A0(t)e(t)=λ(t)(F(t)e(t))+(F(t)e(t))             (21)

=ℏ(t)(F(t)e(t))

Then F(t)e(t) is an extended eigenvector of matrix ℏ(t) corresponding to the same extended eigenvalue λ(t). Matrix ℏ(t)∈ℜmxm could admit only m extended eigenvalues from the set of extended eigenvalues of matrix A0(t). Let us note σ(A0(t))=Λ=Λ1∪Λ2, with σ(ℏ(t))⊂Cm and Λ2⊂Cn−m. Where σ(A0(t)) (σ(ℏ(t))) denotes a set of extended eigenvalues of A0(t) (respectively ℏ(t)). Then, for δ∈Λ2, we should have,

A0(t)w(t)=δ(t)w(t)+w(t)                                (22)

then

F(t)A0(t)w(t)=δ(t)(F(t)w(t)+(F(t)w(t))

=ℏ(t)(F(t)w(t))                                    (23)

Implies,

F(t)w(t)=0, ∀t≥t0                                           (24)

For w satisfying A0(t)w(t)=δw(t)+w(t).

Since A0(t)=A(t)+B(t)F(t), we should also have:

A(t)w(t)+B(t)F(t)w(t)=δ(t)w(t)+w(t)

From (24), we obtain A(t)w(t)=δ(t)w(t)+w(t), and then Λ2⊂σ(A(t)).

If δ=0, then from (23), (ddt)(F(t)w(t))=0, implies F(t)w(t)=cste. In this case, vector w(t) do not belong necessarily to ℵ(F). Further, condition (7) ensures that μA0(t)≤−ξ, ξ>0, ∀t≥t0, using the fact that Re(λi(A0(t))≤μ(A0(t)), [7], then, the set of extended eigenvalues of matrix A0(t) is stable. Consequently, Λ2 contains n-m stable and non-null extended eigenvalues corresponding to n-m common extended eigenvectors to matrices A(t) and A0(t) and belonging to ℵ(F).

We now give two lemmas on the ℵ(F) with F(t)∈ℜmxn and rank(F(t))=m.

Lemma 3.2 : There exists a matrix H(t)∈ℜmxm satisfying relation (19) if and only if the existence of t>0 such that x(t)∈ℵ(F) impliesx(t+τ)∈ℵ(F), ∀τ>0, ∀t.

Proof: (If): Assume that there exists a matrix H(t)∈ℜmxm satisfying (19) and let x(0)∈ℵ(F), that is,

F(0)x(0)=0                                                      (25)

Let us present the solution for system (6) in the following form,

x(t)=e0t(A+φ(τ)BF0)dτx(0),∀t≥0                            (26)

Using the fact that,

e0t(A+φ(τ)BF0)dτ=k=0∞0t(A+φ(t)BF0)dτk

=I+k=1∞0t(A+φ(t)BF0)dτk

By using (19) and the following relation obtained from (19)

F0e0t(A+φ(τ)BF0)dτ=e0t−φ(τ)φ(τ)I+H(τ)dτF0

then,

F(t)e0t(A+φ(τ)BF0)dτx(0)=e0t−φ(τ)φ(τ)I+H(τ)dτF0φ(t)x(0)By using (25) and the fact that φ(t)≠0,∀t≥0, we obtain

φ(t)F0x(t)=0,∀t≥0, i.e., x(t)∈ℵ(F), ∀t≥0.

(Only if): Assume that the existence of t>0 such that x(t)∈ℵ(F) implies x(t+τ)∈ℵ(F), ∀τ>0, and show that condition (19) holds. Let, that is φ(t)F0x(t)=0,∀t≥0. It is clear that ddt(Fx(t))=0 and obviously φ(t)F0x(t)+φ(t)F0x(t)=0,∀t≥0.

We obtain:

φ(t)F0x(t)=0Fφ(t)φ(t)I+A+φ(t)BF0x(t)=0, ∀t≥0            (27)

In this step, we can generalize the results of [1] to the relation (27). This implies the existence of H(t)∈ℜmxm such that (19) is satisfied.

Lemma 3.3

If domain D(F(t),q1,q2) is positively invariant w.r.t. system (6), ∀t≥0, then if x(t)∈ℵ(F), x(t+τ)∈ℵ(F), ∀τ>0.

Proof: Letx(0)∈ℵ(F), it is clear that x(0)∈D(F(t),q1,q2). From (26), we can deduce

F(t)x(t)=F(t)e0t(A+φ(τ)BF0)dτx(0), ∀τ>0.

At this step, we can use the proof given in [1] as the proof remains unchanged. We can deduce that F(t)x(t)=0,∀t≥0.

We are now able to give the main result of this paper.

Theorem 3.4 : Domain D(F(t),q1,q2) is positively invariant w.r.t system (6) if and only if there exists a matrix H(t)∈ℜmxm, such that:

i)F0A+φ(t)BF0=H(t)−φ(t)φ(t)InF0, ∀t≥0       (28)

ii) H(t)q≤0 ,    ∀t≥0                                  (29)

with matrix H(t) and vector q are defined by (13).

The proof is the same as given in [1] and is omitted for brevity.

Remark:

When φ(t)=1, we obtain the result given in [1].

The symmetrical case is obtained directly by Corollary3.5.

## Corollary 3.5

If q1=q2=ρ, domain D(F(t),q1,q2) is positively invariant w.r.t system (6) if and only if there exists a matrix H(t)∈ℜmxm, such that:

i) F0A+φ(t)BF0=H(t)−φ(t)φ(t)InF0, ∀t≥0

ii) H(t)q≤0, ∀t≥0.

matrix H is given in Corollary 2.4.

The result of this Theorem is based on the existence of a matrix H(t)∈ℜmxm satisfying (19). A necessary and sufficient condition of the existence of a matrix H(t) is giving by the following Theorem.

## Theorem 3.6

There exists a matrix H(t)∈ℜmxm solution of (19) or (28), where F0∈ℜm and rang(F0)=m, m≤n if and only if :

rangF0F0A=m                                                  (30)

Proof: We change only matrix A by φ(t)φ(t)I+A in the proof given in [8] and by observing that:

F0F0φ(t)φ(t)I+A=I0φ(t)φ(t)IIF0F0A

The proof remains unchanged.

In order to ensure a rate of increase of the system dynamics, one should impose to matrix H(t) :

H(t)q≤−εq ,  ∀t≥0

where ε is a positive real number (ε≥0).

Comments

Conditions (28) and (29) guarantee that domain D(F(t),q1,q2) defined by (8) is positively invariant w.r.t system (1)-(7), despite the existence of non-symmetrical constraints on the control, but these conditions are very difficult to verify, because we can not compute the matrix H(t)for all t. Then, we propose to employ only H(0) and H(∞) to handle such situation.

Before proving the Proposition 3.7, we first need the following assumptions about the function φ(t):

(a) φ(t)>0, ∀t≥0

(b) φ(t) is a nondecreasing function.

(c) φ(t)φ(t)≤φ(0)φ(0)φ(t)−φ(∞)φ(0)−φ(∞), ∀t≥0.

## Remarks

1) From assumption (a) and (b), we have:

0<φ(0)≤φ(t)≤φ(∞), ∀t≥0

It follows that,

0≤φ(t)−φ(∞)φ(0)−φ(∞)≤1,  ∀t≥0

2) From (b), we have φ(t)≥0, ∀t≥0, then from (a), we can conclude that:

φ(t)φ(t)≥0, ∀t≥0

3) Giving the inequality (c), and taking its limit as t tends to infinity, one has:

limt→∞φ(t)φ(t)≤limt→∞φ(0)φ(0)φ(t)−φ(∞)φ(0)−φ(∞)

It is clear that: φ(∞)φ(∞)≤0,  ∀t≥0.

Combining this condition and the condition giving by Remark2, (i.e.φ(t)φ(t)≥0, ∀t≥0), this implies that:φ(∞)φ(∞)=0. From assumption (a), one has φ(∞)=0. This suffices to conclude that: φ(∞)=cste=K.

Proposition 3.7

The polyhedral set defined by (8) is a positively invariant w.r.t. system (6) if and only if there exists H(0) and H(∞) such that:

F0A+φ(0)BF0=H(0)−φ(0)φ(0)InF0                    (31)

F0A+φ(∞)BF0=H(∞)−φ(∞)φ(∞)InF0                  (32)

H(0)q≤0                                                         (33)

H(∞)q≤0                                                        (34)

Proof:

(IF) It follows from (31), (32) and (19) that:

F0A=H(0)F0−F0φ(0)φ(0)Im+φ(0)BF0                     (35)

=H(∞)F0−F0φ(∞)φ(∞)Im+φ(∞)BF0               (36)

=H(t)F0−F0φ(t)φ(t)Im+φ(t)BF0                    (37)

Then the full rankness of the matrix F0 leads to the following equation,

H(∞)−φ(∞)φ(∞)I−φ(∞)F0B=H(0)−φ(0)φ(0)I−φ(0)F0B  (38)

Then,

H(∞)=H(0)−φ(0)φ(0)−φ(∞)φ(∞)I−φ(0)−φ(∞)F0B (39)

From (37) and (38), we have:

H(t)=1−φ(t)−φ(∞)φ(0)−φ(∞)H(∞)+φ(t)−φ(∞)φ(0)−φ(∞)H(0)+φ(t)φ(t)−1−φ(t)−φ(∞)φ(0)−φ(∞)φ(∞)φ(∞)−φ(t)−φ(∞)φ(0)−φ(∞)φ(0)φ(0)Im

(40)

We note:

e(t)=φ(t)−φ(∞)φ(0)−φ(∞)                                            (41)

Then,

μ(H(t))=μ(1−e(t)H(∞)+eH(0)+c(t)Im)     (42)

where:

c(t)=φ(t)φ(t)−1−e(t)φ(∞)φ(∞)−e(t)φ(0)φ(0)≤0    (43)

and e(t) is giving by (41).

By applying Lemma 2.2 ©, we have,

μ(H(t))=μ(1−e(t)H(∞)+e(t)H(0))+c(t)        (44)

φ(t) is chosen to satisfy (a), (b) and (c), then by applying Lemma 2.2 to equation (44), we obtain,

μ(H(t))≤1−e(t)μ(H(∞))+e(t)μ(H(0))+c(t)        (45)

where c(t) is giving by (40) and  e(t) by (41).

Furthermore,

μ(H(t))≤max(μ(H(0),μ(H(∞))+c(t),c(t)≤0  (46)

It follows that if (33) and (34) holds, from the above results, one should obtain H(t)q≤0, ∀t≥0.

(Only if): We assume that the polyhedral (8) is positively invariant w.r.t. system (6). By using Theorem3.4, there exists H(t)∈ℜmxm such that:

F0A+φ(t)BF0=H(t)−φ(t)φ(t)InF0

H(t)q≤0   ∀t≥0

In particular, for t=0 and t→∞, we obtain:

F0A+φ(0)BF0=H(0)−φ(0)φ(0)InF0

F0A+φ(∞)BF0=H(∞)−φ(∞)φ(∞)InF0

H(0)q≤0

H(∞)q≤0

Remarks

1) The symmetrical case is easily deduced.

2) In order to augment the system dynamics, one should impose to matrices H(0) and H(∞):

H(0)q≤−εq                                                     (47)

H(∞)q≤−εq                                                    (48)

where ε is a positive number ε≥0.

Comments:

When the regulator F0 is changed to F∞=φ(∞)F0, the eigenvalues of H(∞) will be placed in a region of the left half-complex space, which makes them more stables than the eigenvalues of H(0). Furthermore, the control law increases the gain as the trajectory converges towards the origin.

φ(t) is chosen to satisfy assumptions (a), (b) and (c). This means that the dynamics amelioration cannot be made with enough liberty.

4. APPLICATION

The assumption (a), (b) and (c) institute the class of regulator, which permit to achieve the desired performance. In particular, we can choose φ(t) in the form:

φ(t)=1+β(1−e−αt), α,β≥0

It is clear that the assumption a)-c) are satisfied.

The aim of this kind of regulator is to permit to start with a slow dynamics very close to the regulator with the gain F0 and to force this dynamics to increase until it reaches the one of the regulator with the gain (1+β)F0 at asymptotic behaviour. In addition, this permits the boundless of the time-varying control gain φ(t).

In this case, equation (31) and (32) become the following:

F0(A+BF0)=H(0)−αβImF0

F0[A+(1+β)BF0]=H(∞)F0

with :

H(t)=(1−e−αt)H(∞)+e−αtH(0)          I

+αβe−αt1+β(1−e−αt)−αβe−αtI

and

H(0)=H(∞)+αβI−βF0B

Two parameters must be found to satisfy assumption  (a), (b) and (c) with:

φ(t)φ(t)=αβe−αt1+β(1−e−αt), φ(0)φ(0)=αβ, φ(∞)φ(∞)=0.

From (45), we have:

μ(H(t))≤(1−e−αt)μ(H(∞))+e−αtμ(H(0))          I                        −αβ2e−αt(1−e−αt)1+β(1−e−αt)    , ∀t≥0

In order to recapitulate all the steps required to satisfy our purpose, we present the following algorithm.

Algorithm

Step0: Verify that A possesses (n-m) stable eigenvalues. When it is not the case, we proceed to an augmentation of the vector entries without losing assumption (3a), this technique is given in [2a].

Step1: Give ε, α , β≥0 and a matrix H(0) such that;

H(0)q≤−εq

Step2: Solve equation (31) by using the inverse procedure detailed in [7] to obtain F0.

Step3: Solve equation (32) to obtain H(∞).

Step4: If H(∞)q≤−εq holds, then use α, β and F0 to realize a time-varying regulator. If not, we return to step1.

5. COMPUTER SIMULATION

In this section, we present several numerical examples illustrating the performance of the proposed regulator.

Example1

Consider the second order system (1) given by:

A=1−2−3−0.5;     B=11,     j≠immij−+q2jq2imij+].

σ(A)=−2.3120,2.8117

We choose, α=0.21, β=3 and ε=0.3 and let: H(0)=−0.39.

The resolution of equation (31) gives:

F0=−9.6745.8422

and then :

F∞=(1+β)F0=−38.6923.36

According to (32), H(∞) is given by:H(∞)=−12.4554 and

H(∞)q=
              −161.9202−62.2770T≤−εq

We obtain the desired results given by:

σ(A+BF0)=−2.3120,−0.9998

σ(A+(1+β(1−e−αt))BF0)t=10=−2.3120,−11.0358σ(A+BF∞)=−2.3120,−12.5152

Note that the eigenvalues -2.3120 is common to A and A+BF0

According to the result given in [3], we choose N=3 and H0 such that F0A+F0BF0=H0F0 and H0q≤−εq, which implies from (31) that H0=H(0)−αβI=−1.02.

From [3], if we choose α[3]=1.01, we obtain the following results, with:

σ(A+BF0)=−0.9998,−2.3120

σ(A+(α[3])BF0)=−1.0581   ,−2.312.

σ(A+(α[3])2BF0)=−1.0968  ,−2.3120  .

σ(A+(α[3])3BF0)=−1.1359   ,−2.312.

Finally, the dynamics amelioration is guaranteed by the choice of this regulator. The state and the control components for time varying control, piece-wise control [3] and for a fixed gain chosen to be F0, the initial gain is represented in figure1 and figure2 respectively.

fig 1 : Space state

Fig2: Control evolution

Example2

Consider the system (1) with:

A=1−2−30.45−442−0.915 , B=10.20.3−10−0.5

Matrix A is unstable,

i.e, σ(A)=1.5046,14.2937,−3.7983  .

q=372625.87  T.

We choose α=0.1 and β=2.

Let:

H(0)=−0.400−0.3

By applying the algorithm, the resolution of equation (31) gives:

F0=−4.54431.9614−17.47743.9817−1.770631.0186

and

F∞=(1+β)F0=−13.63295.8842−52.432211.9451−5.311893.0558If we choose ε=0.1, according to (32), we obtain:

H(∞)=−8.511811.73696.9010−26.3847

With :σ(H(∞))=−4.7653,−30.1312 and :

H(∞)q=−9.772−430.6652−137.4461−6.6471T            ≤−εq

Finally, we obtain the following results:

σ(A+BF0)=−0.6,−0.5,−3.7984.σ(A+(1+β(1−e−αt))BF0)t=10=−19.2358,−3.2275,−3.7984.σ(A+(1+β)BF0)=−30.1312,−4.7653,−3.7984.

Note that  -3.7984 is a common eigenvalues of A, A+BF0 and A+(1+β)BF0.

Furthermore,

Re(λi(H(∞))≤Re(λi(H(0)), i=1,…,m

Which means that in the control, the dominant eigenvalues of H(∞) is more stable than the eigenvalues of H(0).

According to the result given in [3], we choose N=3 and a diagonal matrix H0 such that F0A+F0BF0=H0F0 and H0q≤−εq, which implies from (31) that:

H0=H(0)−αβI=−0.600−0.5

From [3], we obtain 0<α[3]≤1.0260, if we choose α[3]=1.025, we obtain the following results, with:

σ(A+BF0)=−0.6,−0.5,−3.7984

σ(A+(α[3])BF0)=−0.6361,−0.9117,−3.7983.σ(A+(α[3])2BF0)=−0.6955,−1.3119,−3.7983σ(A+(α[3])3BF0)=−0.7546,−1.7247,−3.7983

Then, compared to the results given in [3], the dynamics amelioration with a time-varying regu1ator is guaranteed and is better than that derived in [3].

The state and the control components for time varying control, piece-wise contro1 [3] and for a fixed gain chosen to be F0, the initial gain is represented in figure3 and figure4 respectively.

fig 3: Space state

fig.4: Control Evolution

# Appendix 1:  Matrix norm M∞:

The matrix norm given by the vector norm:

z∞=maximaxzi+q1i,zi−q2i

is giving by

M∞=maxz∞=1Mz∞

then :

Mz∞=maximax(Mz)i+q1i,(Mz)i−q2i

For this, we use the result of [2b,c]

Mz∞=maximax(M+q1)i+(M−q2)iq1i,(M−q1)i+(M+q2)iq2iz∞Thus,M∞=maximaxj=1mq1jq1imij++q2jq1imij−;j=1mq1jq2imij−+q2jq2imij+

6 CONCLUSION

In this paper, a time varying regulator is derived for linear continuous time systems. Necessary and sufficient conditions for domain D(F(t),q1,q2)to be a positively invariant set w.r.t. system (6) are given. The proposed technique guarantees the admissibility of the control and enables system in the closed loop to admit the largest non-symmetrical constrained control. The asymptotic stability of the origin is also guaranteed. The results have been shown to be better than the literature ones.

REFERENCES

[1] Benzaouia A. and A.Hmamed, (a)" Regulator problem for Linear continuous time systems with nonsymmetrical constrained control using non-symmetrical Lyapunov function," In Proc.3th CDC IEEE-Arizona, 1992; (b )"Regu1ator problem for continuous time systems with nonsymmetrica1 constrained control, "IEEE Trans.Aut. Control. vol.38, no10, pp 1556-1560, October 1993.

[2] Benzaouia A. and C.Burgat, (a)" Regu1ator problem for linear discrete-time systems with nonsymmetrica1 constrained control", Int.J.cont, vo1.48, n06,pp.244-245, 1988; (b)" Existence of nonsymmetrica1 Lyapunov functions for systems," Int.syst.Sci., vo120,pp 597-607, 1989; (c)" Existence of non-symmetrical stable domains for linear systems," Linear A1gebra Appl., vo1.l21,pp.217-231,1989.

[3] Benzaouia A. and A.Baddou," Piecewise linear constrained contro1 for continuous-time systems," IEEE Trans. Aut. Control, Vol. 44, no. 7, pp. 1477, July, 1999.

[4] Benzaoiua A.," Application du concept d'invariance positive à l'étude des problèmes de commande des systèmes dynamiques discrets avec

contraintes sur la commande". Thesis of UPS LAAS, No 88322, 1988.

[5] Bistoris G.,"Existence of positively invariant polyhedral sets for continuous time linear systems" Control theory and advanced technology. voI.7,no3,ppA07-427, September 1991.

[6] Wredenhagen G.F. and P.R.Bélanger," Piecewise linear LQ control for systems with input constraints," Automatica, vol.30, no3,pp. 403-416,

1994.

[7] Benzaouia A., "The resolution of equation XA+XBX=HX and the pole assignment problem" IEEE. Trans.on aut.Cont. vo1.39, nol0, pp2091-2094 ,October 1994.

[8] B.Porter, "Eigenvalue assignment in linear multivariable systems by output feedback," InU.contr.,voI.25, no.3, ppA83-490, 1977.

[9] W. Hahn, Stabilty of Motion, Berlin: Springer-Verlag, 1967.

[10] P.O. Gutman and P.A. Hagander, "New design of constrained controllers for liner systems," IEEE Trans. Automat. Contr., vol. AC-30, pp. 22_23,

1985.

[Il] M. Vassilaki and G. Bistoris, "Constrained regulation of linear continuous-time dynamical systems," Syst. Contr. Lett., vol. 13, pp. 247-252, 1989.

[12] C.A., Desoer and M. Vidyasagar, "Feedback Systems: Input-Output Properties (New York: Academic Press), 1975.

[13] Min-Yen Wu, " On stability of linear time-varying systems", CDC-IEEE, pp. 1211-1214, 1982.

[14] Makoudi M and Radouane L. (1992). On decentralized discrete time varying feedback control. Advances in modelling and analysis C, vol.33, NA, pp.29-38.

[15] Makoudi M and Radouane L. (1991). On decentralized time varying feedback control of linear continuous systems. Troisième colloque magrébin sur les modèles numériques de l'ingénieur. November, 26-29, Tunis.

[16] Anderson B.O.D., Moore LB., 1981, "Time-varying feedback laws for decentralised control", IEEE. Trans. Autom. Control, vo1.26, N5, 1133.