Unit Commitment Using Embedded Greedy Search Particle Swarm Optimization with Mutation Operation

SUN LIYONG, ZHANG YAN, JIANG CHUANWEN

Department of Electrical Engineering

Shanghai Jiaotong University

1954 Huashan Road, Shanghai

CHINA

Abstract: A hybrid particle swarm optimization (PSO) is presented for solving unit commitment (UC) problems. Using fixed threshold to cope with unit status variables, the proposed algorithm can directly solve UC and avoid coping with economic dispatch (ED) problems. Mutation operation is used to renew slow evolution particles for enhancing the algorithm’s performance. The greedy search based on priority list is applied to improve the search speed and the solutions quality. The proposed algorithm is tested in 10 up to100 unit systems. The results show that the proposed algorithm for UC is feasibility and efficiency.

Key words：Unit commitment; Economic dispatch; Particle swarm optimization; Greedy search; Mutation operation; Priority list

1 Introduction

The unit commitment (UC) plays an important role in the economic operation of a power system, which is aimed at scheduling the unit on/off status and real power output to meet the load demand over a horizon period. The resultant generation schedule should minimize the system production cost while satisfying system and unit constraints.  Mathematically, the UC is a large-scale, non-convex, nonlinear, mixed-integer optimization problem. The best solution to UC can be obtained by complete enumeration, but the excessive computational resource requirement is impossible in practice. Various approaches have been proposed to solve UC. These approaches consist of two species, deterministic and Meta-heuristic techniques [1].

Deterministic techniques include priority list [PL], dynamic programming [DP], integer programming, Lagrangian relaxation [LR], and so on. Among these techniques, PL is simple and fast, but the final solution is quite far from the optimum. DP and integer programming suffer from the “curse dimensionality” when the size of system is large. LR is considered the most efficient method for large-scale system. Nevertheless, using LR to cope with the UC gives rise to some technical problems, such as “duality gap”.

Meta-heuristic techniques include simulated annealing [SA] [2], expert system [ES] [3], evolutionary program [EP] [4], genetic algorithm [GA] [5,6], and so on. Among these methods, SA can theoretically converge asymptotically to a global optimum solution, but the SA approach for solving the UC takes long CPU time [2]. The knowledge of the ES used to solve UC is extracted from experienced system operators, but much interaction with the system operator for improving the UC solution is time-consuming [3]. EP has been used to solve UC, but the final result isn’t superior to other methods [4]. Although GA has been employed successfully to solve the UC, it must cope with the ED in each period [5,6].

Particle swarm optimization (PSO) is one of the evolutionary computation techniques and has been found to be robust in solving continuous nonlinear optimization problems [7,8]. In this paper, a novel embedded greedy search PSO with mutation operation is presented for solving UC. After using fixed threshold to cope with unit status variables, the proposed algorithm can solve UC and avoid coping with ED. The mutation operation and the greedy search are used to enhance the algorithm’s performance. Simulation results demonstrate the feasibility and efficiency of the proposed algorithm.

2 Problem Formulation

2.1 Notation

N		Number of units.

i 		Unit index (i=1,…,N)

T		UC horizon.

t    	Hour index (t=1,…,T)

D(t) 	Demand during hour t (in MW)

R(t) 	Reserve requirement during hour t (in MW)

Pimax 	Maximum power output of ith unit (in MW)

Pimin 	Minimum power output of ith unit (in MW)

URi 	Up ramp limit of ith unit (MW/hour)

DRi 	Down ramp limit of ith unit (MW/hour)

Pi(t) 	Power output of ith unit for hour t (in MW)

FCi(t) 	Cost function of ith unit (in dollars)

SCi(t) 	Start-up cost of ith unit (in dollars)

TC		Total generation cost of system (in dollars)

Tion 		Minimum uptime of ith unit (in hour)

Tioff 		Minimum downtime of ith unit (in hour)

Xion  	Duration time during which ith unit is continuously ON

Xioff 	Duration time during which ith unit is continuously OFF

CSTi 	Cold start time of ith unit (in hour)

HSCi 	Hot start cost of ith unit (in dollars)

CSCi 	Cold start cost of ith unit (in dollars)

ui(t) 	Unit operating status of ith unit for hour t (1=ON, 0=OFF)

2.2 Unit Commitment Formulation

The objective of UC is to schedule the operation of generation units at minimum operating cost over a given scheduling period, while satisfying the load demand, the spinning reserve, and physical and operational constraints of the individual units.

2.2.1 Objective Function

The objective function of UC problem is

minTC=t=1Tn=1Nui(t)FCi(t)+SCi(t)           (1)

The production cost, FCi (t), of a committed unit i, is conventionally taken in a quadratic form:

FCi(t)=ai+biPi(t)+ciPi2(t)                  (2)

where ai, bi, ci are fuel cost coefficients.

In this paper, start up cost is taken as follows:

SCi(t)=HSCiifTioff≤Xioff≤Tioff+CSTi(3)

2.2.2 System Constraint

(a) Load balance:    i=1Nui(t)Pi(t)=D(t)       (4)

(b) Spinning reserve: i=1Nui(t)Pimax≥D(t)+R(t)  (5)

2.2.3 Unit Constraint

(a) Generation limit:   Pimin≤Pi(t)≤Pimax     (6)

(b) Ramp rate constraint:

Pi(t−1)−DRi≤Pi(t)≤Pi(t−1)+URi            (7)

(c) Minimum up-time constraint:	   Tion≤Xion  (8)

(d) Minimum down-time constraint: 	Tioff≤Xioff(9)

3 Problem Solution

The basic idea of the proposed algorithm is to use PSO to search the global optimal solution. Mutation operation is used to renew slow evolution particles. Greedy search is applied to accelerate the searching speed in last dozens of generations.

3.1 Representation of Candidate Solution

Each candidate solution, namely generation schedule, is represented by a real number matrix G (Fig.1).

12⋯t⋯TG=C1,1C1,2⋯C1,t⋯C1,TC2,1C2,2⋯C2,t⋯C2,T⋮⋮⋯⋮⋯⋮Ci,1Ci,2⋯Ci,t⋯Ci,T⋮⋮⋯⋮⋯⋮CN,1CN,2⋯CN,t⋯CN,T12⋮i⋮N

Fig.1 Candidate solution representation (matrix G)

The matrix contains all information of generation schedule. The detail is described as follows. The element Ci,t is represented the power output of ith unit in the tth hour. The row vector Ri is represented the generation schedule of ith unit during schedule period. The column vector Vt is represented the ED solution in the tth hour among N units.

3.2 Variable and Constraint Handling

3.2.1 Unit Status Variable Handling

The status of ith unit in the tth hour is determined according to Ci,t value, describing as follows

ui(t)=1ifCi,t≥λPimin          (10)

where is constant number in range of (0,1), which is equal to 0.8 in this paper.

3.2.2 Generation Limit and Ramp Rate Constraint Handling

Ci,t=Pi,maxifCi,t>Pi,maxCi,tifPi,min≤Ci,t≤Pi,max    (11)

where Pi,max=min{ Pimax, Ci,t-1+URi} and Pi,min=max{ Pimin, Ci, t-1-DRi}.

3.2.3 Other Constraints Handling

(a) Spinning reserve of system can be satisfied through adjusting the status of units, namely a random unit of off-status is started up untili=1Nui(t)Pimax≥D(t)+R(t).

(b) The load of tth hour is re-dispatched among N units according to following steps:

Step1: Select an on-status unit in Vt randomly.

Step2: Increase or decrease power output of the selected unit to satisfy the system load demand of tth hour until i=1Nui(t)Pi(t)=D(t) or the power output of the selected unit reaches its generation output limit.

Step3: If i=1Nui(t)Pi(t)=D(t), the adjustment of (b) is finished, else select another unit of on-status in Vt and go to Step2.

(c) When the algorithm is near to convergence, minimum up-time and down-time constraints can be satisfied through adjusting unit status. The detail is that the on-status duration of ith unit is extended if Tion>Xion, and if Tioff>Xioff the off-status is converted to on-status during the off-status duration of ith unit.

3.3 PSO with Mutation Operation

PSO developed by Eberhart et al. is a multi-agent search technique that traces its evolution to the motion of a flock of birds searching for food [7,8]. It uses a number of particles that constitute a swarm and each particle traverses the search space looking for the global optimal solution.

3.3.1 Particle Initiation

The real number matrix G representing the solution is taken as particle. Each particle is initiated according to the sequence of column vectors, and each element of the column vector is created basing on the random number. The details of the initiation of Vt are described as follows

Step1: A vector, RAND which consists of random positive numbers, is generated firstly. The RAND can be expressed as RAND=[rand1, …, randi, …, randN],   where i=1,2,…,N.

Step2: A percent vector, PER which comprises decimal fractions, is created, and can be described as

PER=[per1,…,peri,…,perN]

where peri=randii=1Nrandi, i=1,2,…,N.

Step3: The column vector Vt is initiated, and can be described as Vt=[C1,t, …, Ci, t,…, CN, t]T ,                                          where  Ci, t= peri D(t) , i=1, 2,…, N.

3.3.2 Iterative Search Method

Let C (element of matrix G) and v denotes a particle position and its corresponding flight speed in search space, respectively. The best previous position of a particle is recorded and represented as Bp. The index of the best particle among all the particles in the group is represented as Bg. The modified flight velocity and position of each particle can be calculated as follows

vi,tm(d+1)=k(w∗vi,tm(d)+ϕ1∗rand()∗(Bi,tp(d)−Ci,tm(d))

+ϕ2∗rand()∗(Bi,tg(d)−Ci,tm(d)))    (12)

Ci,tm(d+1)=Ci,tm(d)+vi,tm(d+1)                (13)

where d is pointer of generations, Cm(d) is current positing of mth particle at the dth generation, v is velocity of mth particle at dth generation, rand( ) is a uniform random value in the range [0,1]. The acceleration coefficients, ϕ1 and ϕ2, control how far a particle will move in single iteration. The coefficient k is the constriction factor which is function of ϕ1 and ϕ2 as showed in followed:

k=22−ϕ−ϕ2−4ϕ                       (14)

where ϕ=ϕ1+ϕ2 and ϕ>4.

The inertia weight factor w is used to control the convergence of PSO. In this paper, it was set according to the following equation:

w=wmax−wmax−wminitermaxiter                  (15)

where itermax is the maximum number of generations, and iter is the current number of generations.

The value of each dimension of every velocity vi,t is clamped to the range [-vi,max,vi,max] to reduce the probability of the particle leaving the search space. The vi,max is set to 0.15*(Pimax- Pimin) in this paper.

3.3.3 Mutation Operation

The mutation operation is adopted to improve slow evolution particles during iterations. The detail is described as follows.

The particle before mutation and after mutation is respectively denoted Gp=[R1p,R2p,…,Rip,…,RNp]T and Gm=[R1m,R2m,…,Rim,…,RNm]T. The Gm was created according to the following equation:

Rim=(1−α)Ri1pifi=i1Ri2p+αRi1pifi=i2                  (16)

where α is a random value in the range (0,1) and i=1,2,…,N. Ri1p and Ri2p are two different row vectors selected from Gp randomly.

The Gp is replaced by Gm if Gm is superior to Gp. Mutation operation is imposed on particles which haven’t improved the Bp for 4 continuous iterations.

3.4 Greedy Search

Owing to the large dimensions of UC, the search speed of the PSO with mutation operation is decreasing when near to the optimal solution. The greedy search is used to cope with this problem in this paper. And during the last 20 iterations, greedy search operations are employed to all the local best solution, namely the group of Bp.

3.4.1 Creating Priority List

Priority list is created based on each unit parameters. Unit priority is created as descending order maximum power output. In addition, the priority of equal maximum power output units are created according to the ascending order of heat rate (hr). The hr of ith unit is given by the following equation:

hri=FCi(Pimax)Pimax                         (17)

3.4.2 Greedy Search Operation

Two continuous time period during which the unit status is change is defined search area of greedy search. In this paper, the greedy search includes two operations, conversion and shifting. Fig. 2 shows the sketch of greedy search operations and the shaded areas in Fig. 2 mean search area.

Fig. 2 Greedy Search Operations

(a) Conversion operation. If the unit status converts 1 to 0 while satisfying spinning reserve and minimum up/down-time constraints, the unit status will be changed from 1 to 0.

(b) Shifting operation. This operation must satisfy the following conditions:

ⅰThe lower priority unit status converts 1 to 0 while satisfying spinning reserve and minimum up/down-time constraints.

ⅱThere is higher priority unit in the same time period whose status can convert 0 to 1 while satisfying minimum down-time constraint.

If the conditions are satisfied, the power output of the lower priority unit is shifted to the higher priority unit while changing the two units’ status.

If the particle is superior to Bp after greedy search operation, Bp is replaced.

3.5 Simulation

The parameters of the algorithm are set as follows: the number of particles is 20, generations=50*N, ϕ1＝ϕ2＝2.05, inertia weight factor w=0.7295.

The proposed algorithm has been tested with 10 up to 100 unit system. The 10-unit data and 24-hour load demand are shown in Table 1 and Table 2, respectively. For the 20-unit configuration, the initial 10 unit system data were duplicated. The 40-unit to 100-unit systems were created in the same way. The reserve requirement was 10% of the hourly load. Considering stochastic nature of the algorithm, 20 test trials were made for each case.

The result solved by the proposed algorithm is compare to other algorithms in Table 3. Fig. 3 shows the execute time of the algorithm. Fig. 4 gives the solution distribution of 10 unit system solved by the algorithm with greedy search and without.

Table 1 Parameters of 10-unit

| Unit no. | Pimax | Pimin | ai | bi | ci | Tion | Tioff | HSCi | CSCi | CSTi | Initial hour |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 455 | 150 | 1000 | 16.19 | 0.00048 | 8 | 8 | 4500 | 9000 | 5 | 8 |
| 2 | 455 | 150 | 970 | 17.26 | 0.00031 | 8 | 8 | 5000 | 10000 | 5 | 8 |
| 3 | 130 | 20 | 700 | 16.6 | 0.002 | 5 | 5 | 550 | 1100 | 4 | -5 |
| 4 | 130 | 20 | 680 | 16.5 | 0.00211 | 5 | 5 | 560 | 1120 | 4 | -5 |
| 5 | 162 | 25 | 450 | 19.7 | 0.00398 | 6 | 6 | 900 | 1800 | 4 | -6 |
| 6 | 80 | 20 | 370 | 22.26 | 0.00712 | 3 | 3 | 170 | 340 | 2 | -3 |
| 7 | 85 | 25 | 480 | 27.74 | 0.00079 | 3 | 3 | 260 | 520 | 2 | -3 |
| 8 | 55 | 10 | 660 | 25.92 | 0.00413 | 1 | 1 | 30 | 60 | 0 | -1 |
| 9 | 55 | 10 | 665 | 27.27 | 0.00222 | 1 | 1 | 30 | 60 | 0 | -1 |
| 10 | 55 | 10 | 670 | 27.79 | 0.00173 | 1 | 1 | 30 | 60 | 0 | -1 |

Table 2 Load Demand of 24-hour

| Hour | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Load | 700 | 750 | 850 | 950 | 1000 | 1100 | 1150 | 1200 | 1300 | 1400 | 1450 | 1500 |
| Hour | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| Load | 1400 | 1300 | 1200 | 1050 | 1000 | 1100 | 1200 | 1400 | 1300 | 1100 | 900 | 800 |

Table 3 Results analysis of different methods from 10 to 100 unit Systems ($)

| Unit no. | EP[4] | GA2[5] |  | GA3[6] | The Proposed Algorithm |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Avg. | Best | Worst | Avg. | Best | Worst | Avg. |
| 10 | 565352 | 563977 | 565606 | 566404 | 563977 | 564986 | 564487 |
| 20 | 1127257 | 1125516 | 1128790 | 1127244 | 1124648 | 1124747 | 1124664 |
| 40 | 2252612 | 2249715 | 2256824 | 2254123 | 2245784 | 2247737 | 2246233 |
| 60 | 3376255 | 3375065 | 3382886 | 3378108 | 3367409 | 3369111 | 3367945 |
| 80 | 4505536 | 4505614 | 4527847 | 4498943 | 4489288 | 4491916 | 4490336 |
| 100 | 5633800 | 5626514 | 5646529 | 5630838 | 5610918 | 5616382 | 5612844 |

Fig. 3 Execute Time of the Proposed Algorithm

Fig. 4 Solution Distribution of 10 Unit System

4 Conclusion

This paper presents a new solution to the unit commitment. Owing to taking fixed threshold to cope with unit status variables, the proposed algorithm can solve UC problems directly, thus avoid solving ED problems in each hour. Mutation operation and greedy search are embedded in PSO to accelerate the searching speed and enhanced the algorithm performance. The results demonstrate the proposed algorithm for UC is feasibility and efficiency.

References

- H.Y. Yamin, Review on methods of generation scheduling in electric power systems, Electric Power System Research, Vol.69, No.2-3, 2004, pp. 227-248

- A.H. Mantaway, Y.L. Abdel-Magid, S.Z. Selim, A simulated annealing algorithm for unit commitment, IEEE Trans. Power System, Vol.13, No.2, 1998, pp.197-204

- X. Bai, S.M. Shahidehpour, Extended neighborhood search algorithm for constrained unit commitment, Int. J. Electric Power Energy System, Vol.19, No.5, 1997, pp.349-356

- K. A. Juste, H. Kita, E. Tanaka, J. Hasegawa, An evolutionary programming to the unit commitment problem, IEEE Trans. on Power System, Vol.14, No.4, 1999, pp.1452-1459

- T. Senjyu, H. Yamashiro, K. Uezato, T. Funabashi, A unit commitment problem by using genetic algorithm based on unit characteristic classification, IEEE PES Winner Meeting, Vol.1, 2002, pp.58-63

- I. G. Damousis, A. G. Bakirtzis, P. S. Dokopoulos, et. al, A solution to the unit-commitment problem using integer-coded genetic algorithm, IEEE Trans. on Power System,  Vol.19, No.2, 2004, pp.1165-1172

- J. Kennedy and R. Eberhart, Particle swarm optimization, Proc. IEEE Int. Conf. Neural Netw., Vol.4, 1995, pp.1942-1948

- R. C. Eherhart and Y.Shi, Comparing inertia weights and constriction factors in particle swarm optimization, Proc. Congr. Evol. Comput., Vol.1, 2000, pp.84-88