A New Reduced-Order Sliding Mode Observer Design Method:

A Triple Transformations Approach

Elbrous M. Jafarov

Istanbul Technical University, Aeronautical and Astronautical Eng. Department,

80626 Maslak, Istanbul, TURKEY

Abstract: In this paper, a triple state and output variable transformations based method to design a new reduced-order sliding mode observer for perturbed MIMO systems is developed. The state and output variables of original system is triple transformed in to suitable canonical form coordinates where the dynamical reduced order observer can be successfully designed. Existing reduced-order observer design techniques and state-output variables transformations are summarized in this study and presented systematically. A new combined observer configuration is proposed. Some new adequate evolution of matrix inequalities is adopted. Global sufficient asymptotical stability and sliding conditions for the coupled observer error system are established by using Lyapunov full quadratic form and formulated in terms of Lyapunov matrix equations and matrix inequalities. Reduced analysis of separated reaching and sliding modes of motion of decoupled observer error system is discussed also. A numerical example is given to illustrate the usefulness of proposed design method.

Keywords: Reduced order observer; Triple state transformations; Sliding mode control; Uncertain dynamical systems.

1 Introduction

The observer for linear systems was first proposed and developed by Luenberger [1]. An observer that estimates all of the state variables is called a full-order observer. But, an observer, that estimates a part of the state variables referred to be a reduced-order observer.

In recent years, the state observation problem of uncertain dynamical systems subject to external disturbances has been a topic of considerable interest. Variable structure control with a sliding mode is an established method for control and observation of uncertain dynamical systems. There are several modification of discontinuous state observers which were successfully designed by Utkin [2]; Hashimoto, Utkin, Xu, Susuki and Harashima [3]; Walcott and Zak [4]; Edwards and Spurgeon [5]; Sira-Ramizer, Spurgeon and Zinober [6]; Young, Utkin and Özgüner [7]; Watanabe, Fukuda and Tzateftas [8]; Slotine, Hedrick and Misawa [9]; Mielczarski [10]; Jafarov [11] etc.

Moreover, some new configuration of Utkin reduced-order observer for canonical systems without external disturbances, Walcott-Zak full-order observer for MIMO systems with external disturbances and Edwards and Spurgeon reduced order observer for canonical MIMO systems with external disturbance have been successfully designed by Edwards and Spurgeon [12]. The min-max observer control term with non-linear gain parameters is used for stabilization of observer error systems. These types of observers are designed by Lyapunov V-function method such that observer state error dynamics is globally asymptotically stable or globally uniformly ultimately bounded because some times the sliding and stability regions are restricted by some small ball [4]. Recent advances in design of sliding mode controllers and observers are presented by Jafarov and Tasaltin [13], [14]; Choi [15]; Yeh, Chien and Fu [16]; Singh, Steinberg and Page [17]; Sabanovich, Fridman and Spurgeon [18]; and special journal issues [19] and [20].

In this paper, a triple state and output variable transformations based method to design a new reduced-order sliding mode observer for perturbed MIMO systems is developed. The state and output variables of original system is triple transformed in to suitable canonical form coordinates where the dynamical reduced order observer can be successfully designed. Existing reduced-order observer design techniques and state-output variables transformations are summarized in this study and presented systematically. A new combined observer configuration is proposed. Some new adequate evolution of matrix inequalities is adopted. Global sufficient asymptotical stability and sliding conditions for the coupled observer error system are established by using Lyapunov full quadratic form and formulated in terms of Lyapunov matrix equations and matrix inequalities. Reduced analysis of separated reaching and sliding modes of motion of decoupled observer error system is discussed also. A numerical example is given to illustrate the usefulness of proposed design method.

Further, we will use the following notation:

║x║=xTx  is the Euclidean norm; ║A║=λmax(ATA) is matrix norm; T is the transpose of a vector or matrix; Rayleigh’s min-max matrix inequality for a positive definite matrix P:

0 <λmin(P)║x║²  xTPx   λmax (P)║x║²;

where λmin (P) and λmax(P) are minimum and maximum eigenvalues of the matrix P, respectively.

2 Reduced-order observer configuration

2.1 System description and assumptions

Consider the original uncertain MIMO system described by the following state space equations:

x(t)=Aox(t)+Bou(t)+f(t,x)

y(t)=Cox(t)				      (1)

where x∈Rⁿ is an unmeasured state vector, u∈Rm is a control input and y∈Rpis a measured output vector with m  p  n. Nominal linear system (Ao, Bo, Co) is known and input and output matrices Boand Co are both of full rank and p = m.

We need to make the following conventional assumptions used in sliding mode control theory

a) The pair (Ao,Bo) is completely state controllable, that  is

rank[BoAoBoAo2Bo…Ao(n−1)Bo]=n	      (2)

and pair  (Ao,Co) is observable, respectively.

b) f(t,x) is the unknown n-vector function, which represents the system nonlinearities plus any model uncertainties and external disturbances, satisfies the conventional matching condition [12]:

f(t,x)=Boξ(t,x)				(3)

where the function ξ(t,x): R+ xRⁿ →Rm is unknown lumped function, but norm-bounded:

║ξ(t,x)║ρ +μ║y(t)║ρ+μλmax(C0TC0)x(t)  (4)

where ρ and μ are given positive scalars.

Thus, the system (1) can be rewritten as

x(t)=Aox(t)+ Bo[u(t)+ ξ (t,x)]

y(t)= Cox(t)	     		 	      (5)

The problem to be considered is that of reconstructing the unknown state variables x(t) using only measured output information y(t).

First we consider the system described by (5) under above mentioned assumptions that the pair (Ao,Bo) is controllable. As the outputs are to be considered, it is logical to effect a change of coordinates so that the outputs appear as components of the state coordinates.

2.2 Triple state transformations

First transformation: Original system (5) can be transformed by the following similarity transformation [21]-[23]:

z=z1z2=T1x				       (6)

into following conventional controllable canonical form:

z1.=A111z1+A121z2				       (7)

z2.=A211z1+A221z2+B21(u+ ξ)			       (8)

y1=Fy=C1z1+C2z2				       (9)

where

T1AoT1−1=MWAo[ U   V ] = MAo UMAoVWAoUWAoV

=A111A121A211A221= A1;

T1Bo=MWBo=MBoWB0=0B21=B1;		     (10)

FC0T1−1=FCo[ U  V ]=[  FC0U FC0V ] =[ C1 C2];

T1T1−1=MW[ U V ]= MUMVWUWV=In−p00Ip;

T1−1T1=[ U V ]MW=UM+VW=In

z1is a (n-p)-vector

z2 is a p-vector

M is a (n-p)n of full rank matrix

W is a pn-matrix

U is a n(n-p)-matrix

V is a np-matrix

B21 is a pp-matrix because p = m

C1 is a design p(n-p)-matrix

C2 is a design pp matrix of full rank and non-singular

In is a nn-identity matrix.

F is an mm-output design matrix satisfying structural constraint [12]:

C2=FCoV=B21TP2			(11)

where P2 is a positive definite (p×p)-matrix to be selected.

Second transformation: This state transformation is given by similar to [24]:

T2z1z2=x1y1 		  	(12)

where x1 is an unmeasured state (n-p) vector. This transformation leads system  (7) – (9) to the following

controllable canonical form in space [x1, y] :

x1=A112x1+A122y1

y1=A212x1+A222y1+B22(u+ξ)			    (13)

where

T2A1T2−1=A112A122A212A222=A2; T2=In−p0C1C2,

T2B1=In−p0C1C20B21=0C2B21=0B22=B2;    (14)

where B22 is a (pp)-matrix.

Note that, T2 is non-singular, because C2 is a non-singular.

T2−1=In−p0-C2-1C1C2-1, T2T2−1=In−p00Ip	    (15)

[ C1  C2  ]T2−1=[   0    Ip  ]		    (16)

which are therefore correct.

Therefore, from (14)-(16) we calculate

T2A1T2−1=In-p0C1C2A111A121A211A221In-p0-C2-1C1C2−1

=In-pA111In-pA121C1A111+C2A211C1A121+C2A221In-p0-C2-1C1C2−1

=In-pA111In-p−In-pA121C2-1C1In-pA121C2-1(C1A111+C2A211)In-p(C1A121+C2A221)C2-1

=A112A122A212A222=A2				    (17)

Hence

A112=A111−A121C2-1C1			    (18)

and the design matrices C1 or C2can be selected such that the matrix A112 always has stable desirable eigenvalues.

In other words, in terms of the structural conditions the second transformation means that the following conversion is true. Indeed, from (9) we have

z2=−C2−1C1z1+C2−1y1				    (19)

which acts in (7) as control input. Then

z.1=(A111−A121C2−1C1)z1+A121C2−1y1		     (20)

Therefore in terms of [x1y] we have the same results (13):

x.1=A112x1+A122y1				     (21)

where A112 always has stable desirable eigenvalues.

In order to verify the first and second transformations in terms of output equation let us evaluate the output variables in various state coordinates :

y1(t)=Fy(t)=FCox(t)=C1z1+C2z2

C1C2z1z2 [C1  C2 ] T2−1x1y1

= [  C1  C2  ] In−p0-C2-1C1C2-1x1y1

=[ 0  Ip ]x1y1y1(t)			     (22)

Therefore, the first and second transformations are true.

Observer configuration: Now, let us form a simple reduced order observer, structure of which is similar to the twice transformed system (13) but is different from [12] :

x1=A112x1+A122y1

y1=A212x1+A222y1+B22u-B22v		     (23)

where x1 and y1 are the new state estimates vector of the observer; v is the observer control vector term to be selected.

If the error between the estimates and the true states are written as  e1=x1−x1 and observer residual:

r(t)=y1−y1				     (24)

then the following observer error system can be obtained from (13) and (13) as follows:

e1=A112e1+A122r
r=A212e1+A222r+B22v+B22ξ			     (25)

Now, our goal is to construct an observer control term such that on the formed sliding surface defined for the error system (25) at the finite time generated be asymptotically stable sliding motion. Then the unmeasured reduced state x1(t) will be corrected by the observer. A remaining part x2(t) or z2(t) will be calculated from (19).

2.3 Observer control term

Let us select observer control term of the form:

v(t)=vlin(t)+vvsc(t)−Ls(t)−δ+ky1(t)(B22TP2B22)−1s(t)s(t)	   (26)

where L is an observer gain (p×p)-matrix, δ and k are some scalars to be designed; s(t) is the switching function to be defined. The observer combined control term (26) consists of two parts: 1) linear Luenberger control term vlin(t) for compensation of linear perturbation portion of error system (25); 2) variable structure control term vvsc(t) for compensation of lumped external disturbance.

The switching function can be defined as:

s(t)=B22TP2r(t)			    (27)

where P2 is a (pp)-positive definite design matrix to be determined; from (14), (10) B22=CB21=B21TP2B21

=B22T>0 is a positive definite (pp)-matrix.

Third transformation: Thus, we have constructed the a reduced-order observer simple observer (23) and control term (26) with the sliding surface s(t)= 0 (27) defined for the observer error system (25) in space of e1  r. Now, on the formed sliding surface s(t)= 0 can be organized the stable sliding mode. For this reason, it is rational to transform the observer error system (25) into e1  s state-space form [25] by the third change of state and residual coordinates:

T3e1r=e1s				    (28)

where

T3=In−p00B22TP2.			    (29)

Then, at last after triple coordinate transformations we obtain an observer error system in suitable canonical state-space form of  [e1  s] :

e1=A11e1+A12s

s.=A21e1+A22s+B2v+B2ξ			   (30)

where

T3A2T3−1=A11A12A21A22=A;T3B2=In−p00B22TP20B22=0B22TP2B22=0B2=B			   (31)

B2=B22TP2B22=B2T>0 is a positive definite matrix.

T3−1=In−p00B22TP2−1, T3T3−1=In−p00Ip  (32)

Then, from (28) and (31) we calculate

T3A2T3−1

=In−p00B22TP2A112A122A212A222In−p00B22TP2−1

=A112A122B22TP2−1B22TP2A212B22TP2A222B22TP2−1=A11A12A21A22

=A		   				     (33)

Hence we see that A11=A112 and therefore A11 has always stable desirable eigenvalues because of (17) and (18). Moreover

A21=B22TP2A212=B22TP2(C1A111+C2A211)	   (34)

In particular, C2 or C1 can be selected such that

A211=−C2−1C1A111 and A21=0	    (35)

Thus, the reduced-order observer error system (30), (26) can be presented in the canonical state-space form of [e1   s]:

e1(t)=A11e1(t)+A12s(t)				     (36)

s(t)=A21e1(t)+A22s(t)−δ+ky1(t)B2(B22TP2B22)−1s(t)s(t)+B2ξ(t,x)

with sliding surface 	s(t)=0 		     (37)

where  A22=A22−B2L is stable matrix. Desirable eigenvalues of which can be assigned by pole placement method.

3 Stability analysis of observe error system

The sufficient conditions for global asymptotical stability of the observer error system (36) at the point e1=0 with a stable sliding mode on s(t)=0 (37) are established by using Lyapunov V-function method and formulated in terms of Lyapunov matrix equations and matrix inequality. The following theorem summarizes our stability and sliding results.

Theorem 1: The coupling observer error system (36) is globally asymptotically stable and in general on the formed sliding surface s(t)=0 (37) always is generated a stable sliding mode, whenever there exist a family of symmetric positive definite design matrices P1, P2 and Q1, Q2, Q such that the following conditions are satisfied:

P1A11+A11TP1=−Q1;  Q1>0			    (38)

P2A22+A22TP2=−Q2; Q2>0			    (39)

Q=Q1−(P1A12+A21TP2)−(P1A12+A21TP2)TQ2>0	  (40a)

or its Schur complement :

Q=Q1−(P1A12+A21TP2)Q2−1(P1A12+A21TP2)T>0      (40b)

(40c)

kλmin(P2)=μλmax(F−TF−1)λmax(P2B2)                    (41)

Proof:  Choose a Lyapunov full quadratic form of coordinates of  [e1  s] as follows:

V(e1,s)=e1sTP100P2e1s             =e1TP1e1+sTP2s		   (42)

where P1 is a(n-p)(n-p)-matrix, P1 is pp-matrix, which are positive definite. Then, the time derivative of  (42) along the trajectory of the observer error system (36) can be calculated as follows:

(43)

SinceB2=B22TP2B22a is a positive definite matrix, then we can present a feedback gain matrix in (26) also as:

(44)

where δ1,...,δm and k1,…,km  are some gain constants to be selected.

Since, in space of  [e1,y1] :

ξ(t,x)≤ρ+μλmax(F−TF−1)y1(t)		    (45)

and   sTsign(s)=sTss=s2s=s		    (46)

Then

−2δ+ky1(t)λmin(P2)s		   (47)

because

δ+ky1(t)λmin(P2)s2≤sTP2δ+ky1(t)sδ+ky1(t)λmax(P2)s2        	    (48)

Therefore (43) can be evaluated as:

V≤−e1TQ1e1+2e1TP1A12+A21TP2s−sTQ2s−2δ+ky1(t)λmin(P2)s+2ρλmax(P2B2)s+2μλmax(F−TF−1)λmax(P2B2)sy1(t)          (49)

where P2B2 is a positive definite matrix.

Hence

V≤−e1Q1−(P1A12+A21TP2)−(P1A12+A21TP2)TQ2e1

(50)

In view of (50), if the sufficient conditions (38) - (41) are satisfied , then (50) reduces to

V.≤−e1sTQe1s≤0			   (51)

for all e1(t)≠0,s(t)≠0.

Therefore, we conclude that the reduced-order observer error system (36) is globally asymptotically stable and in general on the sliding surface s(t)=0 (37) always is generated a stable sliding mode. The theorem is proved.

4 Reduced analysis of reaching and sliding modes of motion

In section 2.3 we have pointed that in particular the design parameters C1 and C2 can be selected such that A21=0. Then observer error system can be separated into two decoupled reaching and sliding modes. First, let us consider the sliding conditions for the separated observer error system.

Corollary 1: Suppose that A21=0 in (36) and conditions (38)-(41) of Theorem 1. Then sliding surface s(t)=0 (37) is reached in finite time and on the sliding surface always is generated an asymptotically stable sliding mode.

Proof : Choose Lyapunov V-function candidate as

V1(s)=sTP2s		   (52)

where P2 is a positive definite matrix.

Then the time derivative of (52) along the trajectory of the second separated equation of observer error system (36) can be calculated as:

(53)

In view of (53), if conditions (39), (40c) and (41) are satisfied then (53) reduces to:

(54)

Therefore, we conclude that an asymptotically stable sliding mode always is generated on the sliding surface s(t)=0 (37) defined for separated observer error system. The corollary is proved.

Now let us shortly analyze the separated modes. Since in sliding mode s(t)=0 and sTs<0. However, s(t) as control input is going to the first equation of observer error system (36). Therefore, at the reaching phase 0≤t≤ts this state error equation is affected by s(t). In that reaching phase s(t) is acted as first order dynamic regulator. But, when the sliding surface is reached the effect of dynamic regulator is disappeared and then more slowly state error dynamical process so-called a sliding mode is beginning. Consequently, from the first equation of separated observer error system (36) we can obtain a sliding mode motion as follows

e1(t)=A11e1(t)				   (55)

where A11 is a stable matrix, desirable eigenvalues of which each can be assigned by pole placement method. Sliding mode is a slowly mode of motion. It should be noted that a stronger condition, guaranteeing an ideal sliding motion is the -reachibility condition [12], [26].

For our multivariable case a -reachibility condition can be rewritten as

sTs≤−ηs2				   (56)

where η=λmin(Q2)λmax(P) is a some positive constant. Then the sliding surface is rapidly reached at very small time, therefore the reaching time can be evaluated as:

ts≤s(0)2η				   (57)

Thus, there are two time-scale behavior [27] of motion:

- reaching mode of motion and 2) sliding mode of motion.

Reaching mode is a fast mode of motion and can be determined by second equation of observer error system (36).  Nominal part of reaching mode is described by equation :

s(t)=(A22−B2L)s(t)			   (58)

Desired characteristic equation of the closed-loop system (58) is given by

φ(s)=|sIp−A22+B2L|

=sp+αp−1sp−1+...+α1s+α0=0	  	     (59)

where A22 is a stable matrix which can be selected by pole placement method for example by Ackerman’s formula [28]. Pole placement procedure can be adopted to our problem as follows.

(60)

where φ(A22)=A22p+αp−1A22+...+α1A22+α0Ip  (61)

5 Numerical example: Consider a simple numerical example illustrating the design procedure. The second order system is given by

A0=−96−1310 , B0=1, C0=[−0.25 0.25],

Suppose that μ=0.2 , ρ=0.2

λI−A0=λ2−λ−12=0 then λ1=4, λ2=−3. A0 is unstable matrix.

Design procedure can be fulfilled by the following steps:

- Calculate

A0B0=−96-13101

B0 A0B0=19317 determinant of which is different from zero, therefore, system is controllable.

- Suppose

T1=3−1-21 then T1−1=1123, T1T1−1=1001

- Calculate the first transformation

T1A0T1−1=3−1-21−96-13101123 =−1485−211

T1B0=3−1-211

From (10), (11):

FC0T1−1=1×−0.25 0.25 1123=0.25 0.5=C1 C2

Let P2=0.5 then C2=B21TP2=1×0.5=0.5.

- Calculate the second transformation

T2=100.250.5 , T2−1=10-0.52 , T2B1=100.250.50T2A1T2−1=100.250.52101−110-0.52 =2101210-0.52=−32004=A2

A112=−3 is stable.

- Calculate the third transformation

B22TP2=0.5×0.5=0.25

Then

T3=1004 , T3−1=1000.25

T3A2T3−1 =1004−320041000.25=−3504=A

eig(A)= -3, 4

T3B2=10040

- Calculate the sliding and stability conditions

Let Q1=1>0then from (38) using MATLAB command P1= Lyap(-3,1) =0.1667.

Using pole placement MATLAB command

L = Place(4,2,-2) = 3

Then from (36)

A22=4−2×3=4−6=−2<0.

- From (39) Q2=1>0.

- P1A12=0.1667×5=0.8335, A21TP2=0×0.5=0

P2B2=0.5×2=1; μ=0.1

Then from  (40) Q=1−0.8335−0.83351>0 because determinant of which is  0.3053.

- Let ρ=0.2 then from (41) gain parameter k=0,4.

Thus we have determined all the design parameters

of the second order observer.

6 Conclusion

In this paper, a triple state and output variable transformations based method to design a new reduced-order sliding mode observer for perturbed MIMO systems is developed. The state and output variables of original system is triple transformed in to suitable canonical form coordinates where the dynamical reduced order observer can be successfully designed. Existing reduced-order observer design techniques and state-output variables transformations are summarized in this study and presented systematically. A new combined observer configuration is proposed. Some new adequate evolution of matrix inequalities is adopted. Global sufficient asymptotical stability and sliding conditions for the coupled observer error system are established by using Lyapunov full quadratic form and formulated in terms of Lyapunov matrix equations and matrix inequalities. Reduced analysis of separated reaching and sliding modes of motion of decoupled observer error system is discussed also. A numerical example is given to illustrate the usefulness of proposed design method.

References:

[1] 	Luenberger, D.G. An introduction to observers.    IEEE  Trans. Autom. Control, 1971, 16(6), 596-602.

[2] 	Utkin V. I. 'Identification principles using sliding modes'.Dokl. AN SSSR, 1981, 257(3), 558-561,(in  Russian).

[3]	Hashimoto H.,UtkinV.I.,Xu J.X.,Susuki H. and Harashima F., VSS obsserver for linear time-varying     system. Proceedings of IECON,'90, 1990, 34-39.

[4]	Walcott B. L. and Zak S. H.,Combined observer     controller synthesis for uncertain dynamical systems with applications, IEEE Transactions on Systems ,Man and Cybernetics, 1988, 18(1),.88-104.

[5] 	Edwards C. and Spurgeon S. K., On the development of discontinious observers, International Journal of Control, 1994, 59(5), pp. 1211- 1229.

[6]	Sira-Ramizer H.,Spurgeon S. K. and Zinober A.S. I., Robust observer–controller design for linear systems, in Variable Structure and Lyapunov Control, London: Springer-Verlag, 1994, 161-180.

[7]	Young, K.D., Utkin, V.I. and Özgüner, Ü., A control engineers guide to sliding mode control. IEEE Transactions on Control Systems Technology, 7(3), 1999, 328-342.

[8]		Watanabe K., Fukuda T. and Tzateftas S.G., Sliding     mode control and a variable structure system observer as a dual problem for systems with non-linear uncertainities, Int. J. Systems Science , 23(11),1992, 1991-2001.

[9] 	Slotine, J.J.E., Hedrick, J.K. and Misawa, E.A., On sliding observers for non-linear systems, Transactions of the ASME Journal of Dynamic Systems, Measurement and Control, 122(3), 1987, 245-252.

[10]	Mielczarski, W., Very fast linear and non-linear      observers, Part 1. Theory, International Journal of      Control, 48(5), 1988, 1819-1831.

[11]	Jafarov, E.M., Design of new robust sliding mode observer for uncertain MIMO and SISO systems with and without time-delay. Proceedings of the 7th IEEE International Workshop on Variable Structure Systems “Advance in Variable Structure Systems”, Sarajevo, 17-19 July, 2002, 572-577.

[12]	Edwards Ch. and Spurgeon, S. K, Sliding Mode Control, 1998, (Taylor and Francis Ltd., London).

[13]	Jafarov, E.M. and Tasaltin, R., Robust sliding-mode control for uncertain MIMO aircraft model F-18, IEEE Transactions on Aerospace and Electronic Systems, vol. 36, no. 4, pp. 1127-1141, 2000.

[14]	Jafarov, E.M. and Tasaltin, R., Design of autopilot-output integral sliding mode controllers for guided missile systems with parameter perturbations, Aircraft Engineering and Aerospace Technology: An International Journal, vol. 73, no. 1, pp. 16-25, 2001.

[15]	Choi, H.H., An analysis and design method for uncertain variable structure systems with bounded controllers, IEEE Transactions on Automatic Control, vol.49, no. 4, pp. 602-607, 2004.

[16] 	Yeh, F.K., Chien H.H. and Fu, L.C., Design of optimal midcourse guidance sliding mode control for missiles with TVC, IEEE Transactions on Aerospace and Electronic Systems, vol. 39, no. 3, pp. 824-837, 2003.

[17] 	Singh, S.N., Steinberg, M.L. and Page, A.B., Nonlinear adaptive and sliding mode flight path control of F/A-18 model, IEEE Transactions on Aerospace and Electronic Systems, vol. 39, no. 4 pp. 1250-1262, 2003.

[18]	Sabanovich, A., Fridman L.M., and Spurgeon, S. (Editors), Variable Structure Systems: from Principles to Implementation, The IEE, London, 2004.

[19]	Transactions of the ASME, Journal of Dynamic Systems, Measurement, and Control, Special Issue on Variable Structure Systems, vol. 122, 2000.

[20]	International Journal of Control, Special Issue, Dedicated to Vadim Utkin, vol. 76, no. 9/10, 2003.

[21] 	Kwakernaak, H. and Sivan, R., Linear Optimal Control      Systems, 1972(John Wiley, New York).

[22]	Burghes, D.N. and Graham, H., Introduction to     Control Theory: Including Optimal Control, 1980 (John Wiley & Sons, New York).

[23] 	Friedland, B., Control System Design, 1987,  (McGraw-Hill Book Company, New York).

[24] 	Utkin, V.I., Sliding Modes in Control Optimization, 1992, (Springer-Verlag, Berlin).

[25]	Hung, J.Y., Gao, W.B. and Hung J.C., Variable structure control, A survey IEEE Transactions on Industrıal Electronics, 40(1), 1993, 2-22.

[26] 	Slotine, J.J.E. and Li, W., Applied Nonlinear Control,     1997, (Prentice Hall, Englewood Cliffs).

[27] 	Khalil, H.K., Nonlinear Systems, 2002 (Prentice Hall, New Jersey).

[28] 	Ackerman, J.E., Der entwulf linearer regelungs systems in zustandsraum, Regelungstechnik und Prozessdatenvererb., 7, 1972, 297-300.