Generalized componentwise stability of non-stationary continuous-time systems with constrained control

H. Mejhed + , A. Hmamed* and A. Benzaouia

+ Dépt GII, ENSA , BP 33/S, Agadir, Morocco

Abstract:- A time varying control law is proposed for non-stationary linear continuous-time varying systems with non-symmetrical constrained control. Necessary and sufficient conditions for generalized componentwise asymptotic (exponential) stability are given by using the concept of mode-vectors. The asymptotic stability of the origin is also guaranteed. The case of symmetrical constrained control is obtained as a particular case.

Keyword: - Constrained control, Positively invariant sets, Time varying regulator, Lyapunov function.

NOTATIONS: If x is a vector of ℜn then:

xi+=sup(xi,0)and  xi−=sup(−xi,0), i=1,...,n.

We will further note the following: for two vectors x, y of ℜn:

x≤y (respectively, x<y) if xi≤yi (respectively, xi<yi) i=1,...,n.

In is the identity matrix of ℜnxn; σ(A) denotes the spectrum of matrix A; Re(λ) the real part of the eigenvalue λ and λi(A) the ith eigenvalue of A. μ(A) the measure of A , Int(ℜ+m) is the interior of ℜ+m, whereas ∂D denotes the boundary of D. KerF is the null spade of matrix F.

1. INTRODUCTION

This paper is devoted to the study of linear continuous-time varying systems described by (1):

x(t)=A(t)x(t)+B(t)u(t)x(t0)=x0 , x∈ℜn	                 (1)

where x(t)∈ℜn is the state vector and u is the  constrained control, that is:

u∈Ω⊂ℜm,      m≤n	                                       (2)

Matrices A(t) and B(t) are varying and satisfy assumption (3):

A(t),B(t) controllable                                 	  (3)

Ω is the set of admissible controls defined by (4):

Ω=u∈ℜm−q2(t)≤u(t)≤q1(t);q1(t),q2(t)∈intℜ+m	(4)

with

limt→+∞q1(t)=0,     limt→+∞q2(t)=0                 	(5)

This is a variant nonsymmetrical polyhedral set, as is generally the case in practical situations.

This model can also be used in connection with the previous works [1-2], allowing an increase of the dynamics of the state regulator. One can take for example

ρs(t)=αse−βt                                                    	(6)

with β>0 and αs>0 for s=1,2.

Let us first consider the unconstrained case where the regulator problem for system (1) consists in realizing a variable feedback law as:

u(t)=F(t)x(t), F(t)∈ℜmxn with rank(F(t))=m      (7)

The choose of such regulator has been the subject of many works from which we cite, [8] in the decentralized control case.

Taking into account (4), system (1) becomes a non-stationary system in the following form:

x(t)=(A(t)+B(t)F(t))x(t)x(t0)=x0,    ∀t≥t0	              (8)

Generally matrix F(t) is chosen such that an increase of system dynamics is obtained and system (8) is asymptotically stable.

It is well known that a linear time invariant system is stable if and only if all eigenvalues of the system matrix have negative reels parts [9]. However, this is no longer true for linear time-varying systems. Under the assumption of the non-stationary systems, the eigenvalues method for proving the asymptotic stability is not adequate. An alternative method is the use of matrix measure [6], that is:

μ(A(t)+B(t)F(t))≤−ξ, ξ>0, ∀t≥t0	            (9)

Remark: The choice of (9) is justified by using the Lyapunov function v(x)=x(t) for system (8).

In the constrained case, we follow the approach proposed in [11] and further developed in [1] and [12] and references therein. This approach consists of giving conditions on the choice of the stabilizing regulator (7) such that model (8) remains valid. This is only possible if the state is constrained to evolve in a specified region defined by:

D(F(t),q1(t),q2(t))=

x∈ℜn−q2(t)≤F(t)x(t)≤q1(t); q1(t),q2(t)∈intℜ+m	(10)

with q1(t)and q2(t) are given by (5).

Note that this domain is unbounded where m<n. In this case, if x(t)∈D(F(t),q1(t),q2(t)) we may get x(t+λ)∈D(F(t),q1(t),q2(t)), ∀λ≥0.

The purpose of this paper is to define a special type of asymptotic (exponential) stability of (8), namely the generalized componentwise asymptotic (exponential) stability characterized by (5) and (10) ((6) and (10)). Necessary and sufficient conditions of the generalized componentwise asymptotic (exponential) stability of the system (8) are given.

2 PRELIMINARY RESULTS

In this section, we present some useful definitions for the sequel.

Consider two functions q1(t) and q2(t).

q1(t), q2(t): 0,+∞→ℜn with the properties:

q1(t) and q2(t) are differentiable

q1(t)>0; q2(t)>0 for t≥t0

limt→+∞q1(t)=0, limt→+∞q2(t)=0		             (11)

Definition 2.1: The system (8) is called generalized componentwise asymptotically stable with respect to q(t)=q1T(t)  q2T(t)T (GCWASq) if for every t0≥0 and for every −q2(t0)≤z(t0)≤q1(t0), the response of (8) satisfies,

−q2(t)≤F(t)x(t)≤q1(t)  for each t≥t0	(12)

Definition 2.2: The system (8) is called generalised componentwise exponential asymptotically (GCWEAS) if there exist α1>0 and α2>0 such that, for every t0≥0 and for −α2e−βt0≤F(t0)x(t0)≤α1e−βt0, the response of (8) satisfies:

−α2e−βt≤F(t)x(t)≤α1e−βt   for every   t≥t0	(13)

Remark: When F(t)=I, we obtain the definitions concerning CWEASq and CWEAS respectively given in [2].

As is well known, the eigenvalues of a linear time-invariant system is invariant under an algebraic transformation. However, this is no longer the case for a linear time-varying system. In order to preserve the invariance of the eigenvalues of a linear time-varying system. We use the concept of eigenvalues and eigenvectors introduced in [4]. For simplicity of discussions, they will be called the extended eigenvalue (or simply x-eigenvalues) and the extended-eigenvector (or simply x-eigenvector). When they are discussed together, they will be simply called the extended eigen-pair (or simply x-eigenpair).

Definition 2.3 [4]: A differentiable non-zeros vector e(t) is said to be the extended-eigenvector (x-eigenpair) of the nxn matrix G(t), associated with the extended-eigenvalues λ(t) (a scalar time function) if it satisfies,

G(t)e(t)=λ(t)e(t)+e(t)			           (14)

Definition 2.4 [4]: The nonzero differentiable vector mi(t)=expλi(τ)dτui(t) is said to be the mode-vector of G(t) associated with the x-eigenpair λi(t),ui(t).

Remarks:1) From (9) the linear time-varying system (11) is asymptotically stable and then from [4], every mode vector mi(t) of A(t)+B(t)F(t) is bounded and mi(t)→0 as t→∞, ∀i=1,…,n.

2) The algorithm for finding the x-eigenpairs of A(t)+B(t)F(t) is given in [4].

3) When G(t) is constant, the eigenpair λi(t),ui(t) of G are constant (and so are the x-eigenpairs of G). So the mode vectors of G become

mi(t)=expλi(t−t0)ui(t)

which is completely characterized by the eigenvalues λi of G. Clearly mi(t)→0 as t→∞, if and only if Re(λi)<0. This is the well-known stability criteria for linear time-invariant systems.

Consider the following non-stationary continuous time linear system,

z(t)=H(t)z(t)z(t0)=z0, z∈ℑ⊆ℜm  and   0∈Intℑ	(15)

The following results will be required.

Theorem 2.4: A necessary and sufficient condition for the system (15) to be CWASq is,

q(t)≥H(t)q(t), ∀t≥t0			 (16)

with,

H(t)=H1(t)H2(t)H2(t)H1(t);   q=q1(t)q2(t)     	(17)

H1(t)=hii(t)     i=jhij+(t)     i≠j,    H2(t)=0           i=jhij−(t)     i≠j,   ∀t≥t0					(18)

Proof: (If) We change only matrix eH(t−t0) by matrix eH(τ)dτ in the proof given in [2], the proof remains unchanged.

Remark: When H(t)=H, we obtain the result given in [2].

In the symmetrical case q1(t)=q2(t)=q(t), we can deduce the following result.

Corollary 2.5: A necessary and sufficient condition for the system (15) to be CWASq is

q(t)≥H(t)q(t) for every t≥t0 with

H(t)=hii(t)           i=jhij(t)          i≠j  , ∀t≥t0

Proof: By observing that H(t)=H1(t)+H(t). The proof follows readily from Theorem 2.3.

Theorem 2.6: A necessary and sufficient condition for the system (15) to be CWEAS is:

H(t)α≤−βα, ∀t≥t0			   (19)

with α=α1Tα2TT.

Proof: By substituting ρ(t)=αe−βt into relation (16), we obtain (19), the proof is completed.

Remark: When β=0, and H(t)=H, we obtain the result given in [2].

In the symmetrical case α1=α2=α, we can deduce the following result.

Corollary 2.7: A necessary and sufficient condition for the system (15) to be CWEAS is

H(t)α≤−βα, for every t≥t0.

3 MAIN RESULTS

In this section, we apply the results of Theorem 2.4 to the problem of the constrained regulator described in Section I.

Consider system (1) with the feedback law given by (7) and (9). The system in the closed loop is then given by (8). Let us make the change of variables,

z(t)=F(t)x(t)  , F(t)∈ℜmxn		    (20)

with matrix F(t)∈ℜmxn is given by (7) and (9). It follows that

z(t)=F(t)x(t)+F(t)A(t)+B(t)F(t)x(t)	     (21)

If there exists a matrix H(t)∈ℜmxm such that:

F(t)+F(t)A(t)+B(t)F(t)=H(t)F(t)∀t≥t0	(22)

then, the change of variable (20) allows us to transform dynamical system (8) to dynamical non-stationary system (15). The study of the generalized componentwise stability for system (8) with x∈D(F(t),q1(t),q2(t)) defined by (10), becomes possible by the use of the results of last section concerning the componentwise stability for system (15)

Before giving the main result, we present all the necessary Lemmas. The first concerns (22), which is to be satisfied for every t. For this, let us define the set ℵ(F) as follows:

ℵ(F)=x(t)∈ℜn  F(t)x(t)=0,  F(t)∈ℜmxn	(23)

In the stationary case: ℵ(F)=Ker(F).

We note A0(t)=A(t)+B(t)F(t).

Lemma 3.1: If a matrix H(t)∈ℜmxm satisfying (22) exists, then n-m stables extended eigenvectors common to matrices A(t) and A0(t) belong to ℵ(F).

Proof: Let a matrix H(t) satisfying equation (22) exists. Consider an extended eigenvector e(t) of matrix A0(t) corresponding to an extended eigenvalue λ(t), [4], i.e:

A0(t)e(t)=λ(t)e(t)+e(t)			(24)

Equation (22) allows us to write

F(t)A0(t)e(t)=λ(t)(F(t)e(t))+(F(t)e(t))	             (25)

=(H(t)F(t)−F(t))e(t)

which implies,

H(t)θ(t)=λ(t)θ(t)+θ(t)

where,

F(t)e(t)=θ(t)				    (26)

Then F(t)e(t) is an extended eigenvector of matrix H(t) corresponding to the same extended eigenvalue λ(t). Matrix H(t)∈ℜmxm could admit only m extended eigenvalues from the set of extended eigenvalues of matrix A0(t). Let us note σ(A0(t))=Λ=Λ1∪Λ2, with σ(H(t))⊂Cm and Λ2⊂Cn−m.

Where σ(A0(t)) (σ(H(t))) denotes a set of extended eigenvalues of A0(t) (respectively H(t)),[4].

Then, for δ∈Λ2, we should have,

H(t)F(t)w(t)=δ(t)(F(t)w(t))+ddt(F(t)w(t))     (27)

implies,

F(t)w(t)=0, ∀t≥t0			     (28)

for w satisfying A0(t)w(t)=δw(t)+w(t).

Since A0(t)=A(t)+B(t)F(t), we should also have:

A(t)w(t)+B(t)F(t)w(t)=δ(t)w(t)+w(t)	(29)

From (28), we obtain A(t)w(t)=δ(t)w(t)+w(t), and then Λ2⊂σ(A(t)).

If δ=0, then from (27), (ddt)(F(t)w(t))=0, implies F(t)w(t)=cste. In this case, vector w(t) do not belong necessarily to ℵ(F). Further, condition (9) ensures that μA0(t)≤−ξ, ξ>0, ∀t≥t0, using the fact that Re(λi(A0(t))≤μ(A0(t)), [7], then, the set of extended eigenvalues of matrix A0(t) is stable. Consequently, Λ2 contains n-m stable and non-null extended eigenvalues corresponding to n-m common extended eigenvectors to matrices A(t) and A0(t) and belonging to ℵ(F).

We now give two lemmas on the ℵ(F) with F(t)∈ℜmxn and rank(F(t))=m.

Lemma 3.2: If x(t)∈ℵ(F) ∀t>t0, then there exists a matrix H(t)∈ℜmxm satisfying relation (22).

Proof: Let F(t)x(t)=0 ∀t>t0. It is clear that (ddt)(F(t)x(t))=0 and obviously

F(t)x(t)+F(t)x(t)=0			 (30)

It follows:

F(t)(A(t)+B(t)F(t))+F(t)x(t)=0, ∀t≥t0	(31)

In this step, we can generalize the results of [1] to the relations (31). This implies the existence of H(t)∈ℜmxm such that (22) is satisfied.

Lemma 3.3: If system (8) is GCWASq then, x(t)∈ℵ(F) ∀t>t0,.

Proof: Let F(t0)x(t0)=0. It is clear that x(t0)∈D(F(t),q1(t),q2(t)), From system (8), we can deduce F(t)x(t)=F(t)e(A(τ)+B(τ)F(τ))dτx(t0), ∀t≥t0. At this step, we can use the proof given in [1] as the proof remains unchanged. We can deduce that,

F(t)x(t)=0, ∀t≥t0

We are now able to give the main result of this paper.

Theorem 3.4: A necessary and sufficient condition  for the system (8) to be GCWASq is

i)  F(t)+F(t)A(t)+BF(t)=H(t)F(t), ∀t≥t0	(32)

ii)  q(t)≥H(t)q(t) , ∀t≥t0		             (33)

with matrix H(t) and vector q(t) are defined in Theorem 2.4.

Proof: (If) Consider the change of variables (20). Condition (32) implies that system (8) can be transformed to system (15), domain D(F(t),q1(t),q2(t)) to domain D(Im,q1(t),q2(t)). By the use of Theorem 2.4, condition (33) ensures that system (15) is CWASq. Then system (8) is obviously GCWASq.

(Only If) Suppose that system (8) is CWASq. By virtue of Lemma 3.3, x(t)∈ℵ(F) ∀t>t0. According to Lemma 3.2, there exists a matrix H(t) satisfying (40). The same change of variable (20) allows to transform system (8) to system (15) and domain D(F(t),q1(t),q2(t)) to domain D(Im,q1(t),q2(t)). By virtue of Theorem 2.4, it follows that condition (33) hold.

The result of this Theorem is based on the existence of a matrix H(t)∈ℜmxm satisfying (32). A necessary and sufficient condition of the existence of a matrix H(t) is giving by the following Theorem.

Theorem 3.6: There exists a matrix H(t)∈ℜmxm solution of (32), where F(t)∈ℜm and rank(F(t))=m, m≤n if and only if

rankF0F0A(t)=m , ∀t≥t0		    (34)

when condition (34) is satisfied, matrix H(t) can be easily computed [3].

Proof: We change only matrix FA by matrix F(t)A(t)+F(t) in the proof given in [3] and by observing that there always exist a mxm regular matrix Λ(t) such that F(t)=Λ(t)F(0), with Λ(t)=F(t)F0TF0F0T−1,

F(t)F(t)A(t)+F(t)=Λ(t)0Λ(t)Λ(t)F0F0A(t)

The proof remains unchanged.

Comments:1) Conditions (32) and (33) guarantee that system (8) is componentwise asymptotically stable despite the existence of non-symmetrical constraints on the control. The choice of F(t)∈ℜmxn satisfying (9), (34) and (32)-(33) allows system (1)-(7) to be componentwise asymptotically stable. Further, Lemma 3.1 gives a necessary and sufficient condition on matrix A(t) for the existence of matrix H(t) solution of equation (32), i.e., matrix A(t) possesses n-m stable eigenvalues. When it is not the case, we proceed to an augmentation of the vector entries. This technique is given in [1a].

2) The computation of matrix F(t)∈ℜmxn satisfying the above relations can be achieved by two methods, the first classical one is applied here in this paper, the second one also entitled the inverse method will be detailed in feature publication [10].

Example 1: To show the effectiveness of the proposed regulator, we consider the simple first order system described by the following equation:

x(t)=tx(t)−(1+2t)e((t22)−t)u(t)

and let −2e−t22≤u(t)≤e−t22. It is clear that the set of extended eigenvalues of matrix A(t) is unstable δ(A(t))=tand m=n.

Let us impose the extended eigenvalues Λ=−2t and extended eigenvectors e(t)=e((t22)−t) and find F(t)∈ℜmxn satisfying δ(A(t)+B(t)F(t))=−1−t that is,

(A(t)+B(t)F(t))e(t)=(−1−t)e(t)

we obtain,

F(t)=e−(t22)+t

From (32), the matrix H(t) is then computed as: H(t)=−2t.

By applying the algorithm given in [4], the extended eigenvalues of matrix H(t) is -2t and the extende eigenvector is e((t22)−t) and matrix H(t) further satisfies,

H(t)q−q=−te−t22  −2te−t22T0.

In the case when condition (33) is not satisfies, then we choose another set of extended eigenvalues Λ and another set of extended eigenvectors.

Consequently, the componentwise asymptotic stability of the system despite the presence of the constraints is guaranteed.

Example2: Consider the system (1) described by the following:

A(t)=t002tB(t)=−1.1(2+t)0c(t)−1.1−2t

where

c(t)=−0.7355et11+(1.1+2t)(1.11t−0.9091et11+2.3457)

It is clear that the system matrix A(t) is unstable, δ(A(t))=t,2t and n=m.

The set of admissible control Ω is given by:

Ω=u∈ℜm−0.1e−t(1+t)11e−t≤u(t)≤e−t(1+t)2e−t

Let us impose the set of extended eigenvalues Λ=−2,−1.1 and the set of extended eigenvectors e(t)=e1(t)e2(t)T with e1(t)=1.11011et11T, e2(t)=01T and find F(t) realizing δ(A(t)+B(t)F(t))=δ(A0(t))=Λ, that is,

A0(t)e1(t)=−2e1(t)+e1(t)A0(t)e2(t)=−2e2(t)+e2(t)

We obtain,

F(t)=0.909101.0101t−0.8264et11+2.13241

Note that F(t) is invertible.

In this case

A0(t)=A(t)+B(t)F(t)=−20−0.6686et11−1.1

By applying the algorithm given in [4], the set of extended eigenvalues of A(t)+B(t)F(t) is δ(A(t)+B(t)F(t))=−2,−1.1.

From (32), the matrix H(t) is then computed as:

H(t)=−20−t−1−1.1

Note that the extended eigenvalues of matrix H(t) are −2,−1.1 and matrix H(t) further satisfies (33) that is:

H(t)q(t)−q(t)=

−t(1+t)2−0.1e−t−0.1t(1+t)2−0.1e−tT≤0

In the case when condition (33) is not satisfies, then we choose another set of extended eigenvalues Λ and another set of extended eigenvectors.

Consequently, the componentwise asymptotic stability of the system despite the presence of the constraints is guaranteed.

4 CONCLUSION

In this paper, a time varying control law of non-stationary linear continuous time varying systems with nonsymmetrical constrained control is proposed. Necessary and sufficient conditions for generalized componentwise asymptotically (exponential) stability are given. The case of symmetrical case is obtained easily by taking q1(t)=q2(t)=q(t). Finally, the presented results are shown to be a generalization of previously results related to continuous time invariant systems.

# REFERENCES

[1] Benzaouia A. and A.Hmamed, (a)" Regulator problem for linear continuous time systems with nonsymmetrical constrained control using non-symmetrical lyapunov function," In Proc.3th CDC IEEE-Arizona, 1992; (b)"Regulator problem for continuous time systems with nonsymmetrical constrained control, "IEEE Trans.Aut. Control. vol.38, no10, pp 1556-1560, October 1993.

[2] A. Hmamed and A. Benzaouia, Componentwise stability of linear systems: A non-symmetrical case. Int. Journal of Robust and non-linear control, Vol. 7, pp. 1023-1028, (1997).

[3] B.Porter, "Eigenvalue assignment in linear multivariable systems by output feedback," Int.J.contr.,vol.25, no.3, pp.483-490, 1977.

[4] Min-Yen Wu, " On stability of linear time-varying systems", CDC-IEEE, pp. 1211-1214, 1982.

[5] Scneider, H. and M. Vidyasagar, 'Cross-positive matrices', SIAM J. Num. Analysis, 7(4), 1970.

[6] Brad Lehman and Khalil Shujaee, "Delay independent stability conditions and decay estimates for time-varying functional differential equations", IEEE Trans. On automatic control, Vol. 39, No. 8, August, 1994.

[7] C.A., Desoer and M. Vidyasagar, "Feedback Systems: Input-Output Properties (New York: Academic Press), 1975.

[8] Anderson B.O.D., Moore J.B., 1981, “Time-varying feedback laws for decentralised control”, IEEE. Trans. Autom. Control, vol.26, N5, 1133.

[9] W. Hahn, Stability of Motion, Berlin: Springer-Verlag, 1967.

[10] A. Hmamed, A. Benzaouia and N.H. Mejhed,” The resolution of equation

X(t)+X(t)[A(t)+B(t)X(t)]=H(t)X(t)and the pole assignment problem”, Submitted to Automatica.

[11] P.O. Gutman and P.A. Hagander, "New design of constrained controllers for liner systems," IEEE Trans. Automat. Contr., vol. AC-30, pp. 22_23, 1985.

[12] M. Vassilaki and G. Bistoris, "Constrained regulation of linear continuous-time dynamical systems," Syst. Contr. Lett., vol. 13, pp. 247-252, 1989.