A New Model to the Synthesis of Sharp Transition FIR Filter

Abstract :- A novel technique of synthesis of a sharp transition, equiripple passband, low arithmetic complexity, linear phase lowpass FIR filter is proposed. The frequency response is modeled using trigonometric functions of frequency and its slopes are matched at the edges of the transition region. This approach has the advantages of ease of computation of the impulse response and reduction in the Gibb’s phenomenon. The synthesized filter proves to be a good alternative to filters of the same class reported in the literature.

Key-Words: -FIR digital filters, Sharp transition filters, Linear phase filters.

1. Introduction

Linear phase FIR digital filters have many advantages such as guaranteed stability, free of phase distortion and low coefficient sensitivity. A serious disadvantage of FIR filters is its complexity and this problem becomes particular acute in sharp filters which renders real time high speed implementation impractical. Several methods have been proposed in the literature for reducing the complexity of sharp FIR filters. One of the most successful techniques for synthesis of very narrow transition width filter is the Frequency Response Masking (FRM) technique because of reduced arithmetic complexity involved [1]-[2]. The major advantages of FRM approach is that the filter has a very sparse coefficient vector so its arithmetic complexity is very low, though its length and delays are slightly longer than those in the conventional implementations. These filters are suitable for VLSI implementations since hardware complexity is reduced.

In the FRM techniques, closed form expressions for impulse response coefficients were not obtained. We propose an analytical approach to the design of sharp transition filters with least arithmetic complexity. The approach is simple, analytical, without extensive computations and can be extended to design sharp cutoff highpass, bandpass, bandstop filters with arbitrary passband. Expressions for impulse response coefficients are derived, coefficients obtained and simulation of the individual filters and resultant filter obtained by cascading the two filters is carried out.

2. Filter Model

The proposed model for the pseudo-magnitude of the filter transfer function, the passband, stopband and transition band are modeled using simple trigonometric functions. The model is formulated for equiripple passband, sharp transition and arbitrary passband. Two filters are cascaded to obtain improvement in stopband attenuation of the resultant filter.

2.1 Filter Model Parameters

In the proposed model for a linear phase, equiripple passband, sharp transition, low pass FIR filter the various regions for the individual filter responses are formulated as follows. The pseudo-magnitude responses of the two filters are modeled as below.

Passband region the frequency responses are

Hpm1ω=1+δ2sin kp ω         0≤ω≤ωp1     (1)

Hpm2ω=1−δ2sin kp ω         0≤ω≤ωp2     (2)

where ω the frequency variable, Hpm1ω, Hpm2ω are the pseudo-magnitudes of the filter response,δis passband loss, kp an integer and is a filter parameter in the passband and ωp1,ωp2 are the end of ripple channel frequencies.

For a specified   ωp1,

ωp2=ωp1 +πkp                                (3)

In the transition region which also spans part of the passbands (ωp1,ωc1), (ωp2,ωc2) as well part of the stopband (ωs1,ωz), (ωs2,ωz) where ωc1,ωc2are cutoff frequencies and ωs1,ωs2 are stopband edge frequencies.

Transition region frequency responses are

Hpm1ω=A1 cos kt1ω -ω01      ωp1≤ω≤ωz   (4)

Hpm2ω=A2 cos kt2ω -ω02      ωp2≤ω≤ωz   (5)

where kt1 , kt2 are the filter parameters in the transition region and are integers, A1andA2 are amplitude parameters and are chosen greater than 1,also A1<A2 so that kt2>kt1 to make the transition of Hpm2ω narrower than that of Hpm1ω. ω01 is the frequency at which Hpm1ω equals A1, ω02 is the frequency at which Hpm2ω equals A2, ω01<ωp1, ω02< ωp2, ωzis the frequency at which both Hpm1ω andHpm2ω  are zero in the common stopband region.

Stopband region the frequency responses are

Hpm1ω=−δs2 sin ks1ω -ωz   ωz≤ω≤π    (6)

Hpm2ω=−δs2 sin ks2ω -ωz   ωz≤ω≤π    (7)

where δsis the stopband loss, ks1, ks2 are the filter parameters in the stopband region.

From (1), sin kp ωp1= 0                                     (8)

kp ωp1=L π                                                    (9)

where L is odd i.e. 1,3,5.. to give negative slope due to roll off.

kp ωp2=L+1 π                                                (10)

Magnitude response of the two individual filter models as shown in Fig.1 at various frequencies are

Hpm1ωp1=1.0=A1 cos kt1ωp1 -ω01         (11)

Hpm1ωc1=1−δ2=A1 cos kt1ωc1 -ω01            (12)

Hpm1ωs1=δs2=A1 cos kt1ωs1 -ω01                 (13)Hpm1ωz=0=A1 cos kt1ωz -ω01                    (14)

Hpm2ωp2=1.0=A2 cos kt2ωp2 -ω02             (15)

Hpm2ωc2=1−δ2=A2 cos kt2ωc2 -ω02          (16)

Hpm2ωs2=δs2=A2 cos kt2ωs2 -ω02              (17)

Hpm2ωz=0=A2 cos kt2ωz -ω02                        (18)

2.2 Slope Equalization:

The parameters of the model are evaluated by equalizing the slopes of the pseudo-magnitude response function at ωp1,ωp2 and ωz. This allows the proposed function to be continuous thus reducing the effects due to Gibb’s phenomenon.

Slope at ωp1=    ddωHpm1ωω=ωp1                   =− kpδ2                                      (19)

Also,

Slope at ωp1=− A1 kt1sin kt1ωp1 -ω01                    (20)

=−kt1A12−1                        (21)Slope at ωp2=    ddωHpmωω=ωp2                   =− kpδ2                                      (22)

Also,

Slope at ωp2=−A2 kt2sin kt2ωp2 -ω01                   (23)

=−kt2A22−1                                (24)

Equalization of slopes at ωp1 and ωp2 yields,

kt1= kp δ2A12−1                                      (25)

kt2= kp δ2A22−1                                      (26)

From (14) and (18),

ωz− ω01=  π2kt14 kz+1                        (27)

ωz− ω02=  π2kt24 kz+1                        (28)

where kz=0,1,2….due to negative slope at roll off. kz=0 gives the narrowest transition region width for the filter.

In the stop band,

Slope at ωz=  ddωHpm1ωω=ωz                                    − ks1 δs2                                    (29)

Slope at ωz=  ddωHpm2ωω=ωz                                    − ks2 δs2                                    (30)

Also, slope at z

=−A1kt1sin kt1ωz -ω01          (31)

=−A2kt2sin kt2ωz -ω02         (32)

From (27) and (28),

sin kt1ωz− ω01=1                                    (33)

sin kt2ωz− ω02=1                                   (34)

Equalization of slopes at ωz yields,

ks1=2A1kt1δs                                     (35)

ks2=2A2kt2δs                                    (36)

The magnitude response Hpm1ω and Hpm2ω are as shown in fig. 1.

(a)

(b)

(c)

Fig.1(a) Magnitude responses Hpm1ω andHpm2ω of proposed model for lowpass filters, (b) Magnified view of the passband of Hpm1ω, (c) Magnified view of the stopband of Hpm1ω

2.3 Impulse Response Coefficients

Let h(n), 0 ≤ n ≤ N-1, be the impulse response of an N-Point linear phase FIR digital filter. The linear phase condition implies that the impulse response satisfies the symmetry condition [3],

h(n) = h (N-1-n), n = 0,1,2…N-1.              (37)

The frequency response for a linear-phase FIR filters for odd N is given by

Hejω=e-j N-12 ω Hpmω                          (38)

where the pseudo magnitude response Hpm(ω) is

Hpmω= hN-12

+2n=1N-12hN−12−n cosn ω                 (39)

The impulse response sequence determined by this frequency response is obtained from

hdn=12πHpmω dω                          (40)

The impulse response coefficients h(n) for the resultant filter are obtained by evaluating the integrals below.

hN−12=1π[0ωHpm1ω Hpm2ω dω +                          ωωHpm1ω Hpm2ω  dω+ωωHpm1ω Hpm2ω  dω       +ωzπHpm1ω Hpm2ω  dω]                              (41)

hN-12−k=1π[0ωHpm1ω Hpm2ω cos kω dω +

ωωHpm1ω Hpm2ω cos kω dω+ωωHpm1ω Hpm2ω

cos kω dω+ωzπHpm1ω Hpm2ω cos kω  dω]        (42)

where k=1,2,…(N-1)/2.

(a)

(b)

Fig. 2 Magnitude response of the proposed lowpass filter (a) Linear plot (b) dB plot

3. Results

A pair of lowpass linear phase FIR filters with passband edges specified at 0.558π and 0.559π respectively, stopband edges at 0.572π and 0.603π respectively, maximum passband ripple each of ±0.4dB and minimum stopband attenuation each of 25dB were designed using our approach. The filters are cascaded to obtain the resultant filter.

The design and numerical computation was done using MATLAB. The integrals in equations (41) and (42) are evaluated to obtain the coefficients of the resultant filter. Stopband attenuation of the resultant filter improves however there is imperceptible change in transition region width. Passband ripple of the resultant filter improves slightly from those of the component filters. The specification of the filter obtained after cascading the two filters were maximum passband ripple of ±0.3dB, minimum stopband attenuation of 41.5dB and transition bandwidth of  0.012π was obtained. The filter order required for realization was found to be 290. The magnitude response of the resultant filter obtained is shown in fig. 2.

4. Conclusions

We have proposed a model for a sharp transition, equiripple passband, low arithmetic complexity, linear phase lowpass FIR filter. Various regions of the filter are approximated with trigonometric functions of frequency, making it convenient to evaluate the impulse response coefficients in closed form. A filter model is formulated and its transfer function is evolved in frequency and time domain. A pair of filters is cascaded to improve the stopband attenuation of the resultant filter.  This approach can be extended to sharp cutoff highpass, bandpass, and bandstop filters with arbitrary passband.

5. References

1.Yong Ching Lim, “Frequency -response masking approach for the synthesis of sharp linear phase digital filters,” IEEE Trans. Circuits Syst.,vol.CAS-33, pp. 357-364, Apr.1986.

2.Ronghuan Yang, Bede Liu and Yong Ching Lim, “A new structure of sharp transition FIR filters using frequency -response masking,” IEEE Trans.Circuits Syst.,vol.35, pp.955-965, Aug 1988.

3.Johnny R. Johnson, Introduction to Digital Signal Processing. Prentice-Hall,Inc.,Englewood Cliffs,  N.J.,U.S.A.