Blind Source Separation Using Wold Decomposition and Second Order Statistics

MASOUD REZA  AGHABOZORGI

Department of Electrical Engineering,

Yazd University

Yazd , P.O.BOX  89195-741

IRAN

http://www.yazduni.ac.ir

Abstract: - The separation of unobserved sources from mixed observed data is a fundamental signal processing problem. Most proposed techniques for solving this problem rely on independence or at least uncorrelation assumption of source signals.

This paper introduces a novel technique for cases that source signals are correlated with each other. The method uses Wold decomposition principle for extracting desired and proper information from the predictable part of the observed data, and exploits approaches based on second-order statistics to estimate the mixing matrix and source signals. Simulation results are provided to illustrate the effectiveness of the method.

Key-Words: - Blind Source Separation, Wold Decomposition,  Second-Order Statistics

1   Introduction

Blind source separation (BSS) consists of recovering source signals from several observed noisy mixtures of them. The observations are obtained from a set of sensors, each receiving a different combination of the source signals. The problem is called “blind” because no information is available about the mixture, i.e. recovering of source signals is achieved without the knowledge of the characteristics of the transmission channel.

The lack of prior information can be compensated by considering particular source statistics assumptions. The most popular condition  used by BSS techniques is the statistically strong assumption of independence between the source signals. These techniques assume that the primary sources are statistically independent, and therefore the goal in these techniques is to achieve a separation process that  produces outputs as independent as possible[1],[2]. A less stringent condition is uncorrelation of sources. These techniques exploit temporal correlation of each source signal (second-order blind identification), and use a joint diagonalization method of several correlation matrices[3],[4].

Other techniques and algorithms introduced for BSS use special source signals structures, e.g. CMA(Constant Modulus Algorithm) or special structure for combination  system such as MUSIC and ESPIRIT[5],[6].

In this paper, the aim is to propose a solution to BSS problem for correlated source signals without imposing special structures on signals or mixing matrix. This paper is organized as follows:  In section 2, the problem of  BSS is  stated  along with  the related  assumptions. Proposed pre-separation procedure is introduced in section 3. Section 4 expresses BSS algorithm, and simulation results are presented in section 5. Concluding remarks are given in section 6.

2   Problem Formulation

Assume that  d  signals  s1(t),...,sd(t) are transmitted from d sources at different locations. By considering a narrowband time-invariant channel, what we receive at m sensors (antennas) will be instantaneous linear combinations of these signals that construct observation data:

x(t)=a1.s1(t)+…+ad.sd(t)+n(t)         (1)

Thus the model is as follows:

x(t)=y(t)+n(t)=A.s(t)+ n(t)                (2)

where x(t)∈ℜm×1 is the observed data vector from  m  sensors, s(t)∈ℜd×1 is the signal vector, composed of  d  unknown source signals, A=[a1, ... , ad]∈ℜm×d characterizes the unknown channel and is referred to as  “mixing  matrix”, n(t)∈ℜm×1is the additive noise vector at the sensor array. Following assumptions are considered in the model:

A1) Each element of  s(t) (source signals) is a zero-mean, stationary process.

A2) The additive noise  n(t) is assumed as a stationary, white zero-mean random process, independent of  source signals.

A3) Mixing matrix  A has full column rank.

A4) The number of sensors (m) must be greater or equal to  d  (the number of sources).

It must be emphasized here that we don’t impose any assumption about independence or uncorrelation of source signals. In other words, the source signals can be correlated, and only the following assumption is considered:

A5)Source signals are jointly stationary.

The aim of blind source separation (BSS) is to identify the mixing matrix A (and consequently recovering the source signals from the observations).

It must be noted that Complete identification of the mixture matrix is impossible because the exchange of a fixed scalar factor between a given source signal and the corresponding column of A doesn’t affect the observations(scaling indeterminacy).

EMBED Equation.3x(t)=A.s(t)+ n(t)=k=1dakαkαksk(t)+ n(t)   (3)

where αkis an arbitrary factor , and akdenotes the k-th column A.

Another indeterminacy is in the order of the separated signals(permutation indeterminacy).

3   Pre-Separation  Procedure

The main step in our approach for correlated sources is a pre-separation process. The observed data is decomposed into regular and predictable components, using Wold decomposition. In the predictable component, the combination of uncorrelated contributions of source signals is identified on whose basis A (and consequently the source signals) is estimated using second order statistics.

3.1  Wold  Decomposition

An arbitrary process  can be written as a sum:

s(t)=sr(t)+sp(t)                                         (4)

where sr(t) and sp(t) are regular and predictable processes. This expansion is called Wold decomposition. In [7] it has been proved that the processes sp(t) & sr(t) are orthogonal. Furthermore, sp(t) is comprised of complex exponentials:

sp(t)=c0+ici.exp(jωit)                  (5)

where ci’s are orthogonal zero-mean random variables. Hence, sp(t) has a line spectrum:

Psp(ω)=i2παiδ(ω−ωi)                       (6)

but sr(t) has a smooth spectrum.

3.2  Observation Decomposition

In this subsection a method is proposed for extracting and decomposing some information from the regular and predictable parts of the observation data. For simplicity ,a special case of model (2) with d=2 & m=2 is considered, that can be extended to general cases. So, we have the following model satisfying conditions expressed in section 2:

x(t)=x1(t)x2(t)=A.s1(t)s2(t)+n1(t)n2(t)                 (7)

where s1(t) & s2(t) are the source signals and A=α1β1α2β2 is the mixing matrix.

Regular and predictable parts of source signal are indicated by  sir(t) and sip(t) (i=1,2):

si(t)=sip(t)+sir(t)                                (8)

where,

s1p(t)=kak.exp(jω1kt)                        (9)

s2p(t)=lbl.exp(jω2lt)                        (10)

in which {ak} & {bl} are sets of orthogonal random variables, and {ω1} &{ω2} are proper frequency sets. Also, since source signals are assumed jointly stationary, ak & bl corresponding to ω1k≠ω2lare orthogonal.

Using (7) , (8) and the fact that regular and predictable parts in each signal are orthogonal ,we get:

xi(t)=xip(t)+xir(t)+ni(t)     ;   i=1,2          (11)

where

xip(t)=αis1p(t)+βis2p(t)        ;   i=1,2             (12)

From (9),(10),(12) obtains:

xip(t)=qdiq.exp(jωqt)    ;   i=1,2               (13)

where {diq} are orthogonal random variables and ωq=ω1∪ω2.

From these equations, each observation signal has regular and predictable components, each corresponding to the combination of individual regular and predictable parts of source signals. A spectral method for separating these parts follows.

The correlation functions of the observation data  are given by: (i,j = 1,2)

rijx(τ)=rijrx(τ)+rijpx(τ)+N0.δ(τ)              (14)

where N0 is the noise variance and rijrx(τ) & rijpx(τ) are correlation functions of regular and predictable parts:

rijrx(τ)=Exir(t+τ)xjr(t)                            (15)

rijpx(τ)=Exip(t+τ)xjp(t)          =qEdiqdiq.exp(jωqt)                            (16)

Hence power spectral density(psd) and cross spectral density(csd) functions of observations have the forms:

Pijx(ω)=Pijrx(ω)+Pijpx(ω)+N0          (17)

where,

Pijpx(ω)=q2π.Ediqdjq.δ(ω−ωq)   (18)

As expected, the spectra of the predictable parts are pure impulsive. So, it is possible to detect and separate these components in the observation spectra. Consequently, correlation functions (rijpx(τ)) of the predictable parts are obtained that will be used next.

3.3  Extracting Desired Information From

Predictable Part

Rewriting predictable parts of source signals (9)-(10), considering {Ωn} as the common frequency set, we obtain:

s1p(t)=k≠ n ak.exp(jω1kt)+ n an.exp(jΩnt)                                                             (19)

s2p(t)=l≠ n bl.exp(jω2lt)+ n bn.exp(jΩnt)                                                              (20)

where:

1)Random variables ak & bl corresponding to ω1k≠ω2l are orthogonal.

2)Correlation of predictable signals  s1p(t) & s2p(t) arises from correlation of random variables an & bn corresponding to {Ωn} (common frequency components of source signals).

3)Removing common frequency components of source signals from s1p(t) & s2p(t) result in two residue signals, s1p(t) & s2p(t), that are uncorrelated:

s1p(t)=k≠ n ak.exp(jω1kt)                    (21)

s2p(t)=l≠ n bl.exp(jω2lt)                           (22)

Hence,(for  i=1,2 )

xip(t)=[αis1p(t)+βis2p(t)]+   n (αian+βibn).exp(jΩnt)          =xip(t)+   n (αian+βibn).exp(jΩnt)                         (23)

and relation (18) can be rewritten as:

Pijpx(ω)=q≠n2π.Ediqdjq.δ(ω−ωq)

+n2π.Edindjn.δ(ω−Ωn)    (24)

Removing the terms corresponding to common frequency components detected from frequencies that their peaks are greater than the average of the common peaks in the self and cross spectral density functions of the observations:

Pijpx(ω)=q≠n2π.Ediqdjq.δ(ω−ωq)    (25)

from which the desired correlation functions are obtained: (for  i = 1,2 )

rijpx(τ)=F−1Pijpx(ω)=E

E(αis1p(t)+βis2p(t)).(αjs1p(t+τ)+βjs2p(t+τ))

At last, because  s1p(t) & s2p(t), are uncorrelated, we get the following matrix form:

Rpx(τ)=Δr11px(τ)r12px(τ)r21px(τ)r22px(τ)

=α1β1α2β2.r11ps(τ)00r22ps(τ).α1α2β1β2

=A.Rps(τ).AH                                  (27)

whereH denotes complex conjugate transpose, and

riips(τ)= Esip(t+τ)sip(t)   ;  i=1,2          (28)

It is seen that in (27) the matrix which is related to source signals is diagonal (a desired condition). This representation is the basis of an algorithm for estimating mixing matrix A.

- Blind Source Separation Algorithm

In this section, an algorithm for estimating A(and recovering source signals) is proposed which is based on the model embedded in eq.(23) and restated in (29) using second order statistics.

xip(t)=αis1p(t)+βis2p(t)        ;  i=1,2                           (29)

Steps of the algorithm are following:

  - Orthogonalization

Although s1p(t) & s2p(t) are uncorrelated and based on the discussion in section 2, we can assume that:

Rps(0)=I                                                (30)

according to eq. (29), x1p(t)  &   {x2p(t) are correlated. Hence, we apply an orthogonalization transformation on x1p(t)  &   {x2p(t). The orthogonalizer matrix is obtained from eigendecomposition of matrix Rpx(τ) at τ=0. If eigenvalues of Rpx(0) are denoted by λ1& λ2 and v1∧v2 are the corresponding eigenvectors, the orthogonalization matrix T, defined by:

T=[1λ1v1,  1λ2v2]H                              (31)

satisfies:

T.Rpx(0).TH=I                                         (32)

Also from (27),(30),(32), it is seen that :

T.A.Rps(0).AH.TH=T.A.AH.TH=I        (33)

This equation shows that matrix U=T.A, is a unitary matrix. As a consequence mixing matrix A can be factored as:

A=T−1.U                                                (34)

4.2  Estimation of  U

By   applying   orthogonalization   matrix  T  to

equation (27) for some τ≠0, Rpx(τ)=ΔT.A.Rps(τ).AH.TH                       (35)

Hence,

Rpx(τ)=U.Rps(τ).UH                                 (36)

where matrix Rpx(τ)is called orthogonal correlation matrix.

Since U is unitary and Rps(τ) is diagonal, equation (36) states that orthogonal correlation matrix Rpx(τ) is diagonalized by the unitary transformation U(unitary diagonalization). In other words unitary matrix Ucan be specified by unitary diagonalizing of orthogonal correlation matrix Rpx(τ) for some lag τ≠0 .

The essential aim is finding a unique unitary matrix Uthat diagonalizes Rpx(τ) for all time lags τ. A method for attaining this aim, as used in most BSS approaches that exploit statistical properties, is joint diagonalization(JD) method which operates as simultaneous diagonalization of the set Rpx(τi)|i=1,2,..., Kof K orthogonal correlation matrices and is described in the following theorem[3]: (In our case n=2)

THEOREM. Let  τ1,τ2,...,τK be  K  nonzero time lags, and let V be a unitary matrix such that :

∀1≤k≤K    VH.Rpx(τ).V=diag.[d1(k), ... , dn(k)]    (37)

∀1≤i≠j≤n    ∃ k,  ∋   di(k)≠dj(k)                          (38)

then:

- V is essentially equal to U(desired     unique unitary matrix)

- A permutation can be operated on diagonal elements of diag.[d1(k), ... , dn(k)]

which states that a unique unitary matrix Ucan be determined if for at least a τ≠0,eigenvalues of  Rpx(τ) are distinct, a condition that is surely satisfied for sources with different spectra.

4.3  Computing  A  and  s(t)

After determination of a unique unitary matrix U, A can be computed from A=T−1.U, and consequently the source signals are estimated as

s(t)=A−1.x(t)                                                                                  (39)

It is important to  note that for computing s(t),  we use observation data x(t) (not x(t)), so there isn’t any information loss.

## 5  Simulation   Results

In this section, the performance of the proposed method is investigated via computer simulation results.

Source signals (si(t)  ;  i=1,2) are composed of regular and predictable parts. Regular component in each source consists of a zero-mean normal process (independent from the other) and a uniform process (common between two sources). Predictable parts consist of random amplitudes sinusoidal functions with some common frequency and correlated amplitude components. These signals are mixed by an arbitrary 2×2 mixing matrix A, and corrupted by AWGN, to obtain the observation signals (xi(t)  ;  i=1,2). Then algorithm is applied on observed data and the estimation of A, A , is obtained.

This procedure is repeated for G=500 independent trials.

For evaluate the approach, the following performance index (PI) is introduced,

PI= 10.log10[1Gg=1GA−1.A−IF2]          (40)

where.F is the Frobenius norm.

Two experiments we performed and compared: without pre-separation process (experiment #1) and with pre-separation process (experiment #2). The experiments were executed under noise free and  SNR=3,5,8,10 (dB) conditions for various number of correlation matrices used in JD algorithm, and with different correlation coefficient of original source signals. Some results are illustrated in  Figures 1-6.

Figures 1 & 2 show performance index(PI)(in dB) versus the number of jointly diagonalized correlation matrices for experiments #1 and #2. In figures 3 & 4, corresponding to experiment #1 and #2, the performance indexes (in dB) versus several SNR (in dB) have been plotted for some constant number of jointly diagonalized correlation matrices with correlation coefficient  0.5. In figures 5 & 6, SNR and the number of jointly diagonalized correlation matrices have been kept constant, and correlation coefficient of source signals has been varied from 0.1 to 0.9.

Almost  in all figures, better performance of proposed algorithm is evident. In figures 1 & 2, it is obvious that the performances of two experiments become better as the number of the jointly diagonalized correlation matrices is increased, but this has a limit (as it is seen from small different between PI for K=5 and K=6). Figures 3 & 4 , show improvement in performance by increasing SNR. From figures 5 & 6, it is revealed that the performance index of two experiments is close for small correlation coefficients, and as the correlation coefficient is increased, the PI of two experiments is decreased, but the performance in experiment #2 is better than that in experiment #1, particularly for intermediate correlation coefficients.

## 6  Conclusion

In this paper, an approach for solving BSS problem in cases where source signals are correlated, is introduced without additional assumptions on signal or mixing matrix structures.

An important step of this BSS algorithm is a pre-separation procedure where based on Wold decomposition principle, the information of predictable part of source signals (i.e. uncorrelated parts of predictable signals) is derived. The diagonal structure of the correlation matrix of this parts is essential for next step of algorithm where using second-order based method and JD technique,  separation process is completed by estimating A and recovering s(t). Simulation results show effectiveness of  algorithm.

Fig.1. Performance versus number of JD covariance             matrices:[ Noise Free]&[Correlation Coef.=0.5]

Fig.2. Performance versus number of JD covariance             matrices:[SNR=3 dB]&[Correlation Coef.=0.5]

Fig. 3. Performance versus SNR  for Experiment  #1:

[K(No. of JD Covariance Mat.)=2,4,6]&[Correlation coef.=0.5]

Fig. 4. Performance versus SNR  for Experiment  #2:

[K(No. of JD Covariance Mat.)=2,4,6]&[Correlation Coef.=0.5]

Fig. 5. Performance  versus  Correlation Coefficient:

[Noise Free]&[K(No. of JD Covariance Mat.)=6]

Fig. 6. Performance  versus  Correlation Coefficient:

[SNR=3 dB]&[K(No. of JD Covariance Mat.)=6]

References:

[1] P. Comon, “Independent Component Analysis, a              New Concept?”, Signal Processing, Vol. 36, No. 3, Apr. 1994, pp. 287-314.

[2] K. Abed-Meraim and et. al., “Blind Source Separation Using Second Order Cyclostationary Statistics”, IEEE Trans. on Signal Processing, Vol. 49, No. 4, April 2001, pp. 694-701.

[3] A. Belouchrani and K. Abed-Meraim and J.-F. Cardoso and E. Moulines, “A Blind Source Separation Technique Using Second-Order Statistics”, IEEE Trans. on Signal Processing, Vol. 45, No. 2, Feb. 1997, pp. 434-444.

[4] E. Moreau, “A Generalization of Joint Diagonalization Criteria for Source Separation”, IEEE Trans. on Signal Processing, Vol. 49, No. 3, March 2001, pp. 530-541.

[5] A.-J. Van Der Veen, “Algebraic Methods for Deterministic Blind Beamforming”, Proceedings of the IEEE, Vol. 86, No. 10, Oct. 1998, pp. 1987-2008.

[6] X., Hu  and H. Kobatake, “Blind Source Separation using ICA and Beamforming”, Fourth International Symposium on Independent Component Analysis And Blind Signal Separation, Nara, Japan, April 2003, pp.597-602.

[7] A. Papoulis, Probability, Random Variables, and Statistic process, McGraw-Hill, Third Ed., 1991.