OTIS-Star an Attractive Alternative Network

AHMAD M. AWWAD

Faculty of Computing and Information Technology

Arab Open University

P.O. Box 3322, Safat 13033

KUWAIT

Abstract:-   This paper proposes a new interconnection network referred to as the OTIS-Star which is constructed from multiplying a factor network star Sn by it self. In this paper we utilize the features of OTIS networks which use both of electronic and optical networks. ِAlthough the star graph has been an important research issue for long time it suffers from many problems mainly its limited efficiency in routing and broadcasting. The OTIS-Star presents a solution for the problems which star networks suffer from. In this paper we conduct a general study on the topological properties for the OTIS-Star by obtaining the main topological properties including size, degree, diameter and number of links,  then we propose an efficient broadcasting algorithm.

Key-words:- Parallel and distributed systems; Interconnection Networks;  Star network; OTIS-Networks; Topological properties; Broadcasting.

1  Introduction

Improving computational power is a decidable dimension in future technology trends. The solution to many real life problems is now possible due to the ability of high-speed parallel computers with hundreds or thousands of processors. In fact, a significant amount of the time required for a high-speed parallel computer to solve a given problem was spent by one or more processors communicating the results of their computations to other processors of the system. These processors are physically interconnected by an interconnection network, and co-ordinate their activities to solve a common problem by exchanging information, depending on the way communication is achieved between the processors.

A major design issue involved in the construction of high-speed parallel computers is the topology of the network interconnecting its processors. The power of high-speed parallel computers is mainly attributed to the efficiency of the topology interconnecting the processors. Among all the studied networks, none can be claimed to outperform all other networks [1,10,12,13]. Many networks have emerged as attractive interconnection networks for parallel computers. The most popular of these interconnection networks are the hypercube [12], the torus [25], the star [1], and the arrangement network [13].

During the last decade a large variety of interconnection networks for parallel computers have been investigated [1, 10, 12, 13, 23]. One of the well-known interconnection networks is the star graph [1], which has been proposed as an attractive alternative to the hypercube [22]. Figure 1 shows the topology of this network. The star graph has attracted a lot of research efforts. For instance, several properties of this network have been studied including its basic topological properties [1], parallel paths characterization [14], and embedding  [15]. Akers and Krishnamurthy [1] have shown that the star graph has several advantages over the hypercube including a smaller diameter, smaller average diameter and lower degree for a fixed network size. Also the star graph has edge and vertex symmetries that maximize its fault tolerant capabilities [1,16].  Furthermore, a number of parallel algorithms on the star network for some well-known problems have been reported in the literature including computing fast Fourier transforms and  broadcasting [17].

The star graph, however, suffers from some drawbacks [18]. One major problem with the star graph is related to its limited efficiency in routing and broadcasting. Despite its attractive topological properties, the star graph has not been used in practical systems yet. One reason for this may be attributed to the difficulty in developing parallel algorithms on this network for common parallel applications. Mapping of data and tasks on the star graph is not as obvious as it is the case with the hypercube or mesh [18]. To fill this gap we have proposed the OTIS-Star as an attractive alternative for the star network by utilizing both electronic and optical technologies.

A lot of research efforts have been devoted to Optical Transpose Interconnection Systems (OTIS) networks [4, 5, 7, 8]. This kind of networks is basically constructed by “multiplying” a factor topology by itself. It is implemented using both electronic interconnect technologies and free-space optical. In the last decade there was an increasing interest in the study of parallel algorithms for optoelectronic networks in general and for OTIS-networks in particular [4, 6, 8, 9, 11]. Sahni and Wang [9] have presented and evaluated a range of algorithms on OTIS-mesh (a special case of OTIS-networks), such as , routing, selection, basic data rearrangements and sorting. Furthermore Sahni and Wang have also developed algorithms for various matrix multiplication operations [19] and image processing [20].

Extensive modelling results for the OTIS have been reported in [5 ,7]. The achievable Terra bit throughput at a reasonable cost makes the OTIS a strong competitor to the electronic alternatives [5, 8]. These encouraging findings prompt the need for further testing of the suitability of the OTIS for real-life applications in general and for OTIS-Star in specific. A number of recent studies have been conducted in this direction [3,4,6,19]. Zane et al [8] have shown that the OTIS-mesh efficiently embeds four-dimensional meshes and hypercubes.

In this paper, 	we focus on the Optical Transpose Interconnection Systems (OTIS) and its implementations by proposing OTIS-Star. OTIS-Star can be easily implemented using free-space optoelectronic technologies [6]. In this model, processors are partitioned into groups, where each group is realized on a separate chip with electronic inter-processor connects. Processors on separate chips are interconnected through free space interconnects. The philosophy behind this separation is to utilize the benefits of both the optical and the electronic technologies.

The advantage of using OTIS-Star as an optoelectronic architecture lies in its ability to deal with the fact that free space optical communication is superior in terms of speed and power consumption when the connection distance is more than few millimetres [7]. In the OTIS-Star, shorter (intra-chip) communication is realized by electronic interconnects while longer (inter-chip) communication is realized by free space interconnects.

Since the study of proposing efficient OTIS networks still not been investigated widely we contribute towards filling this gap by proposing the OTIS-Star and conducting a general study on the topological properties including size, degree, diameter, shortest distance and number of links beside proposing algorithm for broadcasting of OTIS-Star network. Figure 2 shows the topology of OTIS-Star network.

This paper considers the “multiplication” of the star graph by itself with the aim to enhance the topological characteristics of the factor star network [21]. As we shall see below, our study has derived the topological properties of the OTIS-Star and it will enables a solution for the problems, which the factor network suffers from. The OTIS-Star network also preserves all the nice qualities of the star graph topologies including, hierarchical structure, simple shortest path routing and many other properties.

The rest of the paper is organized as follows . Section 2 provides the necessary notations and definitions, and then formally presents the OTIS-Star. Section 3 discusses some of the general topological properties of the OTIS-Star network. Section 4 proposes an efficient broadcasting algorithm for the OTIS-Star. Finally section 5 concludes this study.

1234

1324

3214

2314

4231

2431

3421

4321

3412

3142

4312

1342

2413

2143

1423

4123

c

d

a

b

b

a

c

d

1243

4213

4132

1432

2341

3241

3124

2134

Fig.1: The 4-star graph

123

321

231

132

312

213

123

213

123

321

231

k

132

213

123

321

231

t

312

r

k

213

132

213

123

321

231

132

312

312

132

312

213

123

321

321

321

231

132

312

123

231

u

z

y

q

s

z

x

t

w

q

s

w

r

213

132

231

312

y

x

Fig.2: OTIS-Star network

2  Notations and definitions

Definition 1:	The n-star graph, denoted by Sn, has n nodes each labelled with a unique permutation on ánñ = {1,…,n}. Any two nodes are connected if, and only if, their corresponding permutations differ exactly in the first and one other position.

The diameter, , and the degree, , of the star graph are as follows [3]:

, of n-star graph = ë1.5 (n-1)û

, of the n-star graph = n-1, where n1.

An OTIS based computer contains N2 processors partitioned into N groups with N processors each. A processor is indexed by a pair x, y, 0x, yN where x is the group index and y is the processor index. Processors within the same group are connected by a certain interconnecting topology; while inter-group links are achieved by transposing group and processor indexes. The terms OTIS-computer and OTIS-networks refer to parallel architectures based on the OTIS and will be used interchangeably in the rest of the paper.

The OTIS-network is constructed by "multiplying" a known topology by itself. The vertex set is equal to the Cartesian product on the vertex set in the factor network. The edge set consists of edges from the factor network and new edges called the transpose edges. The formal definition of the OTIS-network is given below.

Definition 2:	Let G0 = (V0, E0) be an undirected graph representing a factor network. The OTIS-G0 = (V, E) network is represented by an undirected graph obtained from G0 as follows V = {x, y | x, y  V0 such that xy} and E = {(x, y, x, z) | if (y, z)E0}  {(x, y, y, x) | x, y  V0 such that xy }.

From the above definition the set of edges E consists of two subsets, one is from G0, called G0-type edges, and the other subset contains the transpose edges. The OTIS approach suggests implementing G0-type edges by electronic links since they involve intra-chip short links and implementing transpose edges by free space optics. Throughout this paper, the terms “electronic move” and the “OTIS move” (or “optical move”) will be used to refer to data transmission based on electronic and optical technologies, respectively.

Definition 3:    Let Sn = (V0, E0) be an undirected star graph representing a factor network. The OTIS-Sn = (V, E) network is represented by an undirected graph obtained from Sn as follows V = {x, y | x, y  V0 such that xy} and E = {(x, y, x, z) | if (y, z)E0}  {(x, y, y, x) | x, y  V0 such that xy }.

3  General Topological properties

This section discusses some of the basic topological properties of the OTIS-Star network including size, degree, shortest distance between 2 nodes, diameter, and number of links. The topological properties of the OTIS-Star network along with those of the star network are discussed and derived below. These topological properties have been derived using the theoretical framework for analyzing topological properties of the OTIS-Networks in general which was proposed by Al-Ayyoub and Day [2], beside other related research work  [9].

In this paper we will refer to g as the group address and p as the processor address. An intergroup edge of the form (g, p, p, g) represents an optical link and will be referred to as OTIS or optical move.  Note that also we will be using the following notations through our paper:

- |Sn| = size of the graph Sn .

- |OTIS- Sn | = size of the graph OTIS-Sn.

- Deg. Sn( p) = Degree of the graph Sn at node p.

- Deg. OTIS-Sn (g,p) = Degree of the graph OTIS- Sn at node address <g, p>.

- Dist_Sn(p1, p2) = The length of a shortest path between the two nodes p1 and p2 in Star graph as indicated below.

- Dist. OTIS-Sn (p1, p2)  =  The length of a shortest path between the two nodes <g1, p1> and <g2, p2>  in OTIS-Star.

In the OTIS-Star the notation g, p is used to refer to the group and processor addresses, respectively. Two nodes g1, p1 and g2, p2 are connected if, and only if, g1 = g2 and (p1, p2)E0 (such that E0 is the set of edges in star network) or g1 = p2 and p1 = g2, in this case the two nodes are connected by transpose edge.

The distance in the OTIS-Star is defined as the shortest path between any two processors, g1, p1 and g2, p2, and involves one of the following forms [24]:

(1) When g1 = g2 then the path involves only electronic moves from source node to destination node.

(2) When g1  g2 and if the number of optical moves is an even number of moves and more than two, then the paths can be compressed into a shorter path of the form: g1, p1 E g1, p2 O p2, g1 E p2, g2 O g2, p2 here the symbols O and E stand for optical and electronic moves respectively.

(3) When g1  g2, and the path involves an odd number of OTIS moves. In this case the paths can be compressed into a shorter path of the form:

g1, p1 E g1, g2 O g2, g1 E g2, p2.

The following are the basic topological properties for the OTIS-Star. For Instance if the factor star network is of size n!, degree is n-1 and diameter is 1.5 (n-1). Then the size, the degree, the diameter, number of links, and the shortest distance of  OTIS-Star network are as follows:

- Size of |OTIS-Sn| =  |n!|2.

- Degree of OTIS-Sn =  Deg.( Sn), if g = p.

- Deg. ( Sn) + 1, if g ≠ p.

- Diameter of OTIS-Sn = 21.5 (n-1) +1.

Number of Links: Let N0 be the number of links in the Sn and let M be the number of nodes in the OTIS-Sn. The number of links in the OTIS-Sn = (M2−M)2+N02. For instance, the number of links in the OTIS- S3 consisting of 6 processors is = 40.

- Dist. of  OTIS-Sn =

min( d(p1, g2)+ d(g1, p2)+1, d(p1, p2)+ d(g1, g2)+2)   if g1≠ g2Dist.(p1,p2) if g1= g2

4  Broadcasting in OTIS-Star Network

One-to-all broadcasting is a communication process in which a single source node sends the same message to every other node in the network.  It is used in many applications.  Examples are Guass elimination method,  and matrix-vector multiplication [26].  An optimal broadcasting algorithm for single communication port star networks is presented in [27]. This algorithm takes (n−1)(lg(n−1)+1) time steps in Sn.  For multi-communication ports star networks a broadcasting algorithm that takes (2n−3) time step is introduced in [28].

Broadcasting in OTIS-Sn is based on broadcasting in Sn networks as follows:

Step1: <gi, sj> broadcasts M locally to every node in the group <gi>, where <gi, sj> is  the source node and M is the message to be broadcasted.

Step 2: Every node <gi,sk>, 1≤k≤n!,k≠i, sends M to <sk,gi> over the optical link.

Step 3: Every node <sk,gi>, 1≤k≤n!,k≠i, broadcasts M locally in the <sk> group.

Using the algorithms in [27] and [28] for local broadcasting in the group star networks, the above algorithm takes 2(n−1)(lg(n−1)+1)+1 steps, or (4n−5) steps in an OTIS-Sn network depending whether it is single or multi-communication port, respectively.

5   Conclusion

In this paper we have contributed to the study of OTIS-Networks in general and to OTIS-Star in particular through proposing the OTIS-Star network as an attractive network to its factor network. Several topological properties including size, degree, diameter, number of links and shortest distance between any two nodes have been derived. Efficient broadcasting algorithm is introduced for the OTIS-Star. The proposed OTIS-Star could be an attractive alternative over its factor network. It has more efficient broadcasting algorithm compared to typical star networks. Furthermore, OTIS-Star can take advantage of deploying both electronic and optical technologies. As a future research work we could utilize the proposed OTIS-Star in solving real life problems on OTIS-Star including Matrix problems, Fast Fourier transforms. Furthermore proposing an efficient routing for OTIS-Star will be also one of our targets as a future research.

References:

[1] S. B. Akers, D. Harel and B. Krishnamurthy, “The Star Graph: An Attractive Alternative to the n-Cube,” Proc.  Intl.  Conf.  Parallel Processing, 1987, pp. 393-400.

[2] K. Day and A. Al-Ayyoub, “The Cross Product of Interconnection Networks”, IEEE Transactions on Parallel and Distributed Systems, Vol. 8, No. 2, Feb. 1997, pp. 109-118.

[3] A. Menn and A. K. Somani, “An Efficient Sorting Algorithm for the Star Graph Interconnection Network,” Proc. Intl. Conf. on Parallel Processing, 1990, pp.1-8.

[4] S. Sahni and C. Wang, “BPC Permutations on the OTIS-mesh Optoelectronic Computer,” Proceedings of the IEEE Conference on Massively Parallel Programming using Optical Interconnect, 1997, pp. 130-135.

[5] W. Hendrick , O. Kibar, P. Marchand, C. Fan, D. Blerkom, F. McCormick, I. Cokgor, M. Hansen, and S. Esener, “Modeling and Optimisation of the Optical Transpose Interconnection System,” In Optoelectronic Technology Centre, Program Review, Cornell University, Sept. 1995.

[6] G. Marsden, P. Marchand, P. Harvey, and S. Esener, “Optical Transpose Interconnection System Architecture,” Optics letters, 18(13), 1993, pp. 1083-1085

[7] W. Hendrick , O. Kibar, P. Marchand, C. Fan, D. Blerkom, F. McCormick, I. Cokgor, M. Hansen, and S. Esener, “Modelling and Optimisation of the  Optical Transpose Interconnection System,” In Optoelectronic Technology Centre, Program Review, Cornell University, Sept. 1995

[8] F. Zane, P. Marchand, R. Paturi, and S. Esener, “Scalable Network Architecture Using the Optical Transpose Interconnection System (OTIS),“ Journal of Parallel and Distributed Computing 60(2000), pp. 521-538.

[9] C. Wang and S. Sahni, “Basic Operations on the OTIS-mesh Optoelectronic Computer,” IEEE Transactions on Parallel and Distributed Systems, Vol. 9, No. 12, 1998, pp.1226-1236.

[10] S. Sahni and C. Wang, “BPC Permutations on the OTIS-hypercube Optoelectronic Computer,” Informatica, 22(1998), pp. 263-269.

[11] S. Sahni, “Models and Algorithms for Optical and Optoelectronic Parallel Computers,” Proceedings of the International Symposium on Parallel Algorithms and Networks, IEEE Computer Society Press, 1999, pp. 2-7.

[12] Y. Y. Saad and M. H. Schultz, “Topological Properties of Hypercubes,” IEEE Trans. Computers, vol. 37, no. 7, July 1988, pp. 867-872.

[13] K. Day and A. Tripathi, “Arrangement Graphs: A Class of Generalised Star Graphs,” Information Processing Letters, vol. 42, 1992, pp. 235-241.

[14] K. Day and A. Tripathi, “A Comparative Study of Topological Properties of Hypercubes and Star Graphs,” IEEE Trans. Parallel & Distributed Systems, vol. 5, no. 1, 1994, pp. 31-38.

[15] I. Jung and J. Chang, “Embedding Complete Binary Trees in Star Graphs,” Journal of the Korea Information Science Society, vol. 21, no. 2, 1994, pp. 407-415.

[16] S. B. Akers, and B. Krishnamurthy, “A Group Theoretic Model for Symmetric Interconnection Networks,” Proc. Intl. Conf. Parallel Processing, 1986, pp. 216-223.

[17] P. Fragopoulou and S. Akl, “A Parallel Algorithm for Computing Fourier Transforms on the Star Graph,” IEEE Trans. Parallel & Distributed Systems, vol. 5, no. 5, 1994, pp. 525-31.

[18] A. Al-Ayyoub and K. Day, “The Hyperstar Interconnection Network,” J. Parallel & Distributed Computing, vol. 48, no. 2, 1998, pp. 175-199.

[19] C. Wang and S. Sahni, “Matrix Multiplication on the OTIS-mesh Optoelectronic Computer,” Proceedings of the 6th IEEE International Conference on Parallel Interconnects, 1999, pp. 131-138.

[20] C. Wang and S. Sahni, “Image Processing on the OTIS-mesh Optoelectronic Computer,” IEEE Trans. Parallel and Distributed Systems, vol. 11, no. 2, 2000, pp. 97-109.

[21] A. Awwad, A. Al-Ayyoub, M. Ould-Khaoua, “On the Topological Properties of the Arrangement-Star Network,” Journal of Systems Architecture, Vol. 48/11-12, 2003, pp. 325–336.

[22] Jehad-Sadi, A. M. Awwad, Efficient Fault-Tolerant Routing Algorithm for OTIS-Cube Using Unsafety Vectors, Proc. Of ACM Symposium on Applied Computing (SAC04), Nicosia, Cyprus, Mar 14-17, 2004, , pp.1426-1430.

[23] C. Wang and S. Sahni, “Basic Operations on the OTIS-mesh Optoelectronic Computer,” IEEE Transactions on Parallel and Distributed Systems, Vol. 9, No. 12, 1998, pp.1226-1236.

[24] K. Day and A. Al-Ayyoub, “Topological properties of OTIS-Networks”, IEEE Transactions on Parallel and Distributed Systems, Vol. 13, No. 4, Apr. 2002, pp. 359-366.

[25] B. Bose,  R. Broeg, Y. Kwon, and Y. Ashir, “Lee Distance and Topological Properties of k-ary n-cubes”, IEEE Trans. On Computers, Vol 44, No. 8, August 1995,  pp.1021 – 1030.

[26] V. Kumar, A. Grama, A. Gupta, and G. Karypis, Introduction to Parallel Computing Design and Analysis of Algorithms, The Benjamin/Cummings Publishing Company, Redwood City, CA, 1994.

[27] J. P. Sheu, C. T. Wu, and T. S. Chen, “An Optimal Broadcasting Algorithm without Message Redundancy in Star Graphs”, IEEE Trans. Parallel and Distributed Systems, Vol. 6, No. 6, June 1995, pp. 653-658.

[28] J. P. Shue, W. H. Liaw, and T. S. Chen, A Broadcasting Algorithm in Star Graph Interconnection Networks, In Proc. 1992 Int. Conf. Parallel Distributed Syst., Hsinchu, Taiwan, Dec 1992, pp. 204-210.

Ahmad M. Awwad finished both of his B.Sc. degree in Computer Science and his M.Sc. degree in Mathematical Science from Tennessee State University, USA, in 1987, and 1989 respectively.

He gained his Ph.D. in Computer Science from the University of Glasgow, United Kingdom, 2002.

Ahmad Awwad served in Oman as senior lecturer  at the Sohar College and at Zarka Private University as Head of CS Department. He is currently the Program Coordinator of the faculty of Information Technology and Computing at Arab Open University in Kuwait.

His research interests are in field of parallel processing, interconnection networks, product networks, opto-electronic networks, algorithms design for HSPC, and distance learning.

Dr. Ahmad M. Awwad is a member of the IEEE-CS.