UNITAU Network - A Simple Secure Solution

CECÍLIA DE A. C. CESAR

Divisão de Ciência da Computação

ITA - Instituto Tecnológico da Aeronáutica

Pça. Mal. Eduardo Gomes, 50 - Vl Acácias

CEP 12228-900 - Sao Jose dos Campos - SP

BRAZIL

cecilia@ita.br     http://www.ita.br

ANTONIO MONTES

Laboratório de Computação Aplicada

INPE - Instituto Nacional de Pesquisas Espaciais

Av. dos Astronautas, 1758 - Jardim da Granja,

CEP 12227-010  -  São José dos Campos - SP

BRAZIL

montes@lac.inpe.br     http://www.inpe.br

MÁRCIO L. X. DOS SANTOS

Coordenadoria de Informática

UNITAU - Universidade de Taubaté

Rua 4 de março, 432 - Centro

CEP - 12020-270 - Taubaté - SP

BRAZIL

mxsantos@directnet.com.br    http://www.unitau.br

Abstract: - The UNITAU network project is a simple network architecture with a well suited configuration to deal with different logical networks in the same physical environment. Using VLAN by software and hardware and appropriated routing the network is relatively secure and able to treat four logical networks in the same cable and with different Internet accesses.

Key-Words: - VLAN, Network Configuration, Routing, Network Management, Security, Protocols.

1   Introduction

UNITAU, a brazilian city university, has its data communication network with particular characteristics:

- Its departments are spread over the town;

- From each campus to the central node, there is only one optical fiber pair crossing the streets;

- Each campus has one switch connected to the backbone and taking care of the local distribution;

- The Internet access is via ANSP (Academic Network at Sao Paulo).

During the installation phase of this project, some new requirements appeared:

- A new distributed administrative system would be implemented. This system should be isolated from the common intranet and would have no Internet access.

- The traffic directed to ANSP should contain only research data, due to constraints imposed by the ANSP provider. This restriction implied that research data should be separated from education and administrative data with a different external access.

- A new library control distributed system would be implemented to interconnect the departamental libraries.

2   Understanding the Requirements

To accomplish these new tasks, four networks were designed sharing the same backbone.

These networks were called:

- Research network;

- Education network;

- Administrative network;

- Library network;

So, as usual, after the initial restrictive project birth, new needs arrived and challenged the network manager: how, within the same fiber, to separate these traffics?

We studied VLANs - Virtual Local Area Network and the IEEE 802.1Q standard to see if this approach would solve the problem. The next subsection - 2.1, explains briefly the VLAN concept just to understand the configuration contained in Section 3. Section 2.2 comments the need for another external access.

  - IEEE 802.1Q

A VLAN is a logical grouping that allows end users to communicate as if they were physically connected to a single LAN, independently of the physical configuration of the network [1].

The proposed standard IEEE 802.1Q for VLANs, is aimed at:

-  Defining an architecture to logically partition bridged LANs and provide services to defined user groups, independently of physical location;

- Allowing interoperability among multivendor equipment.

This was just our case: to group users and to allow interoperability among 3Com, Cisco and other equipments.

There are several ways to configure VLANs.

- A good choice is the Network Based IP VLANs: when a packet arrives, its IP address is analysed to determine to which VLAN it belongs. This kind of VLAN defines the group associating a VLAN number with an IP range. Unfortunately, the equipment available, our central router - Corebuilder 3500 from 3Com - does not support Network Based IP VLANs.

- The alternative used was Port Based VLAN that logically group together one or more bridge ports. In a static way, you can select the bridge ports that you want to be part of the VLAN.

VLANs that share the same port require tagging, that is, a way to associate the packet with the corresponding group or VLANs [2]. According to IEEE 802.1Q the tagging scheme is the following:

- Each VLAN has an identification, called VID;

- The VID is inserted in an outgoing data frame if tagging is enabled on the transmit port for that VLAN.

- Data frames that are received by the system are assigned to the VLAN that corresponds to the VID contained in the tag.

When more than one VLAN was defined in a port, we used VLAN tagging. Of course, the equipment connected to this port has to treat tagging also.

  - Alternative external access

Since the ANSP provider has restricted the traffic in its network, it is necessary to separate research data flowing via ANSP way from the rest of it, flowing via another private ISP.

But there is only one central router, with only one default route. How to separate the traffic from four logical networks arriving in the same optical fiber pair? How to decide the right via, not using BGP, not another router and nothing else complex and expensive?

Through an arrangement with a private ISP, a new IP range was given to the University, but the task of separating the traffic was our responsibility. The next section explains the solution adopted.

3   Problem Solution

For security reason, each workstation is in only one of the four networks.

The solution to integrate these four networks involves basic routing and VLAN. The following items compose the solution:

- The default route is via ANSP for the research computers that uses valid IP addresses. The gateway for each station is the central router that uses its routing table to decide the destiny of each packet.

- For the stations in the education-network the gateway is one firewall connected to one port of the central router. To block the routing of both networks, the education-network and research-network, two decisions were made:

- To stablish a private IP range (10.30.0.0/16) for the education-network. This range is unknown to the central router;

- To create a new VLAN - edu-VLAN. When a packet from edu-VLAN arrives at the central router, this packet is forwarded to the gateway port by the 802.1Q standard, because this port takes part also in the edu-VLAN.

So, even if these private IPs aren’t in the routing table, they are forwarded to the right port.

A similar action was adopted for the administrative network:

- The adm-VLAN was created;

- Another private IP range (10.10.0.0/16) was defined.

The difference here is that this network has no Internet access, and so the ports in the central router for external access don’t take part in the adm-VLAN.

The administrative system has a particular router to allow dial-up access from distant sites. This router is the gateway to the stations in the administrative network and is connected to one port of the central router that takes part in the adm-VLAN.

The library network has the same approach, but the number of stations is smaller than that of the others networks and it has no gateway.

Fig. 1 illustrates the four networks integrated and the flow to the Internet.

Internet via private ISP

Internet via ANSP

lib net

Adm router

Firewall

edu-net

Firewall  research-net

edu-vlan

default

vlan

Central Router

adm-vlan

lib-vlan

Packets from the four networks of  a specific campus

edu-vlan

default-vlan

Fig 1 - Integrated VLANs

4   Refining the solution

After the configuration cited in section 3 was completed, we noticed that the traffic in edu-VLAN was too high. That happens because the main activity of the University is education, and the majority of the laboratories belongs to the education network.

We noticed also that, as the networks were isolated, how were we going to manage them?

Section 4.1 treats the traffic solution and section 4.2 treats the network management problem.

4.1	Increasing the number of VLANs

We decided to define one education VLAN for each department, to isolate the departments from each other, improving the network utilization and its security. A medicine-edu-VLAN, an engineering-edu-VLAN were created, and so on.

At the gateway port to the Internet via private ISP there is a VLAN overlap, which means that tags must be used to differentiate the various VLANs. The central router treats VLAN tags, but what about the firewall connected to it? The equipment connected to a port that treats tags, must treat tags also.

The common NICs do not support the 802.1Q standard, so the idea was to treat VLANs by software. Our firewall is a station with OpenBSD. This operating system allows the creation of a pseudo-interface that takes the packet from the NIC, manipulates the tag and gives the packet to the kernel, as the NIC has treated the VLAN.

Figure 2 shows this refinement.

Internet via private ISP

Internet via ANSP

lib net

Adm router

Firewall

edu-net

Firewall  research-net

med-edu-vlan

default

vlan

eng-edu-vlan

Central Router

adm-vlan

adm-vlan

lib-vlan

lib-vlan

med-edu-vlan

eng-edu-vlan

default-vlan

default-vlan

Packets from  medicine campus

Packets from  engineering campus

Fig. 2 - Refining the solution

Of course there is some delay in the firewall to treat all packets this way, but the output band is below 
2 Mbps, and this manipulation does not compromise the rate.

The central router can manipulate 4094 VLANs and the departmental switches until 16 VLANs, so this increase in the number of VLANs is not a problem any more.

4.2	Network Management

All these networks must be managed.

The first idea was to have a central management station participating in the four networks. The problem with this idea is that this station would be the weak point for  intrusion in the administrative network.

So, we decided that the two networks that had Internet access, and students access, and so would be more exposed to attacks, would be managed by the unique management station. The biggest difficulty in management is in these two networks, because they have a large number of users and stations and greater external exposition.

The critical network that has the administrative system would be managed by another dedicated station in the same room where is the network manager, but physically isolated from the public networks.

The library network is the smallest one and has no dedicated station for management.

5   Conclusion

The solution pointed here indicates that even with few resources it is possible to have an efficient and secure configuration. With just one optical fiber pair from each site, one central router, standards protocols and a free operating system one can achieve independent logical networks with a relatively low security risk and easily managed.

IEEE 802.1Q standard with appropriated routing can solve many real problems and is available on almost all network equipment from different vendors.

References:

[1] 3COM, Corebuilder 3500 - Implementation Guide. 3COM. November 1999, pp 156-207

[2] Marconi Communications, “Virtual LANs and 802.1Q“ http://www.marconi.com/media/vlan100.pdf  consulted on 25/june/2001