Studying Traffic Engineering in Next Generation Internet

+SOLANGE DA SILVA, PAULO ROBERTO GUARDIEIRO*

+Computer Science Department, Catholic University of Goias, Goiânia - GO

* Faculty of Electrical Engineering Federal, University of Uberlândia, Uberlândia - MG

BRAZIL

solanges@alunos.ufu.br, prguardieiro@ufu.br

Abstract: - In this article we present a study about the traffic engineering in next generation Internet, with a specific approach of the MPLS network technology. It describes traffic engineering components and features as well as its advantages, highlighting the results in terms of the best performance and scalability, essential in the IP network. It presents an MPLS overview, and also MPLS as a tool to implement traffic engineering, and the CBR as well. In addition, it is presented a comparison between the Label Distribution Protocols CR-LDP and RSVP-TE, showing the more relevant features in relation to their use in backbone networks.

Key-Words: - MPLS; Traffic Engineering; Label Switching; QoS; CBR; CR-LDP; RSVP-TE

1   Introduction

The exponential growth of the Internet and its new applications, mainly real time applications (Video Conference and Voice over IP), demand that the Internet offers performance that satisfies the users and also the specific requirements of the applications, even in high traffic situations.

Conventional IP networks use hop-by-hop routing for the packet forwarding, where each router needs to check each datagram and determine its next hop in the route. This procedure generates latency and end-to-end jitter, what makes impracticable to have real-time applications in the next generation Internet.

Aiming to solve these issues and searching to adapt the Internet to the current necessities, new mechanisms have been proposed. MPLS (Multiprotocol Label Switching) technology is an example of these mechanisms, which was proposed by IETF (Internet Engineering Task Force).

The aim of this article is to present a study about the traffic engineering in next generation Internet, with specific approaching of the MPLS network technology. It describes traffic engineering components and features as well as its advantages, highlighting the results in terms of the best performance and scalability.

In addition, it is presented a comparison among the signaling protocols CR-LDP (Constraint Routed Label Distribution Protocols), RSVP (Resource Reservation Protocol) and RSVP-TE (RSVP with Traffic Engineering extensions), showing the more relevant features of each when used in backbone networks.

This paper is organized as follows: Section 2 presents the traffic engineering definitions, application necessity and benefits. Section 3 deals with the MPLS technology. Section 4 presents the MPLS as a tool to implement traffic engineering. In Section 5 a comparison among the label distribution protocols CR-LDP, RSVP and RSVP-TE is done. And finally, the last section contains the conclusions.

## 2   Traffic Engineering Feature

Today’s Internet only provides best-effort service. With the rapid transformation of the Internet into a commercial infrastructure, demands for service quality have rapidly developed [16].

The IETF has proposed many service models and mechanisms to meet the requirement for QoS. Among these mechanisms there is Traffic Engineering.

Network congestion can be caused by lack of network resources or uneven distribution of traffic. For simple networks, it may be possible for networks administrators to manually configure the cost of the links so that traffic can be evenly distributed, but for complex ISP networks this is almost impossible [15].

Traffic engineering is the process of arranging how traffic flows through the network, so that congestion caused by irregular network utilization can be avoided.

Traffic engineering is generally done in the core of the network where network conditions are less dynamic and the network topology is fully meshed.

RFC 2702 [10] considers that traffic engineering is responsible for the optimization of performance in the operational level and its greater objective is to improve the efficiency and the reliability of the network operations, while, simultaneously, it optimizes the use of the network resources and traffic performance.

A practical function of traffic engineering in IP networks is the mapping of traffic onto the network infrastructures to achieve specific performance objectives.

Generally, performance objectives can be traffic-oriented and/or resource-oriented. Traffic-oriented performance objectives relate to the improvement of the QoS provisioned to Internet traffic. In a single class, best effort Internet service model, the key traffic-oriented performance objectives include minimization of traffic loss, minimization of the delay, maximization of throughput, and enforcement of service level agreements (SLAs) [8].

Resource-oriented performance objectives relate to the optimization of the utilization of network resources. Efficient resource allocation is the basic approach to secure resource-oriented performance objectives. They also deal with the networks resources, such as communication links, routers, and servers. Efficient management of these resources is vital to the attainment of resource-oriented performance objectives.

And it is important to mention that efficient management of the available bandwidth is the essence of traffic engineering.

The application of the traffic engineering in the Internet brings a series of benefits, such as: more efficient use of the available bandwidth, fast re-routing in cases of failures, reduction of the operational costs of the network, better quality of service, reduction of losses of packets and maximize of throughput for reducing the delay and jitter, taking to the users a better service.

Traffic engineering helps the network providers make the best use of available resources, spreading the load over the layer 2 links, and allowing some links to be reserved for certain classes of traffic or for particular customers [18].

3   MPLS Technology

In the 1990’s decade, the Internet became commercial and a growth phenomenon increased its traffic.

A diversity of applications had appeared, requiring the development of approaches to improve the forwarding of packets.

The IETF has tried to create technologies to integrate routing based protocols, as IP, with the ATM switching technology, to get a network that could offer easiness of management, bandwidth reserve, QoS guarantees and native support to the multicast.

MPOA (Multiprotocol Over ATM) [1] and the I-PNNI (Integrated Private Network to Network Interface) [2] were the first to be developed, but they had not gotten success because of their complexity, management and implementation difficulties.

In the sequence, other solutions had appeared to integrate the advantages of the network switching to IP protocol. Ipsilon presented the IP Switching [3], using ATM switches. In the following, Cisco developed the Tag Switching [4], adding to each packet of data a label (or tag), allowing its transmission throughout virtual circuits. All the forwarding is based on the tag and, the packet headers are not examined at each network hop.

After that, Toshiba developed CSR (Cell Switched Router) [5] and IBM, ARIS (Aggregate Route-Based IP Switching) [6], both based on Tag Switching. Trying to standardize the forwarding-based IP switching, the IETF created the MPLS Group, and the MPLS was developed to reduce the complexity of forwarding of IP network [7].

3.1   MPLS Overview

MPLS evolved from Cisco’s Tag Switching. In the open systems interconnection (OSI) seven-layer model, MPLS is between layer 2 (link layer) and layer 3 (network layer).

MPLS uses a technique known as label switching to forward data through the network. A small, fixed-format labels are inserted in front of each packet on entry into MPLS network. At each hop across the network, the packet is routed based on the value of the incoming label and dispatched to an outwards interface with a new label value.

Fig. 1 – MPLS header

The Fig. 1 shows the MPLS header. Each MPLS packet has a header that contains a 20-bit label, a 3-bit Class of Service (CoS) field, a 1-bit label stack indicator, and an 8-bit Time to Live (TTL) field.

The MPLS header is encapsulated between the link layer header and the network layer header [16]. In MPLS, the assignment of a particular packet to a particular FEC (Forwarding Equivalent Class) is done just once, as the packet enters the network.

FEC is the set of all packets that are forwarded in the same way. One or more FECs may be mapped to a single LSP. The FEC to which the packet is assigned is encoded as a short fixed length value known as “label”.

When a packet is forwarded to its next hop, the label is sent along with it; that is, the packets are “labeled” before they are forwarded [7].

At subsequent hops, there is no further analysis of the packet’s network layer header. The old label is replaced with the new label and the packet is forwarded to its next hop. A label identifies the path that the packet will go through in the network. It is carried or encapsulated in a link layer header together with the packet. Packets that share the same requirements for their transport are congregated in FECs and thus, all the packets receive the same treatment of routing until the egress point of the MPLS network.

An MPLS-capable router is called LSR (Label Switching Router) and examines only the label in forwarding the packet. The path that the data traverses through a network is defined by the transition in label values, as label is swapped at each LSR. Such a path is called a LSP (Label Switched Path).

MPLS needs a protocol to distribute labels to set up LSPs. The MPLS architecture does not mandate of using a single protocol for the distribution of labels between LSRs. In fact it specifically allows multiple protocols to be used in different scenarios.

The IETF MPLS Working Group has specified LDP (Label Distribution Protocol) as a protocol for label assignment, but RSVP and CR-LDP can also be used. LSRs in a MPLS network use link-level forwarding to provide a simple and fast packet forwarding capacity. As MPLS is connection-oriented, it introduces signaling in IP networks so that it can establish and keep the switched virtual circuits or LSPs. This can bring disadvantages; however signaling can play functions of QoS, for example, reserving resources to the longest path. Each LSR in a MPLS domain builds a table to specify the way a packet must be forwarded.

This table is called LIB (Label Information Base) and it contains the labels and the FEC each label is associated (FEC-to-Label). When labeled packet reaches the egress point of the network, the label is removed by the egress LSR of MPLS domain. Thus, the original IP packet can be correctly delivered to its host destination [8].

The basic operation of a MPLS network is illustrated in Fig. 2.

Fig. 2 – Forwarding in the MPLS network

The IP packet enters in an MPLS domain through of router called “Ingress LSR” (router C). This router classifies it into an FEC. It then generates an MPLS header and assigns it a label. In this case the packet receives the label “21”.

This labeled packet will be forwarded to the next LSR. The next LSR, in the core network, receives this packet, examines its internal forwarding table and exchanges label “21” for “14” and sends the packet to the next LSR, that performs the same operation, exchanging label “14” for “44”, and sends the packet to next LSR. The next LSR is the router in the exit of the MPLS network called “egress LSR” (router F). This router removes label “44” from the packet and forwards it as an original IP packet to the destination (host G).

Note that all the forwarding process is done only examining the internal forwarding table in each router, and in accordance with the received label.

In the MPLS, routing can occur of two ways: hop-by-hop routing or explicit routing.

Explicit routing is disentailed of conventional routing. It allows the establishment of alternative paths, less congested, not necessarily the shortest paths, however the most adjusted ones to the requirements of a given application.

Thus, it is possible to implant something essential for the IP operation networks called traffic engineering.

4   MPLS as a Tool to Implement

Traffic Engineering

The RFC 2702[10] defines the requirements for the accomplishment of traffic engineering over MPLS and identify its main components and functionalities. The main issue involves the creation of LSPs tunnels by using traffic engineering (LSP-TE tunnels) in the network for the IP traffic forwarding.

The route to be followed by a tunnel is determined in a static way or through a constraint-based routing carried by the edge routers.

The traffic engineering based on MPLS offers the following benefits: traffic engineering, traffic engineering in layer 3, flows based on the available resources, and recovery of failures in links.

MPLS provides flexibility to the network operator to manage the traffic through two types of explicit routing:

- Explicit Routing Strict, where the complete route (nodes and interfaces) that the ER-LSP will use is specified by the operator;

- Explicit Routing Loose, such a route allows some routing flexibility like rerouting, besides minimizing overhead of configuration.

Explicit routing is particularly useful to force an LSP down a path that differs from one offered by the routing protocol. It can be used to distribute traffic in a busy network, to route around network failures or hot spots, or to provide pre-allocated back-up LSPs to protect against network failures [18].

MPLS is strategically significant for traffic engineering because it can potentially provide most of the functionality available from the overlay model, in an integrated manner, and at a lower cost compared to the currently competing alternatives. [10].

The main use for MPLS will be to allow improved traffic engineering on the ISP backbone networks. MPLS is useful for traffic engineering because it provides faster packet classification and forwarding and it provides an efficient tunneling mechanism.

4.1   Traffic Engineering Using Constraint-

Based Routing

Constraint-based routing (CBR) is a routing scheme and it is an important tool for making the traffic engineering process automatic. The objectives of constraint-based routing are to select routes that can meet certain QoS requirements and to increase utilization of the network.

IGP (Internet Gateway Protocols) routing protocols use SPF (Shortest Path First) algorithm, to select the route that presents the least cost, conserving the resources of the network. However, the routes of different origins can be overlapped in some links, causing congestion in the same ones.

The use of CBR improves traffic engineering; therefore, it allows the selection of distinct routes gotten by the SPF, based on the defined restrictions. Besides it optimizes the resources and the network performance using only one algorithm. For the purposes of QoS and traffic engineering, flows in MPLS are called traffic trunks.

Traffic trunks are defined as a collection of individual flows that share two properties: the first one is that flows are forwarded along the same path and the second is that they all share the same class of service.

So, to be possible to implement the traffic engineering in MPLS network, it is necessary:

A set of attributes associated to the traffic trunks that specify their behavior, and a set of attributes of the LSPs where these trunks pass through; #### Basic Attributes of traffic trunks

Traffic parameter attributes and management attributes like peak rates, average rates, and burst size;

Generic path selection and management attributes like resource class affinity, adaptivity and load distribution;

Priority attribute to define the relative importance;

Preemption attribute to determine whether the traffic trunk can preempt a specific traffic trunk

Resilience attribute that determines the behavior of a traffic trunk under fault conditions;

- An environment for the implementation of CBR, to select the path for the traffic trunks.

Table 1 - Basic Attributes of traffic trunks

5   A Comparison Among Label

Distribution Protocols

In this section is presented a comparison among the label distribution protocols CR-LDP [11], RSVP [13] and its extension, the RSVP-TE [12].

The Table 2 summarizes the main technical similarities and differences among the label distribution protocols.

RSVP uses a message exchange to reserve resources across a network for IP flows. RSVP uses IP datagrams (or UDP at the margins of the network) to communicate between LSRs peers. The extensions to RSVP for LSP tunnels enhance generic RSVP so that it can be used to distribute MPLS labels.

MPLS traffic engineering is established dynamically, and keeps a LSP tunnel through MPLS domain using the CR-LDP protocol and the RSVP-TE protocol. CR-LDP was considered to implement explicit routing by using CBR.

| CR-LDP was constructed under LDP protocol already developed by MPLS Group, and provides ER-LSP with the option of resource reserve. With the hard state control, CR-LDP keeps the LSP active as long as its entrance in the routing table exists. ### Protocol | RSVP | RSVP-TE | CR-LDP |
| --- | --- | --- | --- |
| Objective | Created to support soft state resource reservation of IntServ of IP networks | Proposed with modifications to support the DiffServ with RSVP for MPLS networks | Created to enable LSP setup for reliable end-to-end DiffServ in MPLS networks |
| Application | For edge and host services | For Backbone Networks | For Carrier Backbone Networks |
| Routing Types | Strict, Loose, No Pinning | Strict, Loose, Pinning | Strict, Loose, Pinned |
| Scalability | No | Yes | Yes |
| Management of the state of the LSP | Soft State | Soft State | Hard State |
| LSP state Refresh | Periodically, in all nodes | Periodically, in all nodes | It is not necessary |
| LSP Architecture | ##### Source Tree | ##### Source Tree | Sink Tree |
| ###### LSP Setup Action | ##### Backwards, Upstream | Backwards, Upstream | Forward, downstream |
| Path Rerouting | Yes | Yes | Yes |
| Failure Notification | Unreliable Procedure | Unreliable Procedure | Reliable Procedure |
| Transport Mechanisms | UDP | UDP | TCP |
| ###### Differentiated Services | Not supported | Supported | Supported |

CR-LDP extends LDP by adding the following functionality [11]:

- Strict and loose explicit routing, encoded as a series of ER-hops contained in a constraint-base Route TLV (Type-length-value encoding scheme – all LDP messages have this structure);

- Route Pinning feature, that prevents the LSP from changing its path even when a better path becomes available at some hop along the loosely source routed portion of the LSP; If a route with sufficient resources cannot be found, existing paths can be rerouted to reallocate resources to the new path by a process called pre-emption;

- Failure handling by re-routing, is supported by using different mechanisms;

- LSPID, to identify the LSP that had been set up;

- Resource class feature that specifies the class of resources that can be explicitly included or excluded from the CR-LSP path;

According to RFC 2205, in summary, the RSVP includes the following features [15]:

- It can make reservations for both unicast and many-to-many multicast flows;

- It is receiver oriented;

- It transports and maintains traffic control and policy control parameters; It has several reservation styles to fit a variety of applications;

- It provides transparent operation through non RSVP-capable routers;

- It supports both IPv4 and IPv6;

- It can make reservations only for unidirectional flows but RSVP-TE has been extended to perform bi-directional path setup.

RSVP-TE is an extension of RSVP. This protocol presents some modifications so it can adequate to traffic engineering in MPLS networks.

RSVP-TE protocol supports:

- Establishing LSP tunnels with or without QoS requirements;

- Dynamic rerouting of an established LSP tunnel;

- Identification and diagnosis of LSP tunnels;

- Preemption or replacement of an established LSP tunnel for administrative reasons;

- Downstream-on-demand label allocation, distribution, and binding;

Controlled-load IntServ service and class-of-service IntServ service [17].

Table 2. Comparison among Label Distribution Protocols

The most obvious difference between CR-LDP and RSVP is the choice of transport protocol used to distribute the label requests. CR-LDP uses UDP to discover MPLS peers and uses connection-oriented TCP sessions to distribute labels requests and RSVP uses connectionless raw IP (or UDP packets at the margins of the network) [18].

Both protocols have similar flows for label setup, sending an end-to-end request and replying with and end-to-end response.

CR-LDP uses TCP to guarantee the reliable transport of its signaling messages. When a failure occurs, a message indicating error is sent using reliable transport and it expects a reply. On the other hand, RSVP does not guarantee this failure notification because no reliable transport infrastructure is used.

CR-LDP protocol supplies a scalable infrastructure to satisfy the requirements of traffic engineering in a backbone network environment.

6   Conclusion

The aim of this article is to present a study about the traffic engineering in next generation Internet, with specific approach of the MPLS technology.

It described traffic engineering components and features as well as its advantages, highlighting the results in terms of the best performance and scalability, essential in the IP network, today and in the future.

MPLS and constraint–based routing can be used together to control the path of traffic to avoid congestion and improve the utility of the networks [16].

By comparing the label distribution protocols:

- Both CR-LDP and RSVP offer flexible approaches to re-routing.

- Both protocols have similar flows for label setup, sending an end-to-end request and replying with and end-to-end response.

- CR-LDP is considered superior to the RSVP-TE, offering a reliable transport mechanism for its signaling messages.

Traffic engineering in MPLS networks improves, effectively, the performance and scalability of the ISPs (Internet Service Providers) and the backbone networks, besides it facilitates the support of new services, such as the real time applications.

References:

[1] ATM Forum, Multiprotocol over ATM (MPOA) version 1.0, STR-MPOA-01.000, 1997.

[2] D. Mcdysan, and Spohn, D., Hands-On ATM, Nova York, McGraw-Hill, 1998.

[3] J. McQuillan, The Arrival of IP Switching, Business Communications Review, 1996.

[4] Y. Rekhter et al., Cisco System Tag Switching Architecture Overview, RFC2105, 1997.

[5] Y. Katsube et al., Internetworking based on Cell Switch Router - Architecture and Protocol Overview, Proc of IEEE vol. 85 nr. 12, 1997.

[6] A. Viswanathan et al., ARIS: Aggregation Route-Based IP Switching, IBM Technical Report TR29.2353, 1998.

[7] R. Eric et al., Multiprotocol Label Switching, RFC-3031, 2001.

[8] B. Ulisses, MPLS and Label Switching Networks, Prentice Hall, 2001.

[9] X. Xipeng, Providing Quality of Service in The Internet, Doctor of Philosophy thesis, Michigan State University, 2000.

[10] A. Daniel O. et al., Requirements for Traffic Engineering over MPLS, RFC 2702, 1999.

[11] J. Bilel et al., Constraint–Based LSP Setup using LDP, work in progress, draft-ietf-mpls-cr-ldp-05.txt, 2001.

[12] A. Daniel O. et al., RSVP-TE: Extensions to RSVP for LSP Tunnels, draft-ietf-mpls-rsvp-lsp-tunnel-08.txt, 2001.

[13] C. Jong-Moon, Analysis of MPLS Traffic Engineering, Proc. 43 rd IEEE Midwest Symp. On Circuits and Systems, Lansing MI: 550-553, 2000.

[14] R. Braden et al., Integrated Services in the Internet Architecture: an Overview, RFC 1633, 1994.

[15] R. Braden et al., Resource ReServation Protocol (RSVP) - Version 1 Functional Specification, RFC 2205, 1997.

[16] X. Xipeng and L.M. Ni, Internet QoS: A Big Picture, IEEE Network, March /April 1999.

[17] R. Srihari, An MPLS-based QoS Architecture for Heterogeneous Networks, Thesis of Master of Science, 2001.

[18] B. Paul and F. Adrian, MPLS Traffic Engineering: a choice of signaling protocols, http://www.dataconnection.com, January, 2000.