Calculation of Operational Parameters in Primary Distribution Networks using Artificial Neural Networks

ANA MARÍA GARCÍA CABEZAS                HERNÁN PRIETO SCHMIDT

Department of Electrical Engineering

# University of  São Paulo

Av. Prof. Luciano Gualberto No.158 trav 3, São Paulo-SP

BRAZIL

acabezas@pea.usp.br

Abstract: A new methodology for estimation of performance parameters in electricity distribution systems is presented in this paper. These parameters include maximum voltage drop, maximum power losses and length of feeders. The methodology, which can used in both planning and operational studies, is based on the MLP - Multi Layer Perceptron paradigm of Artificial Neural Networks.

A second methodology was developed so as to improve MLP performance in the parameter estimation process, as well as to improve the statistical expressions used in the training and validation of the MLP model. This methodology is based on probabilistic simulation and represents an alternative way of generating scenarios for the establishment of statistical expressions. The differences arising between the conventional scenario generation (through enumeration of parameter combinations) and the probabilistic simulation motivated the inclusion of a detailed analysis of the statistical expressions themselves.

Key-Words: Electricity Distribution Systems, Operational parameters, Artificial Neural Networks, Multi Layer Perceptron, Probabilistic simulation.

1  Introduction

Some previous works using the Multi Layer Perceptron (MLP) model of Artificial Neural Networks (ANN) in problems of Electric Power Distribution, especially in load forecasting area, have showed that this approach presents some important advantages, such as implementation easiness, absence of complex mathematical models, high-speed calculation and good precision of results [1], [2].

This work presents a new methodology, based on the ANN technique, for the operational parameters calculation in primary distribution electrical networks. Such parameters included the maximum voltage drop, maximum power losses, trunk length and total feeder length. The proposed methodology can be applied in the planning and in the operation of distribution systems.

In this work, some partial results from a distribution systems planning methodology, developed previously and implemented in SISPAI (Sistema de Planejamento Agregado de Investimentos), are used [3]. In the SISPAI program, operational parameters of primary distribution networks, are estimated through a statistical approach. The developed estimation methodology for such parameters can be considered as an alternative procedure to the SISPAI methodology.

The simplicity of the MLP modeling, which allows the estimation of the electric parameters directly, constitutes a very interesting aspect in comparison with the indirect calculation through invariants implemented in SISPAI. It should be considered the robustness of both models (statistical expressions and MLP), measured by the precision of the results obtained in the case of the networks whose parameters differ considerably from the average network values used in the parameters adjustment stage (regression in the case of the statistical expressions and training in the MLP case).

2 Methodology for Calculation of Operational Parameters in Primary Distribution Networks

The proposed methodology for estimation of performance parameters in primary distribution networks, starting from the network descriptors, is based on the MLP model of Artificial Neural Networks, which uses the Backpropagation algorithm as training method.

2.1  The Multi Layer Network

The MLP model can be defined as a higher-connectivity arrangement of elements called neurons. The MLP model consists of an input layer, one or more hidden layers, and an output layer. Each layer contains neurons and each neuron in the layer is connected to the neurons of the adjacent layer by weighted connections.

The main characteristic of the ANN resides in their capacity to learn [4]. The learning process of a neural network is known as training. An ANN is trained so that the application of a set of inputs produces a set of expected outputs. Each input or output set is constituted by vectors. During the training process input vectors are applied sequentially, while the network adjusts the weights in agreement with an algorithm that tends to minimize the error. As a consequence, the weights of the connections converge gradually to a value such that each input vector produces the expected output vector. The neural network learns when reaches a generalized solution related to problems analyzed.

2.2  Data Collection and Separation in Sets

The problem data are separate in two categories: training data, which will be used in the ANN training, and the validation  or test data, which will be used to verify the ANN performance under real conditions of use.

The data for the training set were obtained from the SISPAI program, once established the combinations of the descriptors (independent variables) [5].

Next are presented the input and output MLP variables:

Input variables (independent):

- action angle (o)

- area of service (km2)

- number of load points

- maximum demand (kVA)

- exponent of the load density function

- resistance of the trunk feeder (ohm/km)

- resistance of the lateral branches (ohm/km)

Output variables (dependent):

- total length of the feeder (km)

- length of the trunk feeder (km)

- maximum voltage drop in the peak hour (%)

- maximum power losses in the peak hour (kW)

The data for the generation of the test set are obtained from the evolution of the network through the time. In the SISPAI program, this stage is executed by modules that use statistical expressions generated previously, to estimate the performance parameters of the electric networks.

2.3 Definition of the MLP Architecture and Training

The adopted model was the Multi Layer Perceptron (MLP), with Backpropagation training. Several configurations were previously studied, being finally chosen those producing the best results. In the studied cases, both the number of hidden layers and the number of neurons in those layers have been varied. All the Cases analyzed had 7 neurons in the input layer and 4 neurons in the output layer.

2.4  Validation of the Obtained Results

The objective of this stage is to assure that ANN is capable of generalize the output vector to any input vector; that is, to prove that the ANN generalized the input/output relation but did not memorize it. In order to achieve the above objective the validation data set will be used to verify the ANN performance with another data not previously used.

## 3   Results

3.1  First Application: Data Generated by        SISPAI

In this first application, Cases 1 to 4 are presented. The state space of independent variables was generated by enumeration of parameter combinations within the SISPAI program.

In order to improve the obtained results in those cases, new cases were generated (Cases 5 to 8) in which the number of combinations of the independent variables, was increased considerably.

In Table 1 the MLP configurations corresponding to Cases 1 through 8 are shown.

Table.1 ANN Topology

| Case | Hidden Layer | Hidden Neurons | Tolerance |
| --- | --- | --- | --- |
| 1, 5 | 1 | 25 | 0.001 |
| 2, 6 | 2 | 20/20 | 0.01 |
| 3, 7 | 2 | 20/20 | 0.001 |
| 4, 8 | 2 | 25/25 | 0.001 |

3.1.1  Analysis of Results

Fig.1 shows the average error obtained as a result of  the training process for Cases 1 to 8. It can be observed that the smallest errors are obtained in Cases 3, 4, 7 and 8. It also can be appreciated that these errors are lightly smaller for the total length (Ltot) and trunk length (Ltr) variables in Cases 7 and 8, and evidently smaller for the maximum voltage drop (dV) and maximum power losses (P) variables in the same cases (7 and 8).

Fig.1 Average training error for Cases 1 to 8

Fig.2 shows the average errors obtained in the test of Cases 1 to 8. Since the MLP network interpolates  exclusively with the data used in the training process, it  will tend to supply a high error value in the case of those validation data that are not close to the data used during the training process.

When compared case to case, the errors in the tests become much higher than the errors obtained in the training session. On average this increased error was 50% for Cases 1 to 4, and 30% for Cases 5 to 8.

Fig.2 Average tests error for Cases 1 to 8

The training time needed for the current analysis, (using a Pentium III 700MHz computer), varied from 2h02min to 8h46min. The smallest time corresponded to the configuration of Cases 1,5,9,13, with a single hidden layer of 25 neurons (2345 input vectors), and the largest time corresponded to the configuration of the Cases 4,8,12,16, with two hidden layers of 25 neurons (2345 input vectors). However, the MLP network trained needed only 0.25 seconds to be tested when using a test set with 228 vectors.

The error tolerance is the parameter which determines if the MLP output is close to the expected output, during the training process. When a small value of error tolerance is chosen, the difference between the MLP output and the expected output will be smaller. Among the chosen configurations to be presented in this work, those pertaining to Cases 2 and 3 (similarly Cases 6 and 7, 10 and 11, 14 and 15) only differ in the error tolerance value, being 0.01 in Case 2, and 0.001 in Case 3 (see Tables 1,2 and 3).

In the results shown in Figs.1 and 2, it can be seen that the MLP networks trained in the first four cases, generated the highest interpolation errors in the tests. In such cases the training file was obtained with a smaller number of independent variables combinations. This behavior is linked to a deficient generalization of the data during the training stage.

  - Second Application: Independent Variables Obtained by Probabilistic Simulation

In order to improve the interpolation quality of the MLP network, a new methodology was developed and implemented for the independent variable generation in SISPAI. This methodology utilizes probabilistic simulation instead of explicit enumeration used in the conventional version of SISPAI.

## Generation of the Independent Variables through Probabilistic Simulation

In Fig.3, typical values of the independent variables (i.e. Number of load points and Demand), are represented.

Fig.3 Representation of the two independent variables

The existence of a great number of ‘empty spaces’ that hinders the establishment of a good interpolation (mainly if a non linear model, as in the case of the MLP, is adopted), can be observed.

With the objective of improving the interpolation quality produced by the MLP network, a new way of generating values for the independent variables, through probabilistic simulation, was implemented. As a consequence, a state space where the points are distributed with uniform density (Fig.4), is obtained.

Fig.4 Representation of independent variables  generated by probabilistic simulation

3.2.2  Cases 9 to 12

In Table 2 the MLP configurations corresponding to Cases 9 through 12 are shown.

Table.2 ANN Topology

| Case | Hidden Layer | Hidden Neurons | Tolerance |
| --- | --- | --- | --- |
| 9 | 1 | 25 | 0.001 |
| 10 | 2 | 20/20 | 0.01 |
| 11 | 2 | 20/20 | 0.001 |
| 12 | 2 | 25/25 | 0.001 |

Fig.5 shows the average errors obtained, for each output variable from the training process of the cases presented. Once again the MLP network reaches the smaller errors in the configurations pertaining to Cases 11 and 12, highlighting the decrease of the error in the maximum power losses variable.

Fig.5 Average training error for Cases 9 to 12

Fig.6 shows the average errors obtained from testing the MLP networks for Cases 9 to 12. In Case 12, the obtained error for the total length and trunk length variables, were acceptably low (between 4% and 5%). However, for the maximum voltage drop and maximum power losses variables the average error were higher to 10%.

Fig.6 Average tests error for Cases 9 to 12

Considering that the MLP network configurations used in the present cases are equivalent to one used in  Cases 1 to 8, it can be concluded that the superior performance of the MLP networks presented here is due to the fact that this were trained with data generated through probabilistic simulation, which present an uniform distribution in the space.

  - Analysis of the Statistical Expressions Generated by SISPAI

In previous studies the performance of the MLP networks was evaluated having as a reference the results supplied by the statistical expressions. In the first application the conventional methodology of SISPAI was utilized; whereas in the second one using the probabilistic simulation method was used. This fact and some of the errors considered significant, led to a further analysis related to the validity of the statistical expressions. With this way of reasoning, the following analysis involving the statistical expressions and the points (scenarios) that generated them, are made:

- Analysis 1, in which the scenarios are specified by the user, through the combination of the  independent variable values. To these scenarios the conventional statistical expressions have been applied, being finally evaluated the resulting interpolation error;

-  Analysis 2, in which the same scenarios of Analysis 1 (specified by the user) are considered. In this case, the statistical expressions generated by probabilistic simulation were applied.

- Analysis 3, in which the scenarios generated automatically through probabilistic simulation are considered. To these scenarios the statistical expressions generated by themselves are applied.

A final analysis (Analysis 4) was also carried out. In this analysis the MLP network was trained with calculated scenarios by statistical expressions obtained by probabilistic simulation, instead of using the scenarios generated in the simulation.

    - Discussion of the Analyses 1, 2 and 3

Figs. 7 and 8 show the interpolation average error and maximum errors obtained in the three analysis.

Fig.7 Average interpolation error

Fig.8 Maximum interpolation error

Comparing the results of Analysis 1 (scenarios and statistical expressions generated by conventional SISPAI) to those from Analysis 3 (scenarios and statistical expressions generated by probabilistic simulation), it can be verified that both the average and the maximum error are smaller when statistical expressions resulting of scenarios generated by probabilistic simulation are used.

It can also be observed that the error obtained in the Analysis 2 (statistical expressions resulting from scenarios generated by probabilistic simulation, and which are applied to scenarios specified conventionally) are located between the error values of Analysis 1 and 3, with some exceptions.

The results obtained reveal a common characteristic to  Analysis 1, 2 and 3. Such a characteristic is related to the fact that the largest interpolation errors of the statistical expressions correspond to the maximum power losses.

3.3.2 Analysis 4

In this last analysis four new cases are presented (Cases 13 to 16). In each case the MLP network was trained with calculated scenarios by the statistical expressions obtained through probabilistic simulation.

The purpose of this analysis was to separate the contribution of both statistical expressions and MLP to the interpolation error.

In Table 3 the MLP configurations corresponding to Cases 13 through 16 are shown.

Table.3 ANN Topology

| Case | Hidden Layer | Hidden Neurons | Tolerance |
| --- | --- | --- | --- |
| 13 | 1 | 25 | 0.001 |
| 14 | 2 | 20/20 | 0.01 |
| 15 | 2 | 20/20 | 0.001 |
| 16 | 2 | 25/25 | 0.001 |

Fig.9, presents the average errors, obtained in the training process for Cases 13 to 16. It can be observed that the smallest errors are obtained in the MLP networks of Cases 15 and 16.

Fig.9 Average training error for Cases 12 to 16

Fig.10, shows the average errors obtained when testing the MLP networks for Cases 13 to 16. The network configuration of Case 15 generated the smallest error values, being 3.8% and 3% for the total lengths and for trunk length, respectively. Likewise, it presented a 10% and 12.8% error values, for the maximum voltage drop and maximum power losses, respectively.

Fig.10 Average tests error for Cases 12 to 16

Comparing Cases 13 through 16 to Cases 9 through 12, it can be observed that the obtained test errors decrease in all the output variables except for the Ltot variable. This happened when the MLP network was trained with vectors obtained from the application of  statistical expressions to the scenarios used to generate the expressions.

The average error corresponding to the maximum power losses decreased from 23.5% to 12.8%. The average error of the maximum voltage drop decreased from 13.1% to 10%. In the same way, the average error of the trunk length decreased from 4.3% to 3%. Finally, the average error of the total length remained at approximately 3.6%.

Since in Cases 13 through 16 the scenarios used in  MLP training were calculated starting from the statistical expressions, it can be concluded that the error in those cases corresponds to the intrinsic interpolation error of the MLP network. Reduction of the error in Cases 13 through 16 in relation to Cases 9 through 12 corresponds to the error component from the statistical expressions.

## 4   Conclusion

Based on the studies and experiences presented in this paper, the following  conclusions are reached:

- The MLP network with the Backpropagation training algorithm can be applied to the calculation of the operational parameters in distribution systems. From the performance point of view,  the MLP network presented a satisfactory characteristic of data generalization. The network was capable to capture the type of relationship existing between its inputs and outputs;

-  The crucial factors for a good performance of ANN are the data availability training and the widespread feature of such data. In other words, if the data include a largest number of possible combinations, the network can supply an appropriate response for any new situation;

- The performance of the MLP network configurations presented in this work varied as a function of the training data origin as well as of the own network structure. This being less satisfactory when using a small number of combinations among the descriptors, and superior when using data generated by probabilistic simulation. It also proved that, in order to achieve the proposed objective, it is necessary to use MLP networks with two hidden layers each with 20 neurons;

- When applied the statistical expressions to the scenarios  generated by themselves, the error values for the maximum power losses became significant. Those errors are present in the data obtained through SISPAI. As a consequence high error values in the tests were obtained;

- The probabilistic simulation used to generate the scenarios produced a considerable gain in relation to the SISPAI conventional scenario generation (enumeration of combinations);

- The performance analysis of the MLP network and  the statistical expressions provided similar results. These results can be translated in significant errors, in the case of the maximum power losses. Calculation of this variable (maximum power loss) presents a high degree of difficulty for both the statistical expressions and the MLP network, and is subject of ongoing research.

References:

[1]  Yasuoka, J.; Brittes, J.L.P.; Schmidt, H.P.; Jardini, J.A. Artificial Neural Network-Based Distribution Substation and Feeder Load Forecast. In. 16th International Conference and Exhibition on Electricity Distribution - CIRED, RAI, Amsterdam, 2001.

[2] Jardini, J.A.; Schmidt, H.P.; Tahan, C.M.V; Oliveira, C.C.B; Ahn, S.U. Distribution transformer loss of life evaluation: a novel approach based on daily load profiles. IEEE Transactions on Power Delivery, Vol.15, No.1, 2000, pp. 361-366.

[3] Nota Técnica CED103/Plan 002/RL 001/OR. Especificação do Modelo de Planejamento Agregado de Investimentos em Sistemas de Distribuição. CESP-CPFL-ELETROPAULO-EPUSP-IEE/USP, March 1994. (In portuguese)

[4]   Wasserman,  P.D.    Neural   Computing:  Theory   and Practice. Van Nostrand Reinhold, 1989.

[5]  Valente,  A.L.   Modelo  probabilístico  para avaliação do desempenho de redes de distribuição primária. Ph.D. Diss, University of São Paulo, 1997. (In Portuguese)

[6] García, A. M. C. Cálculo de parâmetros operacionais de desempenho de redes primárias de distribuição utilizando redes neurais artificiais. M.Sc. Diss, University of São Paulo, 2002. (In Portuguese)