Map-based variable rate applicator of liquid fertilizers based on artificial neural networks identification techniques

JOSÉ ALFREDO ULSON, ROBERTA S. ULSON, ANDRÉ N. DE SOUZA, IVAN N. DA SILVA

Department of Electrical Engineering

State University of São Paulo -UNESP

CP 473, Bauru - SP

BRAZIL

ulson@feb.unesp.br

Abstract: - Variable rate fertilization aims to improve fertilizer use efficiency and reduce leaching by varying the fertilizer rates according to the needs of each portion of the crop and the soil. The application of liquid fertilizers through variable rate techniques implemented by classical control approaches needs a electromagnetic flow meter in the primary controlled variable. This paper proposes an intelligent control system based on Artificial Neural Networks of the type multilayer perceptron for the identification and control the fertilizer flow rate. In this approach, there is no flow meter since the control is made through secondary variables. The neural network training is made by the algorithm of Levenberg-Marquardt with training data obtained from laboratory measurements. The results indicate an accurate, fast, stable and low cost control system for variable rate applicators.

Key-Words: - Precision agriculture, liquid fertilizers, variable rate application, neural networks, intelligent systems, neural control, identification.

### 1   Introduction

According to the principles of the precision agriculture, the need of nutrients for a certain plantation varies over the area and the deepness of the soil (spatial variability). Thus, the rational use of the fertilizers is obtained when the application obeys the real necessity of each portion of the soil.

For a typical Brazilian sugar cane grower, fertilizers  accounts for over one fourth of the total production expenses. Therefore, the effective management of these costs has had a significant impact on the eventual profit obtained from crop production processes. Besides, it is well known that so much the deficiency as the excess of nutrients can reduce productivity and affect the crop quality. For instance, the excess of nitrogen in the soil can increase the vegetative growth, however, it can reduce the sucrose content in sugar beets and leave them more tender, turning the more exposed to the attack of plagues [1]. This excesses that leach through the soil often ends up in groundwater wells, streams and lakes - the same resource that provides water for most of the world population.

In this context, the equipments used for liquid fertilizers application should have some control systems that can change the fertilizer application rate in accordance with the real necessity of the crop and the soil. However, variable rate application systems has have high cost for the most of Brazilian’s farmers and small farming companies. Consequently, the fertilization operation is still made by an average rate within a field. Thus, the precision and the accuracy of the fertilizer application are poor.

This work describes an intelligent control system based on Artificial Neural Networks (ANN) destined to control the application of NPK liquid fertilizer without necessity of flow meter devices. More specifically, it is used neural nets of the type multilayer perceptron (MLP) [2][3] for the identification and control of the fertilizer application system, using indirect measurements like tank level, centrifugal speed pump, and others variables and parameters that are related to the fertilizer flow through the applicator system. The network training is made by the algorithm of Levenberg-Marquardt [4] with training data obtained from laboratory measurements.

### 2   Liquid Fertilizer Applicators

In Brazil, the most of the liquid fertilizer applicators controls the flow rate by branching (Fig.1). The flow of the centrifugal pump is split into one stream, which is redirected into the tank, and another stream, which is fed into the spreading nozzles.

The recirculation flow is vary important because it sustains the stability of the agricultural suspensions avoiding sedimentation and syneresis [5].

In this equipment, the set-point is manually done and  an average application rate is applied. So, the tractor speed and engine speed should be fixed. Since the tractor does not allow to control these variables, the fertilizer quantity for a certain portions can become incorrect.

On-off valve

Nozzle

Centrifugal pump

Manual 3 way valve

Tank

Recirculation

Fig.1. Typical liquid fertilizer applicator.

Control

valve

Flow meter

Radar

Nuzzles

Tank

Yield  monitor

DGPS

Parameters

and map

.

Fig.2. Conventional map-based variable rate applicator of liquid fertilizer.

On other hand, the application process through variable rates can be made through control systems implemented by conventional techniques with feedback as illustrated in Fig.2. In other words, the measurements from a fertilizer flow meter is used to maintain a pre-set application rate come from a fertility map. All these systems are based on maps, so a GPS (Geographic Positioning System) or DGPS (Differential Geographic Positioning System) becomes necessary.

Normally, a magnetic inductive flow meter is used since the fertilizer formulations are suspensions. However, this device represents up to 36% of the total cost of a small liquid fertilizer applicator (up to 1 m3. Besides, this device has dynamics responses very slow for several control propose. Munack, et al. (1999) [6] has showed a typical response for a magnetic inductive device. For typical devices, the time delay is 0.5 to 1.2 seconds followed by a lag time of  0.25 to more 1 second.

The application flow rate is normally controlled by control valve or variable speed pump. An electric actuator or hydraulic cylinder can activate the control valve. For sphere valves with electric motor the time needed for a full opening (10º to 90º) was measured as 0.5...2.0 s and 0.4…1.0 s for hydraulic cylinder.

Consequently, the control process with feedback can become very slow when used electric actuator or marginally unstable when used hydraulic cylinder due to delays of response provided by electromagnetic flow meter. Besides, the electric conductivity of fertilizers varies depending on temperature and concentration. So, a control system with electromagnetic flow meter needs frequents calibrations.

### 3   Intelligent Control Systems

An approach based on Artificial Neural Networks was designed and implemented in order to achieve a cheap, fast, robust and stable control system. The diagram block is illustrated in Fig.3. The control system uses secondary measurements to identify and control the fertilizer application rate and therefore, there is no flow meter device in the system.

Application

rate

Process

ANN 2

DB

NB

ZS

Tp



Tractor

speed

Application

width

ANN 1

qref

q(x,t)

qe





Coordinates GPS

controller

actuator

ZS=f(vol)

Fig. 3. Diagram block of the intelligent variable rate system for liquid fertilizer application based on ANN.

There are two ANN in this system. The output of ANN 1 produces the application rate in function of the GPS or DGPS coordinates [7]. The other one (ANN 2) estimates the fluid fertilizer flow rate (qe). The fertilizer flow rate q(x,t) is controlled through a sphere valve with electric actuator. The flow rate q(x,t) is a nonlinear function of the valve position (), hydraulic suction head (ZS), speed pump (), nozzle diameter (DB), number of nozzles in operation (NB), fertilizer temperature (Tp) fertilizer specific mass, fertilizer apparent viscosity, particles size in suspension (undissolved nutrients and clay added), etc. These factors affect all loss loads in the hydraulic system. Therefore, there are no precise mathematics models that describe the hydraulic process. So an Artificial Neural Network (ANN 2) of the type multilayer perceptron is used for the identification and control of the fluid fertilizer application rate.

The net training is made off-line by the algorithm of Levenberg-Marquardt [4] with training data obtained from laboratory measurements.

### 4   Artificial Neural Networks

A system identification technique is used to simplify the system control and make its implementation cheaper than conventional control techniques. In this problem, some identification method P(w) (in this case an ANN) with a convenient algorithm finds the best parameterization that represents the dynamic of the process, as shown in Fig.4. The parameterization is tuned through the minimization of  reconstruction error obtained from the output e(k), which it is used to adjust the ANN weights [8].

(Error)

e(k)

(Input)

P(w)

P(w)

P(w)

Unknown

process

ANN

-

+

Adaptation

Fig.4. System identification based on ANN.

In this paper, ANN of the type multilayer perceptron has been used to identify the process and to control the fertilizers application rate. The ability of Artificial Neural Networks in mapping functional relationships has become them an attractive approach that can be used in several types of problem [2]. This characteristic is particularly important when the relationship among the process variables is non-linear and/or not well defined, and thus difficult to model by conventional techniques.

An artificial neural network is a dynamic system that consists of highly interconnected and parallel non-linear processing elements that shows extreme efficiency in computation. The main benefits of using ANN on the control system of fluid fertilizer application are the following: a) the ability of learning and therefore generalization; b) the capacity of mapping complex systems without necessity of knowing the eventual mathematical models associated with them; c) the facility of implementation in hardware and software after ANN training.

A typical feedforward ANN is depicted in Fig.5, with m inputs and p outputs, where each circle represents a single neuron. The name feedforward implies that the flow is one way and there are not feedback paths between neurons. The output of each neuron from one layer is an input to each neuron of the next layer. The initial layer where the inputs come into the ANN is called the input layer, and the last layer, i.e., where the outputs come out of the ANN, is denoted as the output layer. All other layers between them are called hidden layer.

x1

x2

xm

y1

y2

yp

Input Layer

Hidden

Layer

Output

Layer

Fig.5. Typical feedforward ANN.

Each neuron can be modelled as shown in Fig.6, with n as the number of inputs to the neuron. Associated with each of the n inputs xi is some adjustable scalar weight, wi (i=1,2,…,n), which multiplies that input. In addition, an adjustable bias value, b, can be added to the summed-scaled inputs. These combined inputs are then fed into an activation function, which produces the output y of the neuron, that is:

y=g(i=1nwixi+b)                   (1)

where g is a sigmoid function defined by                        g(u) = (1+e-u)-1.

Bias

Weights

g(u)

b

Activation Function

Linear

Combination

Inputs

I1

I2

I3

IN

w1

w2

wN

w3

.

.

y



Fig.6. Single artificial neuron.

The training process of the neural network consists of the successive presentations of input-output data pairs. The basic structure having one hidden layer has been shown to be powerful enough to produce an arbitrary mapping among variables. During the training, the data are propagated forward through the network, which adjusts its internal weights to minimize the function cost (weighted squared deviation between the true output and the output produced by the network) by using the back-propagation technique. The details of the derivation of the back-propagation algorithm are well known in literature and its steps can be found in Haykin (1999) [2]. A review of the main steps of the algorithm is presented here. The function to be minimized is the sum of the average squared error (EAV) of the output vector,

EAV=1Nk=1NE(k)                 (2)

where N is the number of training points and E(k) is the sum of squared errors at all nodes in the output layer, i.e.,

E(k)=12j=1p(dj(k)−yj(k))2         (3)

For an optimum weight configuration, E(k) is minimized with respect to the synaptic weight w, so that for each data set,

∂E(k)∂wjil=0                         (4)

where w is the weight connecting the neuron j of the l-layer to neuron i of the (l – 1)-layer.

Finally, the weights of the network are updated using the following relationship:

wjil(k+1)=wjil(k)−η∂E(k)∂wji(k)        (5)

where  is a constant that determines the rate of learning of the back-propagation algorithm.

### Results and Discussion

For evaluation of the intelligent control system, some preliminary tests were accomplished with two NPK fertilizers, N32 and 16:08:08.

5.1 Liquid fertilizer: NPK 32:00:00

This test was realized with the solution NPK 32:00:00 (uan) under conditions showed in Table 1. It is observed in Fig.7 that the proposed neural approach provides flow rates near to real values. The correlation coefficient (R-value) between measured flow rates and those provide by the ANN 2 is close to 1, which indicates a good ANN generalization.

The mean relative error obtained was 2,91% and its standard deviation was 3,56%. Some relative errors are appreciable, but during the application operation their lifetime is very low, so their effect is not significant.

Table 1. ANN 2 topology and test conditions for

NPK 32:00:00.

#### ANN 2 topology

Architecture: multilayer perceptron (MLP)

Number of hidden layers: 1

Number of neurons of the hidden layer: 25

Data set training: 871 vectors

Data test pattern: 200 vectors

Test conditions

Fertilizer temperature: 26.5 °C

Application width: 3.6 meters

Number/diameter of nuzzles: 4/5 mm

Tractor speed: 5…12 km/h

Valve angle position: 5…90°

The Fig. 8 shows the time response of the system under agricultural field operation. The desired flow rate is close to the flow rate estimated by the ANN 2. The fertilizer volume required in the area was 41.0 L and the measured volume was 39.60 L. So, the volume error was 3.53%. This value indicates a good accuracy for maps-based variable rate fertilizer applicator proposed in this paper.

Fig.7. Correlation between measured flow rate and values provided by the ANN 2 for NPK 32:00:00.

Fig.8. Time response of the intelligent system control using NPK 32:00:00.

5.2 Liquid fertilizer: NPK 16:08:08

This test was realized with the suspension NPK 16:08:08 under conditions showed in Table 2. The test result for suspensions indicates a precision close to the N32. The mean relative error obtained was 3.07% and its standard deviation was 3.77%. However, for agricultural suspensions, the nuzzles should be made of porcelain to minimize the erosion due to abrasion. To avoid clogging in the control valve, the initial operational angle () must be greater than 10°.

Table 2. ANN 2 topology and test conditions for

NPK 16:08:08.

#### ANN 2 topology

Architecture: multilayer perceptron (MLP)

Number of hidden layers: 1

Number of neurons of the hidden layer: 28

Data set training: 820 vectors

Data test pattern: 185 vectors

Test conditions

Fertilizer temperature: 27.3 °C

Application width: 3.6 meters

Number/diameter of nuzzles: 3/6 mm

Tractor speed: 5…15 km/h

Valve angle position: 10…90°

An important aspect in this approach is the transient response of the system when a step excitation is applied. The settling time observed in Fig.9 decreases the accuracy of the distributor in the field boundary. This problem can be minimized whether the system always to keep its operation close to desired flow rate value. So, it is necessary electric on-off valves near the nuzzles to start the application in the field boundary.

Estimated

flow rate

Desired

flow rate

Fig.9. Step response (25 L/min) of the intelligent control system using an electric control valve with 12 s for full opening.

### Conclusions

The proposed method provides a systematic approach to control the application of liquid fertilizer and others agricultural inputs such as solid fertilizers, pesticides and seeds.

The test results demonstrate that the proposed approach is an efficient alternative to the conventional models that are usually used to control these processes. The main advantages in using the proposed approach are:

- the flow meter device can be removed, allowing a reduction up to 36% in cost of small map-based variable rate applicators;

- very good accuracy for precision agriculture;

- economy of agricultural inputs;

- reduction of environmental impacts and;

- effective obtainment of economical and operational gains.

- simplicity of implementation in hardware and software after the ANN training;

In sake of a better understanding, the system has still been developed. Several fluid fertilizer formulation  will be considered to validate the proposed approach.

#### References:

[1] Morgan M. and Ess D. (1997). The precision  farming guide for agriculturists. John Deere Publishing, Moline, Illinois.

[2] Haykin, S. (1999). Neural Networks  - A comprehensive foundation. McMillan Inc., Englewood Cliffs – NJ.

[3] Kosko, B. (1992). Neural networks and fuzzy systems – A Dynamical Systems Approach to Machine Intelligence. Prentice-Hall, Englewood Cliffs – NJ.

[4] Hagan, M. T. and M. Menhaj (1994). Training feedforward networks with the Marquardt algorithm. IEEE Trans. on Neural Networks, vol. 5, pp. 989-993.

[5] Palgrave, D.A. (1991) Fluid fertilizer: Science and Technology. New York: Marcel Dekker Inc., 1991, p.786.

[6] Munack, A., E. Bunnig and H. Speckmann (1999). A high performance control system for spreading of liquid manure. Proceeding of the 14 th IFAC, Pequin, China, pp.389-394.

[7] Ulson, J.A.C., I.N. Silva, S. H Benez, R.L.V. Boas (2000). Modeling and identification of fertility maps using artificial neural networks. 2000 IEEE  Internacional Conference on Systems, Man and Cybernetics. Nashville, USA.

[8] Hagan, M. T. and H. B. Demuth (1999). Neural networks for control. Proceedings of the American Control Conference. San Diego, California, USA, pp 1642-1656.