Analysis of Stability and Transition in Compressible Boundary Layers Using Linear Stability Theory

SENEM ATALAYER, SERKAN ÖZGEN

Department of Aerospace Engineering

Middle East Technical University

06531, Ankara

TURKEY

Abstract: In this paper, the stability and transition problems are investigated for compressible boundary layers using the Linear Stability Theory. As the prediction tool, Smith-Van Ingen en Transition Prediction Method is used. Two dimensional momentum and energy equations are solved for the mean flow. For fluctuating flow, stability equations that are derived starting from three dimensional Navier Stokes equations are handled. The equations are then solved using an efficient shoot-search technique. With this method, critical Reynolds (Rcr) and transitional Reynolds numbers (Rtr) are sought for a wide range of Mach numbers. The numerical results thus obtained are then compared with the numerical and experimental data available in the literature.

Key-Words: linear stability theory, laminar-turbulent transition, compressible boundary-layer flow

1   Introduction

Laminar-turbulent transition is still an important issue in fluid dynamics and aerodynamics with a large number of engineering applications. The reason is that it controls the evolution of important aerodynamic quantities such as heat transfer and skin friction drag. The reduction of heating rates for the reentry vehicles, the reduction of drag on the high subsonic-speed commercial aircraft wings and the reduction of hydrodynamic noise in commercial aircraft are only some of areas that a good knowledge about transition is needed.

The Linear Stability Theory is mainly concerned with individual sine waves propagating in the boundary layer parallel to the wall. These waves, referred as the instability waves, were first explained by Rayleigh (1887) and Prandtl (1921) as small, regular oscillations traveling in the laminar boundary layer. A complete theory of boundary layer instability was studied by Tollmien (1929) and the total amplification of the most unstable frequencies was calculated by Schlichting (1933). That is why, the instability waves are also known as Tollmien-Schlichting waves. Besides, the first demonstrations of the existence of these waves were done by Schubauer and Skramstad (1948) through their well-known experiments. According to these, T-S waves are the first stage of the transition process.

Compressibility makes this problem not only more realistic for most flow regimes, but also more complex. The studies on this issue start with Küchemann (1938). He was the first who tried to build a compressible linear stability theory. Lees and Lin [1] followed him with their important theoretical investigations. From 1960 up to now, Mack [2,3] has performed a complete numerical investigation in compressible laminar boundary layers and discovered higher modes at supersonic speeds.  He found that the first of these higher modes, named as second mode, is the dominant one for the zero pressure gradient boundary layers over a flat plate. This is why it is also known as the Mack mode.

Smith-Van Ingen  en transition prediction method is one of the most popular techniques preferred even today as a prediction device in spite of its deficiencies, such as neglecting the nonlinear mechanisms and receptivity. The method was first devised by Smith [4] and Van Ingen [5].

2   Problem Formulation

  - Mathematical Modeling

The compressible stability theory starts with three dimensional equations of motion, i.e. momentum, energy, continuity equations and the equation of state for a viscous, heat-conducting, perfect gas. Each flow quantity is divided into steady mean flow and unsteady fluctuating terms as follows:

ui=Ui+ui    i=1, 3,         p=P+p,

T=T+T              ,         ρ=ρ+ρ,

μ=μ+μ                 ,         κ=κ+κ.                 (1)

In the above, asterisk (*) denotes dimensional quantities and (^) denotes fluctuations.

As a next step, parallel flow and steady basic flow assumptions are employed. The fluctuations are small so the quadratic terms are dropped. The resulting equations are then nondimesionalized with respect to the proper freestream values. This procedure leads us to the nondimensional, linearized system of equations.

2.1.1   Normal Mode Analysis

One way of computing the range of unstable waves is to introduce small disturbances into the dimensionless, linearized system of equations in the form of normal modes. A possible normal mode is:

(2)

where,

ϕ→Any fluctuating quantity (u, v, p, T, etc.),

ϕ(y)→Disturbance amplitude function,

α,β→x and z components of the wave number vector k, respectively,

ω→ Circular frequency.

Normal modes are the traveling waves in x, z plane. The physical solutions are the real parts of these modes. If ,  and  are all real, the wave is neutrally stable and it propagates in this plane with constant amplitude and phase velocity c=/. Any of them being complex, means that the amplitude will change as the wave propagates.

In this study, temporal amplification formulation is used since it results in a linear set of equations. Due to the temporal case,  and  are real, whereas  is complex. Thus, c is introduced as c=/k=cr+ci, where k is the magnitude of k. Substituting the normal modes into the dimensionless, linearized system of equations and performing the necessary algebra, one can reach the following system of ordinary differential equations.

The momentum equation in the direction parallel to the wave number vector:

(3)

Momentum equation in the normal (y) direction:

(4)

Momentum equation in the direction normal to the wave number vector:

(5)

Energy equation:

(6)

Continuity equation:

(7)

Equation of state:

p=TT+ρρ.                                                           (8)

The boundary conditions are as follows:

bounded as y→∞.                            (9)

In the above, R is the Reynolds number based on the Blasius length scale. D denotes the derivative with respect to dimensionless y. These equations are the basic equations of the compressible stability theory.

    - First Order Equations

Defining new variables in order to form a system of first order equations as:

Z5=T,

Z6=DZ5,

Z7=αw−βu,

(10)

With these definitions, Eq. (3)-(8) are rewritten as:

DZi(y)=18aij(y)Zi(y)   i=1,8,                              (11)

where aij are the elements of the coefficient matrix. The corresponding boundary conditions are:

Z1(0)=Z3(0)=Z5(0)=Z7(0)=0,

bounded as y→∞.          	(12)

Resulting system of homogeneous equations, together with the homogeneous boundary conditions constitute an eigenvalue problem with the eigenvalues (,,cr,ci,R) and the eigenfunctions  u,v,w,p.

    - Uniform Mean Flow

In the freestream, U=U1, W1=0, T1=1, µ1=1, 1=1, and the y-derivatives of mean flow quantities are zero. Applying these conditions to the first order system of equations defined in Eq. (11), one can get a system of equations with constant coefficients. The solution of this system is in the form of:

Zi=Aieλiy            i=1,8.                                      (13)

Here, the eight characteristic values i are found as:

λ1,2=±b11,λ3,4=±(b22+b33)2+12b33−b222+4b23b32,     			             (14)

with

(15)

These characteristic values should not be confused with the eigenvalues (,,cr,ci,R) of the system expressed in Eq. (11).

The freestream boundary conditions defined in Eq. (12) require decaying solutions far away from the wall. For this reason, only the characteristic values with a negative sign are taken into account. For each negative characteristic value, there exists a distinct eight component characteristic vector Ai. These vectors together with the characteristic values form the eight component solution vectors, Zi defined in Eq. (13).

  - Numerical Techniques

The solution vectors in Eq. (13) that are obtained using the freestream conditions provide the starting values for the numerical integrations of Eqs. (11). For the numerical integration, a fourth order variable stepsize Runge-Kutta method is used. Due to this method, integration proceeds from the freestream to the wall. One problem is that, as the integration proceeds, growth speeds of four solutions start differing from each other and as integration proceeds this difference increases. This causes the linear independence between these solutions to be lost. In order to prevent this, the Gram-Schmidt Orthonormalization Technique is used in certain intervals. When the wall is reached, it is checked whether the boundary conditions given in Eq. (12) are satisfied. These will be satisfied only for the proper combinations of the eigenvalues (,,cr,ci,R), which forms the essence of the shooting method.

The stability diagrams have been obtained using Newton Iteration in two variables. This method requires two initial points on the curve so that the iteration can proceed in the specified Reynolds number direction. These points can be found by using a function minimization routine employing the simplex method.

A  FORTRAN code is written in order to solve the linear stability problem by implementing the methods mentioned above and to obtain proper numerical results. The program is capable of solving the eigenvalue problem for either two or three dimensional sinusoidal disturbances using temporal amplification theory. With the use of the techniques mentioned above the correct   combinations of ,   and R are found.

  - en Transition Prediction Method

The resulting combinations of eigenvalues are used to obtain the amplitude ratios of the disturbances. The amplitude ratio or the amplification rate is the most useful quantity that the stability theory provides to transition problem. The calculation of amplitude ratio is maintained by en transition prediction method. According to this method, transition starts when a small disturbance is introduced at a critical Re and amplified by a factor of en as moving downstream. The typical value for n is 9 for low turbulence and free flight conditions due to the experimental observations. In other words, the breakdown to turbulence occurs when the amplitude of the most amplified Tollmien–Schlichting wave becomes e9 times as large as its initial amplitude. Gaster’s transformation is used to convert the temporal growth rate to spatial growth rate. In this respect, n factor can be calculated by

(16)

in terms of the dimensionless parameters. Here, R0 is the Reynolds number at the neutral stability point; R is any Reynolds for fixed frequency; A0 is the amplitude of the disturbance corresponding to R0; A is the amplitude at R; and at each R, n represents the maximum amplification factor of the disturbances. In the developed code, the integration is replaced by a summation:

n=k=1N2αcicgkΔRk,                                            (17)

where, cg is the velocity of the wave packets called the group velocity. With the use of this summation in Eq. (17), the calculations are performed for finite number of Reynolds numbers between R0 and R.

The integration as a result, gives the amplification rates for different frequencies. The tangent line to those curves, so called the envelope curve, is the final step of the stability problem and shows the maximum amplification possible at a given Reynolds number. The Reynolds number corresponding to n=9 (from experimental results) on this curve is the one that breakdown to turbulence occurs; that is the transitional Reynolds number (Rtr).

3   Problem Solution

This paper is concerned with two dimensional mean flow with two dimensional disturbances in compressible boundary layers over a flat plate with adiabatic wall condition.

Comparison of the results obtained for M=0 with the ones for incompressible flow =0 case was the first step in confirmation of the solution method and the code written in this study. The constant amplification factor curves, minimum critical Reynolds number ((Rcr)FS=301.7 in terms of the Blasius length scale) and transitional Reynolds number value that corresponds to n=9 ((Rtr)FS=1800) obtained using the code are all in perfect agreement with the ones found by many researchers for the  incompressible flow over a flat plate in literature [6], [7].

Fig. 1 Effect of Mach number on neutral stability curves for two dimensional disturbances.

Fig. 1 shows the change in shapes of the neutral stability curves with respect to Mach number. As Mach number increases from 0 to 1.5, critical Reynolds number decreases gradually. Then, it starts increasing as Mach number further increases. On the other hand, the shapes of these curves do not change very much up to M=2. The reason could be the dominant effect of viscous instability on the boundary layer in this Mach number range. In other words, the generalized inflection point remains very close to the wall in this interval which causes the boundary layer to be unstable under the effect of viscosity. This inflection point moves away from the wall and Rayleigh’s instability mechanism starts to be the dominant as the Mach number increases. Such that, after a certain Mach number, ci=0 curves tend to be parallel with the Reynolds axis at high Reynolds numbers. It is possible to notice it for M=2.5 and M=3   in Fig. 1. This can be observed much earlier for M=3. This phenomenon is due to an inviscid mechanism.

Fig. 2 Stability diagram, viscous and inviscid modes for M=4, adiabatic wall.

Being different from ci=0 in Fig. 1, there exists a second mode for the neutral stability curves in Fig. 2.  This additional mode (acoustic mode), is the first and the most unstable of the additional modes that has been first found by Mack for a zero pressure gradient boundary layer over a flat plate [2], [3]. These inviscid modes,  called the Mack modes, exist when the relative flow Mach numbers at the wall are supersonic while they are are subsonic in the freestream. The relative Mach number is the flow Mach number in a reference frame moving at the velocity of an instability wave. These higher modes are the creations of a continuing reflection of acoustic waves in a relative supersonic region between the wall and the relative sonic line. This second inviscid mode could be obtained after M=3 [2], but since their first occurance are at very large wave numbers compared to the first (viscous) mode, it is not possible to plot it in Fig. 1. However, this additional mode is observed at lower and lower wave numbers as the Mach number is increased. At M=4 these two modes come closer so that the second mode appears now at =0-0.4 range. It is noteworthy that there is almost no difference between the amplification rates of the two modes.

Fig. 3 Effect of Mach number on critical Reynolds number, from M=0 to M=3.

Fig. 4 Integrated growth rates and the envelope curve  for M=1.

As seen in Fig. 3, critical Reynolds number tends to increase with increasing Mach number from M=0 to 3.

Plotting the constant frequency curves, which is one of the most important consequences of Linear Stability Theory is the last step in transition Reynolds number estimaton.

In compressible viscous theory, for a flat plate flow on an adiabatic wall,  the effect of compressibility on transition is highly affected by freestream Mach number. Due to this, compressibility first has a stabilizing effect on flow from between M=0 and M=2, then destabilizing effect from M=2 to M=4, and stabilizing again at hypersonic flows [3].

When compared with the results at M=0, the growth rates plotted in Fig. 4 belong to the waves with smaller frequencies. That is, the wave leading to transition has smaller frequency than the one at M=0. In addition, the slope of the envelope curve is larger. As a result, the Reynolds number corresponding to maximum amplification rate n=9 (Rtr) on this curve is  larger than the one obtained at M=0 which is a clear evidence that the flow is more stable for this Mach number, as expected.

As a final step, Fig. 5 shows the change in envelope curves and consequently the transition Reynolds number with respect to Mach number, approving the results reported above.

One point to notice is that the effect of Mach number on Rcr may not be parallel with its effect on Rtr. It is experimentally proven that Rtr decreases as the flow is destabilized between M=2 and M=4 [3]. However, both this paper (Fig. 3) and other studies in literature [2], [6] confirms the increase of Rcr at that Mach number range.

Fig. 5 Effect of Mach number on transitional     Reynolds numbers.

4   Conclusion

This paper is based on the analysis of stability and transition for two dimensional, viscous, compressible boundary layers over a flate plate.  For this purpose, a FORTRAN code was developed that obtains first the neutral stability curve and constant temporal amplification (ci) curves (the variation of wave number with respect to Reynolds number) of the boundary layer flow for any freestream Mach number. Using the plotted neutral stability curves, the critical Reynolds number can easily be obtained. Besides, it is possible to obtain the integrated growth rates, i.e. constant frequency curves of the waves with the use of this code. The envelope curve defined as the tangent line to these curves is the last step in estimation of the transitional Reynolds number.

To confirm the validity of this study, the numerical and graphical results corresponding to M=0 was compared with the ones in the literature that were obtained using the same method for incompressible boundary layers corresponding to  =0. The result is satisfactory; they match both graphically and numerically (Rcr, Rtr).

Next, the same procedure was repeated for every 0.1 Mach number interval between M=0 and M=1. But, because of the lack of data for such small Mach number values in literature, it was not possible to confirm the obtained results either graphically or numerically. The increase in flow stability up to M=2, in other words, the increase in transitional Reynolds number from M=0 to 2 is the only information in literature and this is confirmed by the results of this study.

The results for higher Mach numbers were compared with the ones in literature obtained both experimentally [3] and numerically [2], [6]. But, in a general point of view, a good agreement with those in literature was hard to reach. For instance, Fig. 2 has a good agreement qualitatively with the graphical data for the same Mach number in [2] and [6], but not quantitatively. This is expected because in this study the Prandtl number is taken as constant whereas, it depends on temperature in [2]. Besides, there is no an exact information about how the energy equation has been included to the stability problem in [2]. That’s why, it is hard to claim that the procedure in [2] exactly matches with the one followed in this study. Same question is valid for the procedure, equations used and the results in [6].

To conclude, the results obtained in this study are the products of a set of complete and a well-proven theory. The code could also be modified to treat three-dimensional mean flows with two- or three- dimensional disturbances. The limitation of the method and the code developed is due to the range of validity of the empirical relation used for the viscosity and the assumption of constant Prandtl number.

References:

[1]	Lees L., Lin C. C., Investigation of the Stability of the Laminar Boundary Layer in a Compressible Fluid, NACA TN 1115, 1946.

[2]	Mack L. M., Boundary Layer Stability Theory, AGARD Report 709, Special Course on Stability and Transition of Laminar Flows, Brussels, 1984.

[3]	Mack, L. M., Linear Stability Theory and the Problem of Supersonic Boundary-Layer Transition, AIAA Journal, Vol. 13, No. 3, 1975, pp. 278-289.

[4]	Smith, A.M.O., Transition, Pressure Gradient and Stability Theory, in Proceedings IX International Congress of Applied Mechanics, Brussels, Vol.4, 1956.

[5]	Van Ingen, J. L., A Suggested Semi-empirical Method for The Calculation of the Boundary Layer Transition Region, Report VTH74, Delft, 1956.

[6]	Arnal D., Boundary Layer Transition: Predictions based on Linear Theory, AGARD Report 793,  Special Course on Progress in Transition Modeling, Brussels, 1993.

[7]	Özgen S., Degrez, G. ve Sarma, G.S.R., Two fluid boundary layer stability, Phys. Fluids, Vol. 11, No. 11, 1998, pp. 2746-2757.

[8]	Cebeci T., Cousteix J., Modeling and Computation of Boundary Layer Flows, Springer, 1998.